3GPP TSG-SA3 Meeting #123																			S3-252809

Goteborg, Sweden, 25 – 29 August 2025

Source:	Qualcomm Incorporated

Title:	Privacy protection of device ID in group inventory

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	Ambient_IOT_Sec

Comments

This contribution proposes a mechanism to protect AIoT device identifier privacy during Inventory with filtering information (i.e., group inventory). The proposed mechanism is based on the mechanism captured in the living document (S3-252326), and is aligned with the inventory procedure described in S3-252806. Also, this contribution addressed the following editor’s notes in the living document.

Editor’s Note: whether AIOTF or ADM computes and checks T-ID is FFS.

Editor’s Note: in case AIOTF computes T-ID, how the KAIoTF is derived in ADM and how AIOTF retrieves the KAIoTF is FFS. In case ADM computes T-ID, the impact of interaction between AIOTF and ADM and the analysis of load of ADM is FFS.

Editor’s Note: which input key (e.g., KAIoT or KAIoTF) to be used is FFS

The above Editor’s Notes are deleted as the updated procedure proposes to have AIOTF hold the KAIoTF and generate a list of RESAIOT’. The proposal is intended to reduce the signaling overhead between AIOTF and UDM. We note that if AIOTF does not have a valid KAIoT, the bootstrapping procedure is performed as proposed in S3-252807.

Editor’s Note: whether AIOTF can acquire device ID from ADM is FFS.

The above Editor’s Note is removed as the AIOTF can retrieve the list of target AIoT device IDs from the ADM if the AIOTF does not already have the list. This retrieval can be done in the same manner as step 11 of clause 6.2.2 of TS 23.369.

Editor’s Note: How to address paging all case (i.e. paging message does not contain filtering information) is FFS.

As described above, the AIOTF retrieves the list of all target AIoT device IDs from the ADM. Thus, it is proposed to remove the above Editor’s Note.

Editor’s Note: whether and how to address Replay attack (e.g., replay RANDAIOT_n to track the same T-ID) or DoS attack (e.g., energy depletion in AIoT device) is FFS.

Since the AIoT system is deployed as SNPN and limited to indoor scenario, it is proposed to add a NOTE that tracking the T-ID by replaying inventory request is not addressed in this specification, and remove the above Editor’s Note.

Editor’s Note: the format of the configured filtering information is FFS.

Editor’s Note: issues related filtering information is FFS.

It is proposed to remove the above Editor’s Notes and add a NOTE describing the configured filtering information prevents the AIoT device from matching the received filtering information that could reveal its AIoT device ID. Since such restriction (e.g., limiting which bits of AIoT device ID is allowed for filtering information) depends on specific use cases of filtering information, the format of the configured filtering information is left to the application layer.

Editor’s Note: whether a random number from device is required is FFS

For group inventory, random number from device is not used unless there is a following command procedure. Thus, it is proposed to remove the above Editor’s Note.

Editor’s Note: Whether network can provide network assigned T-ID to the AIoT device in command procedure is FFS.

This contribution proposes to use a temporary ID generated based on the KAIoT. Hence, it is proposed to remove the above editor’s note.

Lastly, we defined the function FA for RESAIoT’ calculation. Defining the cryptographic algorithm to realize FA (e.g., HMAC-SHA-256) is not the purpose of this contribution and can be discussed separately.

Proposed Changes

# * * * * First change * * * *

## 5.4.3	Procedure for AIoT Device identifier protection during inventory with filtering information

Figure 5.4.3-1:  Privacy protection of AIoT device ID in inventory with filtering information

0.	Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed. The received AIoT Identification Information includes filtering information.

1.	The AIOTF shall retrieve AIoT device IDs that match with the filtering information. The AIOTF shall also retrieve the KAIoTF for each retrieved AIoT device ID.

If the KAIoTF is not available locally, the AIOTF shall perform a key bootstrapping procedure to establish the KAIoTF.

The AIOTF shall then generate a random nonce, RANDAIOT_n, and compute the list of XRESAIOT'’ for the respective AIoT device IDs as specified in Annex A.yx.

NOTE a: Construction of the list can happen later, but before step 8.

2.	The AIOTF shall send Inventory Request message including the filtering information and RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3.	The NG-RAN shall send the Paging message including filtering information and RANDAIOT_n to the AIoT device.

NOTE: since the AIoT system is deployed as SNPN and limited to indoor scenario in this release, trackability by actively replaying inventory request is not addressed in this specification.

4.	Upon receiving the Paging message, AIoT device shall check if the filtering information matches with the configured filtering information. If they match, the AIoT device shall compute a RESAIOT’ as specified in Annex A.yx for network authenticating AIoT device.

If the filtering information does not match, the AIoT device shall stop further processing of the Inventory Request, i.e., the rest of the steps are not performed.

NOTE b: The configured filtering information prevents the AIoT device from matching the received filtering information that could reveal its AIoT device ID. Since such restriction (e.g., limiting which bits of AIoT device ID is allowed for filtering information) depends on specific use cases of filtering information, the format of the configured filtering information is left to the application layer.

5.	The AIoT device sends D2R message to the NG-RAN, including RESAIOT’.

6.	NG-RAN shall the Inventory report message to the AIOTF, including the RESAIOT’.

7.	Upon receiving the Inventory Report, the AIOTF shall find a XRESAIOT’ that matches with the received RESAIOT’ on the list for authenticating the AIoT device.

8.	If there is a match, the AIOTF shall retrieve the AIoT device ID that corresponds to the matched XRESAIOT’ and continue the steps 12-14 in clause 6.2.2 for inventory procedure or the steps 8-11 of clause 6.2.3 for command procedure in TS 23.369 [2].

# * * * * Second change * * * *

# A.xx	Function FA

The function FA is defined as follows:

Output = FA(key, input parameters)

Editor’s Note: the details of function FA is FFS.

# * * * * Third change * * * *

# A.yx	RESAIoT’ calculation

When calculating RESAIoT’, the following parameters are used as input to the FA as defined in Annex A.xx:

-	AIoT device ID;

-	RANDAIOT_n;

The input key shall be KAIoTF.

# * * * * End of change * * * *