3GPP TSG-SA3 Meeting #123																			S3-252812

Goteborg, Sweden, 25 – 29 August 2025

Source:	Qualcomm Incorporated

Title:	Adding a description of mutual authentication

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	Ambient_IOT_Sec

Comments

This contribution proposes to add a clause for mutual authentication during Command procedure. Particularly, it is proposed that mutual authentication is performed in two steps: AIoT device authentication by the network as described in clause 5.2.2 and network authentication by the AIoT device based on the integrity check of the NAS Command Request as described in clause 5.2.x in this contribution.

Since this proposal describes how mutual authentication is performed, it is also proposed to remove the following Editor’s Note in clause 5.2.2:

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

Proposed Changes

# * * * * First change * * * *

### 5.2.x	Mutual authentication during command procedure

During the command procedure, the AIOTF authenticates the AIoT device based on the Inventory procedure as described in clause 5.2.2. The AIoT device authenticates the AIoTF successfully if the integrity check of the NAS Command Request passes as described in step 3 of clause 5.3.1.

# * * * * End of first change * * * *

# * * * * Second change * * * *

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identifier and RANDAIOT_d to ADM.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

8. ADM derives XRESAIOT using the same method as in AIoT device.

Editor’s Note: Where the authentication credential is processed in AIOT device is FFS.

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

Editor’s note: How and where to derive keys is FFS.

# * * * * End of second change * * * *