3GPP TSG-SA3 Meeting #123	S3-252841

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Thales

Title:	Hybrid PQC for SUCI calculation

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.0.0

Work Item:	FS_CryptoPQC

Comments

This contribution proposes to address SUCI calculation in TR 33.703 and add a solution based on GSMA guidelines for SUCI calculation described in "Post Quantum Cryptography – Guidelines for Telecom Use Cases – v2.0".

* * * First Change * * * *

# 7		Protocols expected to be updated for PQC by 3GPP

Editor’s Note: This clause contains identification of the protocols with asymmetric cryptography listed in TR 33.938 that are not expected to be updated by other SDOs in a near future to use PQC, e.g., MIKEY-SAKKE and SUCI calculation, security threats and alternative solutions for the 3GPP procedures if they are not updated to use PQC.

### 7.1	Threats

Editor’s Note: This clause contains security threats for the 3GPP procedures if they are not updated to use PQC.

Future quantum computing may be able to break the protocols with asymmetric cryptography, rendering data and information vulnerable.

### 7.1.Y	Protocol #Y: SUCI calculation

TS 33.501 specifies protection schemes for concealing the subscription permanent identifier (SUPI). ECIES-based protection scheme uses both symmetric cryptography and asymmetric cryptography. Consequently, ECIES-based protection scheme and associated ECIES profiles need to be updated to support PQC algorithms for SUCI calculation.

* * * Next Change * * * *

### 7.2.Y	Solutions to Protocol #Y: SUCI calculation

Editor’s Note: If only SUCI calculation is considered, this subclause may be removed. If other protocol, e.g. MIKEY-SAKKE is studied, this subclause is used for each of such protocol identified.

#### 7.2.Y.Z	Solution #Z to Protocol #Y: GSMA-based solution

##### 7.2.Y.Z.1	Introduction

GSMA published guidelines "Post Quantum Cryptography – Guidelines for Telecom Use Cases – v2.0" [a] to support the planning, setup and execution of a quantum safe cryptography journey for telco industry. This GSMA report contains a detailed analysis of an initial set of Telcom use cases that are impacted by Post Quantum Cryptography. Concealment of the Subscriber Public Identifier is one of the analysed use cases.

Since publication of GSMA guidelines [a], additional security enhancements have been identified and are proposed in this solution.

##### 7.2.Y.Z.2	Solution details

The solution for concealment of the Subscriber Public Identifier is based on the hybridization between ML-KEM (Level 3) and classic ECC based key exchanged algorithms that is described in clause 5.8 of GSMA guidelines [a].

GSMA solution is enhanced thanks to the addition of Post Quantum ciphertext as input to the Key Derivation Function.

The associated updated profiles are the following ones:

### Profile A’ (update of Profile A to support PQC algorithm)

The ME and SIDF shall implement this profile. The ECIES parameters for this profile shall be the following:

-	KEM domain parameters						: ML-KEM-768 [b]

-	EC domain parameters							: Curve25519

-	KEM primitive									: ML-KEM-768 [b]

-	EC Diffie-Hellman primitive					: X25519

-	point compression								: N/A

-	KDF												: HMAC-based KDF RFC 5869 (SHA-256)

-	Hash												: SHA-256

-	SharedInfo1										:  R(the ephemeral public key octet string – see [c] section 5.1.3)

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 32 octets (256 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES-256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 16 octets (128 bits)

-	backwards compatibility mode					: false

### Profile B’ (update of Profile B to support PQC algorithm)

The ME and SIDF shall implement this profile. The ECIES parameters for this profile shall be the following:

-	KEM domain parameters						: ML-KEM-768 [b]

-	EC domain parameters							: secp256r1

-	KEM primitive									: ML-KEM-768 [b]

-	EC Diffie-Hellman primitive					: Elliptic Curve Cofactor Diffie-Hellman Primitive

-	point compression								: true

-	KDF												: HMAC-based KDF RFC 5869 (SHA-256)

-	Hash												: SHA-256

-	SharedInfo1										: R (the ephemeral public key octet string - see [c] section 5.1.3)

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 32 octets (256 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES-256 in CTR mode

-	enckeylen											: 32 octets (256 bits)

-	icblen												: 16 octets (128 bits)

-	backwards compatibility mode					: false

##### 7.2.Y.Z.3	Evaluation

TBD

* * * Next Change * * * *

# 2	References

[a]	GSMA: "Post Quantum Cryptography – Guidelines for Telecom Use Cases - v2.0"

[b]		NIST FIPS 203, Module-Lattice-Based Key-Encapsulation Mechanism Standard,  https://nvlpubs.nist.gov/nistpubs/FIPS/NIST.FIPS.203.pdf

[c]		SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

* * * End of Changes * * * *