3GPP TSG-SA3 Meeting #123	S3-252847

# Goteborg, Sweden, 25 – 29 August 2025

Source:	OPPO

Title:	Authentication Procedure for Command

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to add AIoT authentication solution for inventory and command procedure to TS 33.369.

- For AIOT Device authenticating network, the MAC should be derived at ADM and using KAIoT.

- For command authentication, ADM can derive MAC and include MAC in authentication data to AIOTF in the same “return authentication data” message. the reason is in step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369, AIOTF knows it’s inventory only or inventory and command procedure. The benefit is NO extra service flow between ADM and AIOTF added for command authentication.

- A second network random number is generated in ADM and used to derive MAC for device authenticating network, because there is no indication to the AIOT device that whether there is subsequent command procedure in the inventory procedure, and no guarantee for the AIOT device to store the first random number received in inventory request.

Proposed Changes

* * * 1st Change * * * *

## 5.2	Authentication procedure

* * * Next Change * * * *

## 5.2.Z	Authentication procedure for inventory and command

Figure 5.2.y-z: Authentication procedure for inventory and command

Step 1-7 is the same as in 5.2.2

- For command procedure and mutual authentication, additionally, ADM decides mutual authentication is needed, and ADM shall generates RANDAIOT_n2, derives MACAIOT using KAIoT and RANDAIOT_n2 for device authenticating network.

MAC = fx (KAIoT,  RANDAIOT_n2)；

ADM shall derive communication protection key KAIoTF in this step. The method of KAIoTF derivation is descried in 5.3.X.

Editor’s Note: Whether Fx is as same as RES derivation function is FFS.

- For command procedure and mutual authentication, additionally ADM shall return RANDAIOT_n2,  MAC AIOT and communication protection key KAIoTF.

- For command procedure and mutual authentication, additionally AIOTF shall store the KAIoTF.

- AIOTF shall construct a NAS Command Request including MACAIOT and RANDAIOT_n2 . AIOTF sends NAS Command Request to AIoT RAN Reader.

- AIoT RAN Reader shall send NAS Command Request includes MACAIOT and RANDAIOT_n2 to AIoT Device as specified in as specified in TS 38.300 [x3] and TS 38.391 [x4] .

- AIoT Device receives NAS Command Request and shall decide mutual authentication is needed, and shall derive XMACAIOT as the same way as ADM. AIoT Device shall verify the MACAIOT to authenticate the network.

AIoT Device shall derive communication protection key KAIoTF in this step. The method of KAIoTF 	derivation is descried in 5.3.X.

- AIoT device sends Command response to RAN Reader as specified in as specified in TS 38.391 [6], indicating successfully authenticating network.

- RAN Reader sends Command response to AIOTF.

Step 11-15 is aligned with clause 6.2.3 Procedure for Command in TS 23.369 [2].

* * * Next Change * * * *

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall send the paging request message including RANDAIOT_n to the AIoT device.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identifier and RANDAIOT_d to ADM.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

8. ADM derives XRESAIOT using the same method as in AIoT device.

Editor’s Note: Where the authentication credential is processed in AIOT device is FFS.

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

Editor’s note: How and where to derive keys is FFS.

* * * End of Changes * * * *