3GPP TSG-SA3 Meeting #123	S3-252872

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Xiaomi

Title:	Key Issue on PSK Delivery

Document for:	Endorsement

Agenda item:	6.1.6

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:	<Work Item>

Comments

According to the new key issue proposed in S3-252871, the pre-share key between the UE and UPF needs to be derived on the UE side and network side. For key derivation on the network side, if the pre-shared key is not derived by the UPF, the pre-shared key needs to be delivered to the UPF by the network function deriving the key.

Accordingly, this paper proposes to add a new key issue on security aspects for pre-share key delivery for QUIC security study.

* * * First Change * * * *

5.X	Key Issue #X: PSK delivery

5.X.1	Key issue details

Based on key issue #X of the present document, the pre-share key between the UE and UPF needs to be derived on the UE side and network side. For key derivation on the network side, if the pre-shared key is not derived by the UPF, the pre-shared key needs to be delivered to the UPF by the network function deriving the key.

This key issue addresses the security aspects for the delivery of the derived pre-share key to the UPF.

5.X.2	Security threats

In case the pre-shared key between the UE or UPF is not derived by the UPF but delivered to the UPF by another NF, if the key delivery to the UPF is not protected, the pre-shared key may be obtained by an attacker, who can:

-	impersonate the UPF towards the UE to set up TLS tunnel to communicate with the UE over MPQUIC protocol; or

-	launch DoS attack by delivering a tampered key to the UPF to fail the TLS authentication and tunnel establishment.

5.X.1	Potential security requirements

The 5G system shall be able to provide integrity protection, confidentiality protect and anti-replay protection for the pre-shared key delivered to the UPF.

* * * End of Change * * * *