3GPP TSG-SA3 Meeting #123	S3-252878

# Goteborg, Sweden, 25 – 29 August 2025

Source:	OPPO

Title:	Security requirements to AIoT Device

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to approve the security requirements to AIoT Device regarding authentication and command protection procedures.

* * * First Change * * * *

#### 4.2.1.2 	Requirements related to authentication between device and network

The AIoT device shall support the following cryptographic primitives:

-	a secure method for random bit generation.

-	a secure method for RES and MAC derivation.

Editor’s Note: Security requirements on the method for random bit generation is FFS.

Editor’s Note: Further cryptographic primitives are FFS.

* * * Next Change * * * *

#### 4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support protection key derivation based on long-term AIoT key.

Editor’s Note: For this clause, the replay protection, privacy and authentication related requirements are FFS.

* * * End of Changes * * * *