3GPP TSG-SA3 Meeting #123	S3-252999

# Goteborg, Sweden, 25 – 29 August 2025									merger of S3-252610, 2660, 2880, 2888

Source:	OPPO, CMCC, ZTE, Huawei

Title:	Security requirements to AIoT NG RAN

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to approve the security requirements to AIoT NG RAN regarding authentication procedure.

* * * First Change * * * *

### 4.2.4	Security Requirements on the NG-RAN

AIOT2 is the reference point between the AIOTF and the NG-RAN.

NG-RAN shall support the use of integrity, confidentiality and replay protection with the AIOTF over the AIOT2 interface.

* * * End of Changes * * * *