3GPP TSG-SA3 Meeting #123	S3-253032

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | Draft CR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 33.512 based on GSMA NESASG agreements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to GSMA NESASG review, several corrections to 33.512 are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Complementing new references, terms, abbreviations, and missing sub-headings. - Replacing modal verbs with normative texts. - Correcting format and font issues. - Correcting the reference to the threat in TR 33.926. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Low quality of SCAS documents. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 4.2.2.1.3, 4.2.2.1.4, 4.2.2.3.1, 4.2.2.3.2, 4.2.2.3.3, 4.2.2.7, 4.2.2.8.1, 4.2.2.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The complemented clause number of the threat (marked in yellow) referred by 4.2.2.9.1 in the 12th change is K.2.10.X for now, will be changed to K.2.10.2 after S3-252775 is approved during this meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANGES****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TS 23.003: "Numbering, addressing and identification".

[5]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[6]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[7]	Void

[8]	3GPP TS 23.501: "System Architecture for the 5G System".

[9]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[10]	3GPP TS 29.509: "5G System; Authentication Server Services".

[11]	3GPP TR 33.916: "Security Assurance Methodology (SECAM) for 3GPP network products".

**************** END of 1st CHANGE****************

**************** START of 2nd CHANGE****************

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Network Function: As defined in TS 23.501 [8].

Network product: As defined in TR 33.916 [11].

Network product class: As defined in TR 33.916 [11].

Pcap file: A file format used to store network packet data captured from a network interface.

Screenshot: A digital image that shows the contents of a display.

Vulnerability: As defined in TR 33.916 [11].

3.2	Symbols

Void.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

HTTP2	Hypertext Transfer Protocol Version 2

JSON	JavaScript Object Notation

**************** END of 2nd CHANGE****************

**************** START of 3rd CHANGE****************

4.2.2.1.3	NAS based redirection from 5GS to EPS

Requirement Name: NAS based redirection from 5GS to EPS

Requirement Reference: TS 33.501 [2], clause 6.16.4, TS 23.501 [8], clause 5.31.3.

Requirement Description: As specified in TS 33.501 [2], clause 6.16.4, when a UE initiates registration procedure with the AMF, the AMF may redirect the UE from 5GC to EPC by including a EMM cause indicating to the UE that it shall not use 5GC, as described in clause 5.31.3 in TS 23.501 [2]. The following requirements apply to Registration Reject message with an EMM cause which indicates to the UE that the UE shall not use 5GC:

-	the AMF only sends such a Registration Reject message once NAS security has been established between the AMF and the UE; and

-	the UE only acts upon such Registration Reject message if received integrity protected and if UE has verified the integrity of the Registration Reject message successfully.

NOTE 1:	Void

In addition, in networks that support CIoT features in both EPC and 5GC, the operator may steer UEs from a specific CN type due to operator policy, e.g. due to roaming agreements, Preferred and Supported Network Behaviour, load redistribution, etc. Operator policies in EPC and 5GC are assumed to avoid steering UEs back and forth between EPC and 5GC.

Threat Reference:  TR 33.926 [6], clause K.2.8, NAS based redirection from 5GS to EPS in 5G CIoT

Test Case:

Test Name: TC_AMF_REDIRECTION_5GS_EPS

Purpose:

Verify that AMF under test does not send a Registration Reject message containing an EMM cause indicating to the UE that the UE shall not use 5GC, if NAS security is not established.

NOTE 2: Void

Pre-Conditions:

-	AMF under test supports the security handling in CIoT.

-	Test environment with a CIoT UE. The UE may be simulated.

-	AMF under test is connected in emulated/real network environment.

-	Tester configures the operator policy of the AMF that all the UEs sending initial registration request should be redirected from 5GS to EPS.

Execution Steps

1.	The tester triggers the UE to initiate an initial registration procedure with the AMF.

2.	The AMF under test determines that the UE shall not use 5GC and needs to redirect the UE from 5GC to EPC.

3.	The AMF under test sends a Registration Reject message with a 5GMM cause indicating to the UE that the UE shall not use 5GC.

Expected Results:

The NAS SMC is performed before sending the Registration Reject message.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

4.2.2.1.4	NAS integrity failure

Requirement Name: NAS integrity failure

Requirement Reference: TS 33.501 [2] clause 6.4.3.3.

Requirement Description: In case of failed integrity check (i.e. faulty or missing NAS-MAC) is detected after the start of NAS integrity protection, the concerned message shall be discarded except for some NAS messages specified in TS 24.501.

Threat Reference:  TBD

Test Case:

Test Name: TC_AMF_NAS_INTEGRITY_FAILURE

Purpose:

Verify that AMF under test drops messages in case the NAS integrity fails or is missing.

Pre-Conditions:

-	Test environment with UE. The UE may be simulated.

-	AMF under test is connected in emulated/real network environment.

- 	NAS Integrity algorithm different than NIA0 is used.

Execution Steps

Test case 1 (wrong NAS-MAC):

1. The tester triggers the UE to initiate an initial registration procedure with the AMF.

2. The AMF sends the Security Mode Complete message to the UE.

3. After the Security Mode Complete message, send a NAS message from the UE to the AMF with a wrong NAS-MAC. The message used shall not be an exception in TS 24.501 [5].

Test case 2 (missing NAS-MAC):

1. The tester triggers the UE to initiate an initial registration procedure with the AMF.

2. The AMF sends the Security Mode Complete message to the UE.

3. After the Security Mode Complete message, send a NAS message from the UE to the AMF removing the NAS-MAC field. The message used shall not be an exception in TS 24.501 [5].

Expected Results:

In both test cases, the AMF discards the NAS messages.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

**************** END of 3rd CHANGE****************

**************** START of 4th CHANGE****************

4.2.2.3.1	Replay protection of NAS signalling messages

Requirement Name: Replay protection of NAS signalling messages

Requirement Reference: TS 33.501 [2], clause 5.5.2.

Requirement Description: The AMF supports integrity protection and replay protection of NAS-signalling as specified in TS 33.501 [2], clause 5.5.2.

Threat References: TR 33.926 [6], clause K.2.3.1, Bidding Down

Test case:

Test Name: TC_NAS_REPLAY_AMF

Purpose:

Verify that the NAS signalling messages are replay protected by AMF over N1 interface between UE and AMF.

Procedure and execution steps:

Pre-Condition:

-	AMF network product is connected in emulated/real network environment.

-	Tester shall have access to the NAS signalling packets sent between UE and AMF over N1 interface.

-	Tester shall ensure that integrity protection algorithm other than NIA0 is used.

Execution Steps:

1.	The tester shall capture the NAS Security Mode Command procedure taking place between UE and AMF over N1 interface using any network analyser.

2.	The tester shall filter the NAS Security Mode Complete message by using a filter.

3.	The tester shall replay the captured NAS Security Mode Complete message.

4.	The tester shall check whether the replayed NAS Security Mode Complete message was not processed by the AMF by capturing traffic over the N1 interface to see if no corresponding response message was sent by the AMF. If applicable, AMF application logs could be checked for the rejection of the replayed NAS Security Mode Complete message.

Expected Results:

The NAS signalling messages sent from the UE to the AMF over N1 interface are replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

**************** END of 4th CHANGE****************

**************** START of 5th CHANGE****************

4.2.2.3.2	NAS NULL integrity protection

Requirement Name: NAS NULL integrity protection

Requirement Reference: TS 33.501 [2], clause 5.5.2

Requirement Description: NIA0 is disabled in AMF in the deployments where support of unauthenticated emergency session is not a regulatory requirement as specified in TS 33.501 [2], clause 5.5.2

Threat References: TR 33.926 [6], clause K.2.3.3, NAS NULL integrity protection

Test Case:

Test Name: TC_NAS_NULL_INT_AMF

Purpose:

Verify that NAS NULL integrity protection algorithm is used correctly.

Pre-Conditions:

-	Test environment with a UE. The UE may be simulated.

-	The AMF under test is configured to initiate authentication for both emergency and non-emergency registrations.

Execution Steps

Test case A:

1.	The tester triggers the UE to initiate an emergency registration.

2.	The AMF derives the KAMF and NAS signalling keys after successful authentication of the UE.

3.	The AMF sends the NAS Security Mode Command message to the UE containing the selected NAS algorithms.

Test case B:

1.	The tester triggers the UE to initiate a non-emergency registration.

2.	The AMF derives the KAMF and NAS signalling keys after successful authentication of the UE.

3.	The AMF sends the NAS Security Mode Command message to the UE containing the selected NAS algorithms.

Expected Results:

In both emergency and non-emergency registrations, the UE was successfully authentication and the integrity algorithm selected by the AMF in the NAS SMC message is different from NIA0.

The NAS Security Mode Command message is integrity protected by the AMF.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

4.2.2.3.3	NAS integrity algorithm selection and use

Requirement Name: NAS integrity algorithm selection and use

Requirement Reference: TS 33.501 [2], clause 6.7.1

Requirement Description: The AMF initiates a NAS security mode command procedure, and include the chosen algorithm and UE security capabilities (to detect modification of the UE security capabilities by an attacker) in the message to the UE (see sub-clause 6.7.2 of TS 33.501 [2]). The AMF selects the NAS algorithm which have the highest priority according to the ordered lists as specified in TS 33.501 [2], clause 5.5.2.

Threat References: TR 33.926 [6], clause K.2.3.2, NAS integrity selection and use

Test Case:

Test Name: TC_NAS_INT_SELECTION_USE_AMF

Purpose:

Verify that the AMF selects the NAS integrity algorithm which has the highest priority according to the ordered list of supported integrity algorithms and is contained in the 5G security capabilities supported by the UE.

Verify that the selected NAS security algorithm is being used.

Pre-Conditions:

-	Test environment with a UE containing its 5G security capabilities, AUSF and UDM. The UE, AUSF and UDM may be simulated.

-	The list of ordered NAS integrity algorithms is configured on the AMF under test.

-	The tester is able to configure the list of ordered NAS integrity algorithms on the AMF under test.

Execution Steps:

1)	The tester triggers the UE to send a Registration Request with Initial Registration type to the AMF under test.

2)	The tester filters the Security Mode Command and Security Mode Complete messages.

3)	The tester examines the selected integrity algorithm in the SMC against the list of ordered NAS integrity algorithm and the 5G security capabilities supported by the UE. The tester examines the MAC verification of the Security Mode Complete at the AMF under test.

4)	The tester changes the default order of the list of ordered NAS integrity algorithms on the AMF to one other valid configuration and repeats step 1-3 once.

Expected Results:

The selected integrity algorithm has the highest priority according to the list of ordered NAS integrity algorithm and is contained in the UE 5G security capabilities.

The MAC verification of the Security Mode Complete message is successful.

Expected format of evidence:

Logs and communication flow saved in a .pcap file.

**************** END of 5th CHANGE****************

**************** START of 6th CHANGE****************

4.2.2.7	RRCRestablishment in Control Plane CIoT 5GS Optimization

Requirement Name: RRCRestablishment in Control Plane CIoT 5GS Optimization

Requirement Reference: TS 38.413 [9], clause 8.3.8.2

Requirement Description: "Upon receiving the RAN CP RELOCATION INDICATION message, the AMF shall authenticate the request using the NAS-level security information received in the UL CP Security Information IE and if the authentication is successful initiate the Connection Establishment Indication procedure including NAS-level security information in the DL CP Security Information IE.

In case the AMF cannot authenticate the UE's request, the CONNECTION ESTABLISHMENT INDICATION message does not contain security information, and the NG-RAN node fails the RRC Re-establishment.

In case of authentication failure, the NG-RAN node and the AMF should locally release the allocated NG resources, if any." as specified in TS 38.413 [9], clause 8.3.8.2.

Threat References: TR 33.926 [5], clause K.2.9.1 –Failed Verification of UE Identity during RRC Reestablishment Procedure for CP CIoT 5GS Optimization.

Test Case:

Test Name: TC_AMF_REEST_CP_CIOT

Purpose: To verify that the verification of RRC Reestablishment is applied correctly.

Pre-Conditions:

-	AMF under test is able to support the CIoT scenario.

-	Test environment with UE and ng-eNB, which may be simulated. The UE is using Control Plane CIoT 5GS Optimization.

-AMF

Capability:

Ability to support the CIoT senario.

Execution Steps:

Test Case A

1)	The tester triggers the UE to send the RRC Connection Reestablishment Request message to the ng-eNB.

2)	The ng-eNB sends RAN CP RELOCATION INDICATION message to the AMF.

Test Case B

1)	The tester triggers the UE to send the RRC Connection Reestablishment Request message to the ng-eNB.

2)	The ng-eNB sends RAN CP RELOCATION INDICATION message to the AMF. The ng-eNB modifies UL NAS MAC in UL CP Security Information

Expected Results:

For test case A, the AMF sends CONNECTION ESTABLISHMENT INDICATION to the ng-eNB, and DL CP Security Information is included.

For test case B, the AMF sends CONNECTION ESTABLISHMENT INDICATION to the ng-eNB, and DL CP Security Information is not included.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

**************** END of 6th CHANGE****************

**************** START of 7th CHANGE****************

4.2.2.8.1	Validation of S-NSSAIs in PDU session establishment request

Requirement Name: validation of S-NSSAIs in PDU session establishment request

Requirement Reference: TS 24.501 [5], clause 5.4.5.2.5

Requirement Description: As specified in TS 24.501 [5], clause 5.4.5.2.5, if the Request type IE is set to "initial request" and the S-NSSAI IE contains an S-NSSAI that is not allowed by the network, then the AMF sends back to the UE the 5GSM message which was not forwarded as specified in subclause 5.4.5.3.1 case e) or case f); of TS 24.501 [5].

Threat References: TR 33.926 [6], clause K.2.10.1, Incorrect Validation of S-NSSAIs

Test Case:

Test Name: TC_VALIDATION_SNSSAI_IN_PDU_REQUEST

Purpose:

Verify that S-NSSAIs which are not within Allowed NSSAI list are not accepted by the AMF under test in PDU session establishment procedure.

Pre-Conditions:

-	AMF under test supports the Network Slice Specific Authentication and Authorization scenario.

-	Test environment with UE, UDM, SMF and NSSAAF, which may be simulated.

-	The tester configures UDM with an S-NSSAI that require Network Slice-Specific Authentication and Authorizationin in UE’s subscription information.

-AMF Capability: Ability to support Network Slice Specific Authentication and Authorization scenario.

Execution Steps

Test Case A

1)	The tester triggers the UE to send the S-NSSAI that require NSSAA to the AMF under test using registration request message.

2)	After receiving the NSSAA request from the AMF, the NSSAAF sends EAP success to AMF.

3)	The UE sends PDU session establishment request to the AMF with the S-NSSAI.

Test Case B

1)	The tester triggers the UE to send the S-NSSAI that require NSSAA to the AMF under test using registration request message.

2)	After receiving the NSSAA request from the AMF, the NSSAAF sends EAP failure to AMF.

3)	The UE sends PDU session establishment request to the AMF with the S-NSSAI.

Expected Results:

For test case A, the AMF continues the PDU session establishment procedure by sending a Nsmf_PDUSession_CreateSMContext Request to the SMF.

For test case B, the AMF aborts the PDU session establishment procedure by sending back the 5GSM message to the UE.

Expected format of evidence

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

List of allowed S-NSSAIs.

**************** END of 7th CHANGE****************

**************** START of 8th CHANGE****************

4.2.2.9.1	NSSAA revocation

Requirement Name: NSSAA revocation

Requirement Reference: TS 33.501 [2], clause 16.5

Requirement Description: If no S-NSSAI is left in Allowed NSSAI for an access after the revocation, and no Default NSSAI can be provided to the UE in the Allowed NSSAI or a previous NSSAA failed for the Default NSSAI over this access, then the AMF executes the Network-initiated Deregistration procedure for the access as described in subclause 4.2.2.3.3 in TS 23.502 [8], and it includes in the explicit De-Registration Request message the list of Rejected S-NSSAIs, each of them with the appropriate rejection cause value; as specified in TS 33.501[2], clause 16.5.

Threat References: TR 33.926 [6], clause K.2.10.X, Failure to deregister UE after NSSAA revocation

Test Case:

Test Name: TC_NSSAA_REVOCATION

Purpose:

Verify that AMF deregisters UE when, after slice specific authorization revocation, there is no allowed NSSAI or Default NSSAI that can be used by UE.

Pre-Conditions:

-	AMF under test supports Network Slice Specific Authentication and Authorization.

-	Test environment with UE. The UE may be simulated.

-	The AMF under test is configured with one specific S-NSSAI in the Allowed NSSAI and no default S-NSSAI.

-	The UE is registered at the AMF using the specific S-NSSAI configured in the AMF.

Execution Steps

A message requesting the AMF under test to revoke the authorization of the S-NSSAI in the Allowed NSSAI is created simulated and sent to the AMF under test by the tester.

Expected Results:

The Deregistration Request message is sent by the AMF under test to the UE.

The Deregistration Request message includes the list of rejected S-NSSAIs, each of them with the appropriate rejection cause value.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

NOTE 1:	Void

**************** END of CHANGES****************