3GPP TSG-SA3 Meeting #124	merges 3463 and 3627 in draft S3-253821-r1

# Wuhan, CN, 13 - 17 Oct 2025

Source:	Huawei, HiSilicon, Samsung

# Title:                            New KI on Security Protection of Compromised WAB Nodes and Core Network Measures.

Document for:	Approval

Agenda item:	5.2.14

Spec:                         TR 33.724

Work Item:                 FS_5G_WAB_SEC

Comments

This contribution proposes a new key issue on the integrity, authenticity, and confidentiality of messages delivered from WAB components during their interaction with the 5G core and RAN, as their deployment a mobile environment may expose the network to rogue signalling, topology manipulation, and persistent infiltration, requiring dedicated mitigation mechanisms.

* * * First Change * * * *

# 2	References

[x1]	3GPP TR 33.745: “Study on security aspects of 5G Next Radio (NR) Femto”.

[x2]	3GPP TS 33.320: “Security of Home Node B (HNB) / Home evolved Node B (HeNB)”.

* * * Second Change * * * *

## Y.X	Key Issue #X: Security Protection of Compromised WAB Nodes and Core Network Measures

### Y.X.1	Key issue details

Wireless Access Backhaul (WAB) nodes, consist of a WAB-gNB (gNB-like functionality) and a WAB-MT (UE-like functionality). These nodes operate in non-trusted environments and may serve as moving backhaul nodes for the 5GS, establishing NG, Xn, and OAM interfaces over PDU sessions through 3GPP backhauls. While 3GPP TR 33.745 [x1] studied NR Femto security and reused procedures from TS 33.320 [x2], security concerns specific to WAB nodes particularly compromised WAB nodes in untrusted environments remain unaddressed.

Additionally, core network components may not be equipped to detect anomalous behavior from compromised WAB-gNBs, due to the decentralized and mobile nature of such nodes. The compromised WAB nodes could lead to topology poisoning, signalling storms, or user-plane hijacking.

This key issue aims to address the security issues introduced by compromised WAB nodes, where failure to protect the integrity, authenticity, and confidentiality of messages delivered from WAB-gNB and WAB-MT components can expose the 5GS to topology spoofing, rogue signalling, and persistent infiltration.

### Y.X.2	Security threats

Potential security threat:

- Rogue WAB-gNB Injection: A compromised WAB node may inject unauthorized signalling or reroute traffic maliciously, particularly via spoofed message. Furthermore, a compromised WAB-gNB can attempt to broadcast unauthorized network identifiers or initiate rogue Xn association attempts with neighbouring gNBs causing service disruption.

- Topology Manipulation and Signalling Abuse: Moving WAB nodes may falsely report neighbour relationships via Xn or behave inconsistently across locations, leading to incorrect handover decisions, topology poisoning, or signalling loops.

- Persistent Threat via Dual Roles: Since WAB-MT behaves like a UE and WAB-gNB like a gNB, a compromised WAB can act in both roles to stage cross-layer attacks, bridging between RAN and CN trust domains.

### Y.X.3	Potential security requirements

The 3GPP system shall support security mechanisms to mitigate risks from compromised WAB nodes, preventing topology spoofing, rogue signalling, and mobility-related traceability threats.

* * * End of Changes * * * *