3GPP TSG-SA3 Meeting #124	S3-253684-r2

# Wuhan, China, 13 – 17 October 2025														merges 3270, 3271

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0056 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information protection during command procedure - corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the current specification, the AIoT device always derives both Kcommand_enc and Kcommand_int regardless of whether the received AIOT NAS Command Request is ciphered or not. As integrity protection is mandatory, the AIoT device is expected to always derive Kcommand_int, however, ciphering is optional, and as such, the AIoT device should only derive Kcommand_enc if integrity verification is succesfull and the ciphering indication in the AIOT NAS Command Request message indicates that ciphering is activated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes cover: - Editorials and corrections - Derivation of Kcommand_enc being conditional on successful integrity verification and ciphering indication indicating that ciphering is activated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification text and unnecessary key derivation performed by the device even when ciphering is de-activated or integrity verification fails. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The procedure as described in clause 5.2.2 shall be performed. AIOTF acquire the KAIOTF key to be used for command protection. The derivation of KAIOTF key is specified in Annex A.3.

3.	The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, and the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether ciphering is activated to NG-RAN. The derivation of KCommand_enc and KCommand_int is specified in Annex A.4.

NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If ciphering is activated (i.e. confidentiality algorithm is not null-scheme), the AIOT NAS Command Request message is partly ciphered with the exception that the indication on whether ciphering is activated (i.e. the selected confidentiality algorithm is 128-NEA2) are in clear text.

4.	The NG-RAN shall send a R2D message containing the protected AIOT NAS Command Request as specified in TS 38.300 [3] and TS 38.391 [6].

NOTE 2: It is left to implementation when KAIOTF is derived on the device.

5.	The device shall derive the KCommand_enc, KCommand_int, and process the protected AIOT NAS command Request. Upon successful integrity verification, the AIoT device shall decipher the AIOT NAS Command Request in case ciphering is activated. The AIoT device shall construct an AIOT NAS Command Response and protect it based on the derived key(s).

6.	The AIoT device shall send a D2R message containing the protected AIOT NAS Command Response to the NG-RAN as specified in TS 38.300 [3] and TS 38.391 [6].

7.	The NG-RAN shall forward the protected AIOT NAS Command Response to the AIOTF.

8-9.	The AIOTF shall verify the integrity of the AIOT NAS Command Response message. If the verification of integrity is successful, the AIOTF shall decipher it in case ciphering is activated. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

NOTE 2: It is assumed that there is only one round of command procedure per device following an inventory procedure. Since the KAIOTF key is fresh, there is no need for additional freshness parameters for replay protection.

NOTE 3: It is assumed that in the present document no new algorithms will ever be introduced for information protection during command procedure.

*** END OF CHANGES ***