3GPP TSG-SA3 Meeting #124	draft_S3-253704-r8

# Wuhan, China, 13 – 17 October 2025	merger of S3-253290, S3-253392, S3-253299

Source:	Interdigital, CATT, Huawei (?), OPPO (?)

Title:	New Key Issue on Security, and Authorization for Exposure of UE Data towards OTT Servers

Document for:	Approval

Agenda item:	5.2.6

Spec:	3GPP TR 33.785

Version:	0.0.0

Work Item:	FS_AIML_CN_Ph2_SEC

Comments

This contribution proposes a new key issue on exposure security, authorization, for UE data towards OTT servers, and focuses strictly on the exposure interface between the core network and OTT.

* * * First Change (all text new) * * * *

## 5.X	Key Issue #X: Security, and Authorization for Exposure of UE Data towards OTT Servers

### 5.X.1	Key issue details

As studied in TR 23.700-04 [2], training data for AI/ML-based NR air interface operation with UE-side model training may be transferred via the 5G Core (5GC) and then exposed to external OTT servers. The exposure of such UE-related data outside the 3GPP domain introduces security and that need to be addressed at the exposure interface (e.g., via NEF

The exposure interface requires mechanisms to:

- Authenticate OTT servers before any data exposure.

- Authorize and apply service-specific access control to restrict exposed data to what is necessary for the OTT server.

- Provide confidentiality, integrity, and replay protection of the exposed data during transport.

- Ensure that exposure of UE-related data complies with user consent

### 5.X.2	Security threats

Unauthenticated or impersonating OTT servers could obtain sensitive UE-related data.

Without authorization, OTT servers can abuse UE-related data exposure services. .

Leakage, tampering, or replay of UE-related data in transit across the exposure interface could compromise integrity, confidentiality.

Exposure of UE information without proper consent may violate regulations and create liabilities for the MNO.

### 5.X.3	Potential security requirements

The 5GS shall support mutual authentication between the 5GC exposure function(s) (e.g., NEF) and OTT servers handling UE-related data.

The 5GS shall support authorization mechanisms for services related to exposure of UE-related data to the OTT server.

The 5GS shall support confidentiality, integrity, and replay protection for UE-related data during transfer to the OTT via NEF.