3GPP TSG-SA3 Meeting #124	draft_S3-253707-r1

# Wuhan, China, 13 – 17 October 2025	revision of S3-253291

Source:	Interdigital

Title:	New Solution: Security for Operator-Controlled UE Data Collection and Exposure

Document for:	Approval

Agenda item:	5.2.6

Spec:	3GPP TR 33.785

Version:	0.0.0

Work Item:	FS_AIML_CN_Ph2_SEC

Comments

This contribution proposes a new solution based on architectural baseline of TR 23.700-04 Solution #19. The solution covers authentication, authorization, transport security, privacy protection, user consent enforcement, replay/freshness protection, and secure exposure via NEF or equivalent.

* * * First Change (all text new) * * * *

## 6.X	Solution #X: Security for Operator-Controlled UE Data Collection and Exposure

### 6.X.1	Introduction

This solution addresses Key Issue #1 and Key Issue #X.

This solution builds on TR 23.700-04 Solution #19 (standardized transfer of standardized data over UP for UE-side data collection) and introduces security enhancements in the 5GS for: (i) secure UE connection setup and data transfer with a Data Collection Function (DCF), and (ii) secure, authorized, and privacy-preserving exposure of UE-related data towards OTT servers via the 5GC exposure function (e.g., NEF).

### 6.X.2	Solution details

Architecture scope and roles

- DCF in the MNO domain manages Data Collection Profiles (DCPs) and orchestrates UE data collection and transfer, consistent with Solution #19.

- The exposure interface uses the NEF (or equivalent 5GC exposure function) to expose authorized subsets of collected data with any applicable post-processing prior to forward to OTT servers.

### Security functions

### 1) Mutual authentication and session protection (UE↔DCF)

- The UE establishes an application-layer secure association with the DCF using shared key derived from network credentials. Transport security (e.g., TLS/QUIC) is bound to the shared key. Options for shared key derivation are:

- Option #1: AKMA-based keys (TS 33.535 [x]). DCF acts as an AF, and obtains KAF from the AAnF over SBI.

- Option #2: KSEAF or KAMF derived shared key. DCF obtains the shared key from AMF/SEAF over SBI.

Editor’s note:	shared key derivation and service operations for exchange of shared key between AMF/SEAF and DCF (option #2) are FFS.

### 2) UE authorization and policy enforcement

### - The DCF authorizes a UE to participate per DCP using subscription, consent, and operator policy.

### 3) Data minimization and privacy protection

### - Before exposure outside the MNO domain, the DCF applies per parameter pseudonymization, masking, or filtering per DCP visibility configuration.

### - Identifiers not authorized for external exposure can be replaced by DCF with pseudonymous identifiers.

### - DCF checks whether UE location information can be exposed externally based on existing LCS privacy profile, as per TS 23.273 [y].

Editor’s note:	whether and what data can be subject to data minimization is FFS based on RAN2 progress on data collection parameters definition.

### 4) Consent enforcement inside the Core Network

### - The DCF acts as the consent enforcement point for data collection from the UE, i.e., checks consent from UDM/UDR for permissions, as per TS 33.501 [z], Annex V.

### 5) Exposure toward OTT servers (NEF-facing)

### - NEF enforces mutual authentication with OTT server.

### - Exposure is constrained to authorized datasets and based on consent.

Editor’s note:	whether and how user consent exposure applies will be decided by SA3 based on SA6 progress.

NOTE: Above points 1, 2 and 4 address Key Issue #1, while points 3 and 5 address Key Issue #X.

### 6.X.3  Evaluation

Editor’s note:	evaluation is FFS.