# 3GPP TSG-SA3 Meeting #124																		    S3-253710-r1

# Wuhan, China, 13th – 17th October. 2025

Source:	CATT

Title:	New solution for Security of UE connection setup with Data collection NF

Document for:	Approval

Agenda item:	5.2.6

Work Item:	FS_AIML_CN_Ph2_SEC

Comments

The contribution proposes to add a new solution for security of connection between UE and Data Collection NF.

Proposed Changes

* * * First Change * * * *

## 6.Y Soution #Y: New solution for Security of UE connection setup with Data collection NF

### 6.Y.1 Introduction

This solution addresses requirements of key issue on Security of UE connection setup with Data collection NF, particularly by hop-by-hop security. For authorization and user consent check between UE and data collection NF, it proposes detailed authorization checks against UE subscription data and operator policies at the data collection NF. For the sake of simplicity, the following document will use DCF for the data collection NF. The name of this data collection NF will follow SA2’s decision.

### 6.Y.2 Soultion details

Figure 6.Y.2-1 Security of UE connection setup with Data Collection NF

1. The UE model training entity/server sends a request to the DCF to collect UE data for UE side model training.

2. The DCF checks subscription data for UE data collection and transfer from the UDM.

3. After successful authorization and user consent check, UE and UPF, DCF sends a request to SMF to establish a secure UP connection.

4. The procedure of secure UP connection shall reuse existing UP security mechanisms from TS 33.501 [x1] between UE and gNB, reuse exiting NDS/IP specified in TS 33.210 [x2] between gNB and DCF.

Editor’s Note: the authentication between user and data collection NF is FFS.

Editor’s Note: user consent is FFS.

### 6.Y.3 Evaluation

TBD

* * * End of Change * * * *