3GPP TSG-SA3 Meeting #124	draft_S3-253712-r1

# Wuhan, China, 13 – 17 October 2025	revision of S3-253292

Source:	Interdigital

Title:	New Solution for MPQUIC/TLS using PSK derived from KgNB

Document for:	Approval

Agenda item:	5.2.5

Spec:	TR 33.778

Version:	0.0.0

Work Item:	FS_PSK_MPQ_TLS

Comments

It is proposed to add this new solution for the support for MPQUIC/TLS using PSK, between UE and UPF.

* * * First Change (all text new)* * * *

## 6.X	Solution #X: MPQUIC/TLS using PSK derived from KgNB

### 6.X.1	Introduction

This solution addresses Key issue #1 by enabling a secure UP communication channel between the UE and the UPF. The approach leverages the current KgNB to derive a pre-shared key (UPF_PSK) and a corresponding identifier (UPF_PSK ID). The UPF_PSK/ID is delivered to the UPF and then used for a mutual-authentication and key exchange using TLS 1.3 PSK psk_dhe_ke.

### 6.X.2	Solution details

Assumptions and scope:

- UE is registered to the 5GS and has established a KgNB with the network.

- Distribution path for UPF_PSK/ID: AMF/SMF → UPF over N2/N4.

Key derivation and identifiers:

- UE and AMF derive UPF_PSK and UPF_PSK ID using current KgNB.

- Input parameters for the KDF include at least the PDU Session ID and a freshness parameter. UPF_PSK derivation can additionally be bound to the selected UPF identity (e.g., FQDN or IP).

Editor’s Note: Motivation for derivation of UPF_PSK using KgNB is FFS.

Setup procedure (PDU Session establishment):

- UE requests a PDU Session indicating support for MPQUIC/TLS using PSK.

- SMF selects a suitable UPF and provides UE with UPF addressing (e.g., IP, port) and obtains UPF_PSK/ID from AMF.

- AMF derives UPF_PSK/ID from current KgNB. SMF forwards UPF_PSK/ID to UPF via N4.

- Upon successful PDU Session Establishment, UE initiates MPQUIC/TLS with the UPF using UPF_PSK, referencing UPF_PSK ID for UPF to locate and use UPF_PSK to perform mutual authentication with the UE.

UPF_PSK update triggers and handling:

- UE CM-IDLE → CM-CONNECTED transition:

- UE and AMF derive new UPF_PSK/ID.

- AMF/SMF updates the UPF with the new UPF_PSK.

- UE initiates MPQUIC/TLS with the UPF using the new UPF_PSK/ID.

Editor’s Note: Handling of UPF_PSK derivation in Xn handover scenario is FFS

### 6.X.3	Evaluation

Editor’s Note: Evaluation is FFS