3GPP TSG-SA3 Meeting #124	draft_S3-253725-r2

# Wuhan, China 13th - 17th October 2025

Source:	ZTE

Title:	New solution on Derivation of Satellite-Specific NAS keys for S&F Operation

Document for:	Approval

Agenda item:	5.2.9

Spec:	3GPP TR 33.700-30

Version:	0.1.0

Work Item:	FS_5GSAT_Ph4_SEC

Comments

The contribution proposes to add a new solution for key issue #1.

Proposed Changes

*************** Start of the change ****************

6.Y	Solution #Y: Derivation of Satellite-Specific NAS keys for S&F Operation

6.Y.1	Introduction

This solution addresses Key Issue #1: Authenticated UE to exchange NAS messages with multiple satellites in split-MME architecture.

This solution proposes a mechanism to derive unique NAS integrity and encryption keys for each satellite by using the satellite ID as an additional input parameter during the NAS key derivation.

6.Y.2	Solution details

In this solution, it is proposed to derive distinct set of NAS keys for each satellite from the common root key KASME. The satellite-specific NAS keys are derived by the UE and the network using the KDF as specified in TS 33.220 [x].

For a serving Satellite n, the NAS integrity key KNASint and the NAS encryption key KNASenc are derived from the KASME with the following parameters as input:

-	FC = 0xxx

-	P0 = algorithm type distinguisher

-	L0 = length of algorithm type distinguisher (i.e. 0x00 0x01)

-	P1 = algorithm identity

-	L1 = length of algorithm identity (i.e. 0x00 0x01)

-  P2 = Satellite ID n.

-  L2: length of Satellite ID n.

Where Satellite ID is an identifier uniquely indicating an MME-onboard. The Satellite ID of a given satellite is broadcast by the eNB within the SIB31 and the Satellite ID of the satellites that might be serving a given UE are included within the S&F Monitoring List, which is sent by the MME to indicate the satellite(s) that the UE may (re)-attempt NAS procedures (TS 23.401 clause 4.13.9.1).

As a result of using satellite-specific keys, the UE and each MME-onboard maintain independent pairs of NAS COUNT for their mutual communication. The NAS COUNTs are not synchronized with other satellites.

Editor’s Note: When the NAS keys are generated in UE and MME-onboard is FFS.

Editor’s Note: How to deal with the warp around case is FFS.

Editor’s Note: How the MME-ground manages and reconciles the multiple UE security context of the same UE for multiple satellites is FFS.

Editor’s Note: How to indicate to the UE whether the solution of the separate NAS keys is implemented or not is FFS.

6.Y.3	Evaluation

TBD

Editor’s Note:The impact for key generation on MME-onboard is FFS.

*************** End of the change ****************