3GPP TSG-SA3 Meeting #123	S3-252625

Goteborg, Sweden, 25 - 29 August 2025

————————————————————————————————————————————————

Source:	ZTE

Title:	New key issue on UE security context synchronization between multiple 	satellites in S&F Satellite operation

Document for:	Approval

Agenda Item:	6.1.10

- 1	Decision/action requested

Approve the new KI for UE security context synchronization between multiple satellites in S&F Satellite

- 2	References

[1]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE) Security architecture".

[2]	S3-250932 “DP on handling NAS counters in S&F Mode when the UE registration is valid in multiple satellites”

- 3	Rationale

In R19, SA3 studies security aspects of Store and Forward Satellite operation, the conclusion is captured in TS 33.401 [1]. According to clause N.2 of TS 33.401, the latest NAS security context of the UE needs to e available at the MME-onboard in split MME architecture. However, as highlighted by S3-250932, UE may interact with several MME-onboard instances for the same registration, creating cross-satellite UE context issues—especially NAS COUNT desynchronization [2]. This requires frequent context synchronization through feeder link. To reduce the signalling overhead in the feeder link, UE security context synchronization mechanism needs to be studied in phase 4.

- 4	Detailed proposal

*************** Start of the 1st Change ****************

  - 5.X	Key issue #X: UE security context synchronization between multiple satellites in S&F Satellite operation

  - 5.X.1 Key issue details

The security aspects for Store and Forward (S&F) Satellite operation have been specified in TS 33.401 [x]. According to the security requirements of  split MME architecture, the latest NAS security context of the UE needs to e available at the MME-onboard. However, for S&F Satellite operation with multiple satellites, those satellites can interact with the same UE successively, while holding their own copies of the NAS context for the UE. Due to the intermittent and time delay of the service link/feeder link, the uplink and downlink NAS COUNT for the same UE are different on different satellites and may lag behind the actual count of the UE.

For such cases, the UE context stored at the MME-onboard may need to be frequently synchronized with the UE  through NAS SMC, or synchronized with MME-ground via feeder link, which will cause signalling overhead. This Key issue studies mechanism for UE security context synchronization between multiple satellites without performing NAS SMC between the UE and satellites.

  - 5.X.2 Security threats

Without NAS COUNT synchronization, the UL NAS COUNT stored in the satellite may be smaller than the ones stored the UE side. Attackers may conduct replay attack by intercepting and replaying old NAS messages.

  - 5.X.3 Potential Security requirements

The 5G system shall provide a mechanism to permit an authenticated UE to exchange NAS messages with multiple satellites without performing NAS SMC between the UE and satellites.

*************** End of the 1st Change ****************

*************** Start of the 2nd Change ****************

  - 2	References

[x]	3GPP TS 33.401: "3GPP System Architecture Evolution: Security Architecture".

*************** End of the Changes ****************