3GPP TSG-SA3 Meeting #123	S3-252914

Goteborg, Sweden, 25 – 29 August 2025

# Source:	IIT Bombay

# Title:	pCR to TS 33.502 - Security related Events for Patch Updates

# Document for:	Approval

Agenda Item:	5.1.1

# 1	Decision/action requested

It is proposed to approve this pCR for incorporation into the draft TS 33.502

# 2	References

[1]	3GPP TS 33.502 v0.0.1

# 3	Rationale

Based on approved WID (SP-250876), the working task needs to specify the general requirements for security events handling and collection.

In WT2, it will specify the events that need to be reported including the following.

- 	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

- 	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

- 	The NF collects information about failed authentication and authorization attempts from inbound connections on the SBA layer.

- 	The NF collects information about potential replay attacks on the SBA layer.

- 	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501.

It is proposed to add security requirements for configuration of security related events.

# 6	Security related Events

Editor’s Note: This clause addresses the list and description of the events as well as naming convention for the events.

* * * First Change * * * *

## 6.x	Security events related to patch updates

Security patches are installed in the system as part of feature update or to fix existing vulnerabilities. The updates need to be tracked as the attackers can use this as a means to update existing versions to the one with vulnerabilities or having malicious code. Keeping track of the updates also helps in identifying if any of these programs, packages and libraries are reported to have vulnerabilities and protect against potential attacks through threat hunting.

Patch updates can be leveraged by attackers to break into the system and perform malicious activities such as exfiltration of data or break into other NFs part of the SBA. Collecting patch update events are critical for ensuring traceability, auditing, and monitoring of the update process.

The patch update event log shall include the following details:

- Name of the packages, libraries, and software programs and its version

- Newer version of the packages, libraries, and software programs name

- Time stamp of the update

- Name of the entity performing the update, like authorized user and system updates

  - The identity of the entity initiating the update can contain:

    - Authorized User: User ID performing the update

    - System Process: Name of the automated system or service

- Update Status (e.g., success or failure)

- Network Function Instance ID

The naming convention for patch update security event can have an unique name/ID

“Patch_Update_<Software_Name>”, including the following content in the security logs

package_name <package:old_version>

new_version <package:new_version>

timestamp <update_time>

update_actor <authorized_user|system_process>

update_status <success|failed>

nf_inst_id <network_function_instance_id>.

* * * End of Changes * * * *