3GPP TSG-SA WG4 Meeting #134	S4- 251699

# Dallas, US, 17 – 21 November 2025

Source:	Nokia, Fraunhofer HHI

Title:	Inclusion of NNC to AI_IMS-MED

Agenda item:	10.5

Document for:	Discussion and agreement

# 1. Introduction

Based on the discussion in S4-251698 that shows the need for compression of AI/ML data in IMS-based transport, we propose to add the following changes to the common base CR for AI_IMS-MED.

# 2. Proposed changes

* * * first change * * * *

X		AI/ML formats

X.Y	    Codecs

X.Y.1		General

DCMTSI clients in terminals supporting AI/ML model download and inference shall support NNC decoding as specified in ISO/IEC 15938-17. To enable support of NNC edition 2 [NNC], the NNC syntax element general_profile_idc shall be set equal to 1.

    - The following NNC parameter settings provide functionality for the download of full AI/ML model data and model data updates.

DCMTSI clients in terminals supporting AI/ML model download may set nnr_compressed_data_unit_payload_type equal to NNC_PT_BLOCK, compressed_parameter_types equal to NNR_CPT_LS | NNR_CPT_BN, either dq_flag equal to 1 or codebook_present_flag equal to 1, and shift_idx_minus_1_present_flag equal to 1.

    - NOTE: This configuration enables local scaling adaptation, batch norm folding, either dependent quantization or use of a codebook, and an optimal initialization of probability estimation.

DCMTSI clients in terminals supporting incremental AI/ML model data exchange may set nnr_compressed_data_unit_payload_type equal to NNC_PT_BLOCK, compressed_parameter_types equal to NNR_CPT_LS | NNR_CPT_BN, mps_parent_signalling_enabled_flag equal to 1, parent_node_id_present_flag equal to 1, row_skip_enabled_flag equal to 1, nnr_pre_flag equal to 1, hist_dep_sig_prob_enabled_flag equal to 1, temporal_context_modeling_flag equal to 1, and scan_order greater than 0.

    - NOTE: This configuration enables local scaling adaptation, batch norm folding, the parameter update tree, row skipping, predictive residual coding, history-dependent significance probability estimation, temporal context adaptation, and parallel decoding.

* * * end of first change * * * *

# 2	References

…

[NNC] "ISO/IEC 15938-17:2024 Information technology - Multimedia content description interface - Part 17: Compression of neural networks for multimedia content description and analysis”, Edition 2, ISO/IEC, January 2024.