3GPP TSG-SA4 Meeting #134	S4-251713

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0031 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] WT#2a - Common server- and network-assisted streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The study item identifies the following work topic - WT#2: Server and Network-assisted media streaming, this includes the following topics: - For Common server- and network-assisted streaming as introduced in clause 5.17, and based on the conclusions in clause 6.17, to continue studying Common Media Server Data (CMSD) and its potential benefits in the context of 5G Media Streaming. The study item identifies the following objectives 1. Document the following additional Key Issues in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture: 2. Study collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. 3. Based on existing architectures, develop one or more deployment architectures that address the key topics and the collaboration models. 4. Map the key topics to basic functions and develop high-level call flows. 5. Identify the issues that need to be resolved. 6. Provide candidate solutions including call flows, protocols and APIs for each of the identified issues. 7. Coordinate work with other 3GPP groups e.g. SA2, SA3, SA5, SA6 and others as needed. 8. Coordinate work with external organizations such as SVTA, CTA WAVE, ISO/IEC JTC29 WG3 (MPEG Systems), 5G-MAG, DVB or IETF, as needed. 9. Identify gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The issues are addressed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

## =====  CHANGE (new) =====

## 5.X	Common server- and network-assisted streaming

### 5.X.1	Description

Common Media Server Data (CMSD) [180] provides parameters to enhance media streaming performance. CMSD uses key–value pairs to allow the flow of information about the state of the origin and the intermediary clients. A client may be an intermediary server or a player. Table 5.X.1-1 provides an overview of the supported headers and the keys in CMSD.

Table 5.X.1-1: Overview of the supported headers and the keys in CMSD

| Header | Key | Description |
| --- | --- | --- |
| CMSD-Static | codec, resolution, duration, encodedBitrate | Static information about the media object. |
| CMSD-Dynamic | currentBitrate, bufferLevel, playbackPosition, throughputEstimate | Dynamic information that can change during the session. |
| CMSD-Cache | cacheStatus, cacheHitRatio, cacheExpiration | Information about the cache status of the media object. |
| CMSD-Error | errorCode, errorDescription | Reports errors encountered during the media session. |
| CMSD-Quality | videoQuality, audioQuality, qualityAdjustments | Quality metrics such as video and audio quality. |
| CMSD-User | userID, sessionID, userPreferences | User-specific data to tailor the media experience. |
| CMSD-Session | sessionStartTime, sessionDuration, sessionID | Session-related information. |
| CMSD-Event | playbackStart, pause, resume, stop | Events related to the media session. |
| CMSD-Performance | serverResponseTime, networkLatency, throughput | Performance metrics. |
| CMSD-Content | contentID, contentType, contentDuration | Information about the content being delivered. |

### 5.X.2	Collaboration scenarios

### 5.X.3	Architecture mappings

### 5.X.4	High-level call flows

### 5.X.5	Gap analysis and requirements

### 5.X.6	Candidate solutions

### 5.X.7	Summary and conclusions

## =====  CHANGE (new) =====

## 6.X	Common server- and network-assisted streaming