Agenda item: 	9.7

Source: 	Qualcomm Inc.

Title: 	[FS_3DGS_MED] Use Cases for GS

Document for	Discussion and Agreement

# Introduction

In this contribution, we outline two use cases that motivate carriage and description of 3D Gaussian Splat assets in 3GPP systems. 3D Gaussian Splatting is a point-based representation that supports real time rendering with high visual fidelity. The use cases target consumer capture and distribution and avatar-based communication.

The goal is to identify SA4 work on media format and protocol descriptions.

# Proposed Use Cases

## 2.1 Use case 1: 3DGS for Multimedia Messaging

A sender captures a short video or an image sequence on a phone. A local or cloud process reconstructs a 3D Gaussian Splat asset. The asset is packaged as a self-contained file with required metadata and optional preview thumbnails. The sender shares the asset through RCS or MMS as an attachment using a registered MIME type. The receiver taps to view. The messaging client launches a 3DGS capable viewer that renders the asset interactively. Offline viewing is supported when the asset is packaged in a single file.

The following figure depicts the use case:

## Use case 2: 3D Avatar communication

A user is represented by an avatar that combines a rigged mesh and a 3D Gaussian Splat layer for fine detail. The sender transmits a time aligned animation stream and a static 3DGS as part of the base avatar. The animation stream drives the rig and blendshapes. The renderer composites mesh shading with 3DGS contributions. The approach reduces texture distribution and enables photoreal micro geometry without heavy UV textures.

The following figure depicts the use case:

# Proposal

We propose to agree the use cases and document them in the draft TR.