Source:	Philips International B.V.

Title:	Philips Listening Lab Report - IVAS characterization Phase

Document for:	Approval

Agenda item:	7.5

# Summary

The document presents a report on subjective testing performed as a part of 3GPP-SA4 IVAS Codec Characterization Phase. Philips performed the experiments according to the procedures and test plan specified in permanent documents IVAS-7b (Processing Plan for Characterization Phase) and IVAS-8b (Test Plan for Characterization Phase). The tests included a single BS.1534 (MUSHRA) tests, and a series of room acoustics tests (ROOM-1 through ROOM-4). Once the tests were launched it was found that the real-time performance of the ROOM-4 test cases cannot be reached on the available equipment using the fixed-point IVAS version (IVAS FX). Therefore, it has been decided to use floating-point IVAS version (IVAS FL) across all room acoustics tests. No further deviations from or exceptions to the listening test procedures and specifications described in the test plan have been observed or experienced.

# Key Acronyms

CuT	Codec under Test

GAL	Global Analysis Laboratory

LL	Listening Laboratory

MC	Material collection entity

VR	Virtual Reality

# Introduction

Philips conducted 5 different experiments within the 3GPP SA4 IVAS Codec Characterization exercise. These experiments are listed in Table 1.

Table 1: List of experiments

| Experiment ID | Audio format | Bitrate | Comments |
| --- | --- | --- | --- |
| BS.1534-12 | ISM (3 or 4 objects) | 64-256 kbps |  |
| ROOM-1 | FOA | 80 kbps | ParamBin renderer |
| ROOM-2 | FOA | 96 kbps | FastConv renderer |
| ROOM-3 | MC 5.1 | 96 kbps | Time-domain object renderer |
| ROOM-4 | MS 5.1+2 | 128 kbps | Crend renderer |

# Test material

## BS.1534-12

The test material for BS.1534-12 (MUSHRA) test was selected and delivered by the MC.

## Room acoustics tests

For the room acoustics tests a combination of publicly available and in-house recorded anechoic samples were used. All the original samples were available in 48-kHz sampled with 16-bit resolution. To generate consistent SBA and multi-channel content, the original mono samples were mixed appropriately, using the following approach:

- First order Ambisonics

  - Single talker/instrument:

W=s2

X=s32

Y=Z=0

  - Two talkers/instruments:

W=s1+s22

X=s1cosα1+s2cosα232

Y=s1sinα1+s2sinα232

Z=0

Where α1=30° and α2=-30°

- Multi-channel content

  - Single talker/instrument

C=s

L=R=LFE=LS=RS=0

  - Two talkers/instruments:

L=s1

R= s2
C=LFE=LS=RS=0

The file naming structure of the input content samples was derived from what was defined in [2] and represented by:

krxxryy.wav

where:

- k is for the listening lab designator (k for Philips)

- r is room acoustics test

- xx is the sample designator, e.g. 01 (see Table 2)

- r is for experiment

- yy is the experiment identifier (ROOM-1 – 01, through ROOM-4 – 04)

Table 2: Signal identifiers

| Signal type | Sources | Identifier |
| --- | --- | --- |
| Speech | 1 | s01 |
|  | 2 | s02 |
| Music | 1 | s03 |
|  | 2 | s04 |

The room acoustics synthesis was based on the actual impulse responses recorded in the rooms. The rooms impulse responses as well as 360° photos thereof were provided by Qualcomm, see Table 3. These impulse responses were processed as specified in [1] using scripts provided in the IVAS source code repository to generate SOFA files, respective binary BRIR files, and room acoustics parameters. The impulse responses and reverb parameters were adjusted to normalize the output levels in different rendering modes. Normalization was performed on output with default head-orientation (i.e., no rotation).

Table 3: Room identifiers

| Room | Identifier |
| --- | --- |
| Auditorium | a01 |
| Kitchen corner | a02 |
| Lounge | a03 |
| Listening lab | a04 |

Snapshots of these room photos as rendered by the VR application are shown in Figure 1.

Kitchen corner

Auditorium

Listening lab

Lounge

Figure 1. Room photo snapshots as rendered by the VR application

# Listening Environment

## BS.1534-12

The tests were performed in a dedicated listening room at the Philips labs in Eindhoven. Listening setup included Focusrite Scarlett 2i2 audio interface and Sennheiser HD 650 headphones. Audio Research Labs’ STEP (Subjective Training and Evaluation Program) tool was used for the execution of the listening tests.

## Room acoustics tests

The tests were performed in a dedicated listening room at the Philips labs in Eindhoven. A VR setup was used, consisting of:

- State-of-the-art gaming-category PC, capable of running the Unity game engine, multiple IVAS decoder executables and a reference Python renderer in parallel,

- Meta Quest 3 VR headset for 3DoF tests (for a few tests HTC Vive Pro has been used instead),

- Focusrite Scarlett 2i2 audio interface and Beyerdynamic DT 990 headphones.

A dedicated VR room acoustics testing software based on Unity gaming engine, as introduced in [3] was used for these tests.

# Test instructions and order

The instructions used for the tests are provided in Annex A (BS.1534 tests) and Annex B (room acoustics tests). These instructions were based on [2] and adapted based on feedback gathered during the preliminary tests.

# Test Subjects

A total of 20 experienced listeners participated in the tests. All subjects had previous experience with MUSHRA tests. However, many listeners had little if any experience with the VR systems. Hence, some participants required more time than initially expected to get accustomed to the VR equipment.

# Issues observed and feedback from the listeners

## BS.1534-12

No essential issues were reported by the listeners.

## Room acoustics tests

While commencing the room acoustics tests it has been found that the real-time performance of the lab equipment did not meet the expectations regarding performance of the IVAS FX decoder with 5.1+2 format. This resulted in audible rendered output deterioration. Therefore, it was considered to perform all the room acoustics tests using the IVAS FL release. To validate the IVAS FX decoder against the FL decoder, MLD (maximum level difference) analysis was performed for all formats applicable to room acoustics tests using the selected test material. The results of this analysis are provided in Table 4. The use of IVAS FL for room acoustics testing was justified based on these MLD values obtained.

Table 4. IVAS FL vs. IVAS FX rendering MLD analysis

| Input Format | Bitrate | Output Mode | MLD max | MLD mean |
| --- | --- | --- | --- | --- |
| 5.1 | 96 | Reverb | 0.2 | 0.0889 |
| 5.1 | 96 | Binaural | 0.1 | 0.0667 |
| 5.1.2 | 128 | Reverb | 0.2 | 0.0722 |
| 5.1.2 | 128 | Binaural | 0.1 | 0.0444 |
| FOA | 80 | Reverb | 0.3 | 0.2611 |
| FOA | 80 | Binaural | 0.3 | 0.2556 |
| FOA | 96 | Reverb | 0.3 | 0.1222 |
| FOA | 96 | Binaural | 0.2 | 0.1000 |

# Raw data delivery

Raw data containing test results were delivered to GAL. For the BS.1534-12, the data delivery file as provided by GAL was used. For the room acoustics tests, the raw text files containing tests results were shared as agreed with GAL.

# References

- S4-251487 – IVAS Permanent Document IVAS-7b: Processing plan for characterization phase, v.1.0.0

- S4-251484 – IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.0.8.1

- S4-250874 – IVAS Characterization – Room Acoustics Test Plan

# Annex A: BS.1534-12 test instructions

The first step in the listening tests is to become familiar with the testing process. This phase is called a training phase, and it precedes the formal evaluation phase. The purpose of the training phase is to allow you to become familiar with some of the sound excerpts under test and their quality level ranges. The sound items which you will listen to will be more or less critical depending on the conditions used for coding these excerpts.

In the training phase containing four trials you will be able to select different codec conditions using buttons on the user interface. You can also use keyboard (A through H, R for reference). Listen carefully considering various aspects of audio, such as timbral quality, spatiality, etc. Use looping functions of the user interface as desired.

In the evaluation phase you need to rate the test samples. There are 12 trials in the evaluation phase. For each trial try first to identify the hidden reference, being the exact copy of the reference signal and rate it 100. This step is essential for the test! Thereafter, rate the remaining audio samples based on your subjective perception of audio quality as related to the reference. Note: it is okay to rate multiple test samples at 100.

Have fun!

# Annex B: Room acoustics tests instructions

In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio is an audio experience where you can perceive sound as if coming from the space around you, outside and around your head. Spatial audio can be enriched by synthetic reverberation and acoustic reflections that simulate the acoustics of a room, creating a more natural and immersive experience.

For this test you will be wearing a VR headset and headphones. The VR scene used in the test allows for three degrees of freedom (3DoF). That means that you can change your position only by rotation, and not by moving around. Please familiarize yourself with the equipment during the training session and adjust it appropriately. It is highly recommended to perform the tests while sitting on a swivel chair. It is advised not to take this test while standing.

Please familiarize yourself with the VR environment and the conditions under test (CuT) first using a training application. This application allows to select the conditions indicating different room acoustic synthesis modes and one without room acoustics for a reference. Please take time to observe the scene around you and listen to all the stimuli representing different conditions. Note that it will take a couple of seconds for the playback to start.

In the evaluation phase, please rank your preference regarding room acoustic synthesis quality and matching the visual environment shown. Please consider factors such as timbral quality, spatial depth, and spatial envelopment. Note that you need to score at least one preferred sample to 100.

Have fun!