3GPP TSG-SA WG4 Meeting #134	S4-251845

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 10 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AvCall-MED] Media Configuration for Avatars |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 26.264, media configuration requirements for AR-MTSI clients in terminals are defined in clause 7.3.1 but SIP register message for avatar calls are not specified and documented as for further study as per the following note. NOTE: The SIP register message for avatar calls is FFS. It may be needed for indicating which UEs require avatar assistance, and which ones are avatar capable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution provides details on SIP register messages for avatar calls and the behaviour of MTSI clients when registering with the level of support for avatar calls with “avatar-capable” or “avatar-assisted” parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete on registering avatar capabilities for MTSI clients. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 7.3	Avatar animation and rendering configuration

### 7.3.1	Avatar capability configuration

The AR-MTSI client in terminal shall indicate its support for avatar calls by including the “webrtc-datachannel” in the “+sip.sub-type” Contact header field.

A new Contact header field parameter, “+sip.3gpp-avatar-support” is used to indicate the level of support for Avatar calls. The possible values for the “3gpp-avatar-support” parameter are:

-	“avatar-capable”: indicates that the terminal is fully capable of receiving, animating and rendering of avatar.

- 	“avatar-assisted”: indicates that the UE has no support for animating or rendering an avatar

In the absence of the “+sip.3gpp-avatar-support”, it shall be assumed that the terminal has no support for Avatar calls. In this case, the MTSI client can only participate in an Avatar call if network animation and rendering are offered.

An MTSI terminal that supports receiving, animating and rendering of avatar and intends to participate in an Avatar call shall register with the “avatar-capable” value for the “+sip.3gpp-avatar-support” parameter and provides an SDP offer/answer that includes a data channel with the sub-protocol “3gpp.avatar-negotiation”.

The MTSI client in terminal supporting avatar communication shall complete avatar media animation and rendering negotiation with the DC AS before it initiates subsequent procedures to activate the network media rendering. An application data channel should be established for animation and rendering negotiation of the avatar with SDP offer/answer between sending MTSI client in terminal and the MF or receiving MTSI client with the sub-protocol “3gpp.avatar-negotiation” and continue to be used for animation and rendering re-negotiation until the end of the Avatar communication.

An AR-MTSI terminal that intends to participate in an Avatar call with the support of network rendering shall register with the “avatar-assisted” value for the “+sip.3gpp-avatar-support” parameter and provides an SDP offer/answer that includes a data channel with the sub-protocol “3gpp.avatar-negotiation”.

In the case of an MTSI client in terminal that does not support Avatar calls, the IMS AS may trigger network animation and rendering on behalf of the terminal upon receiving an (re)INVITE for an Avatar call. The output format for the rendered media shall be conformant to clauses 10.4.3 and 10.4.4 of TS 26.119 [3].

The IMS AS detects support for Avatar capabilities based on the presence or absence of the “+sip.3gpp-avatar-support” parameter of the Contact Header Field in the REGISTER message.

NOTE:	An MTSI client supporting Avatar calls can register with “+sip.3gpp-avatar-support” parameter in the Contact Header Field of the REGISTER message.

* * * End of Changes * * *