3GPP TSG-SA WG4 Meeting #134	S4-251911

# Dallas, Texas, USA, 17 – 21 November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 15 | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to BAR Interface Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Canada |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED, TEI |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The BAR APIs defined in clause B.1 have two conceptual scopes: - Avatar Management APIs: operations for avatar profiles, assets, metadata, etc. - Session Management APIs: operations for sessions and session-avatar bindings (e.g., “join session”, “associate avatar with session”, “update presence state”). It is therefore a cleaner design to separate them into two distinct logical services (or API groups). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate the APIs defined over the BAR interface into two logical services: Avatar Management service and Session Management service, while maintaining the set of existing APIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

Annex B:
Base Avatar Repository Interface

# B.1	Mbar_AvatarManagement service

## B.1.1	Overview

This clause defines the BAR avatar management APIs offered by the BAR and used by the DC AS or MF to manage avatar related data in the BAR. A summary of the resource structure is shown in Table B.1-1 below.

Table B.1-1: Resource structure of Mbar_AvatarManagement APIs

| HTTP request path element hierarchy | Description | Allowed HTTP methods |  |  |  |  | Resource |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Create | Retrieve | Update | Destroy | Non-RESTful operation | structure definition clause |
| avatars | Avatar collection | POST |  |  |  |  | B.1.2 |
| {avatarId} | Avatar resource |  | GET | PUT, PATCH | DELETE |  |  |
| assets | Asset collection | POST |  |  |  |  | B.1.3 |
| {assetId} | Asset resource |  | GET | PUT | DELETE |  |  |
| associatedInfo | Associated Information resource |  | GET |  |  |  | B.1.4 |  |
| representations | Avatar Representation collection | POST |  |  |  |  | B.1.5 |
| {avatarRepresentationId} | Avatar Representation resource |  | GET | PUT, PATCH | DELETE |  |  |

* * * Second Change * * *

# B.2 	Mbar_SessionManagement service

## B.2.1	HTTP resource URIs and paths

Clause B.1.2 applies.

## B.2.2	Usage of HTTP

Clause B.1.3 applies.

## B.2.3	Common API data types

Clause B.1.4 applies.

## B.2.4	Sessions API

### B.2.4.1	Overview

This clause defines a BAR session management API used by the DC AS or MF to provide session-scoped configuration and authorization for avatar representation usage during an avatar call. A session resource is created to bind the base avatar model to be used, a subset of the assets and LoDs selected for that session. An existing avatar representation resource is bound to a session resource to define the selected base avatar model and assets. When the identifiers for ownerId and the session creatorId match, a token is issued by the BAR.

NOTE: Creating a session resource intended only for a specific user is for FFS.

| HTTP request path element hierarchy | Description | Allowed HTTP methods |  |  |  |  | Resource |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Create | Retrieve | Update | Destroy | Non-RESTful operation | structure definition clause |
| sessions | Session collection | POST |  |  |  |  | B.2.1.2 |
| {sessionId} | Session resource |  | GET | PUT | DELETE |  |  |

Table B.2.4.1-1: Operations supported by the Sessions API

### B.2.4.2	Resource structure

The Sessions API is accessible through the following URL base path:

{apiRoot}/3gpp-mbar-sessions/{apiVersion}/sessions/

Table B.2-2 specifies the operations and the corrresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table B.2.4.2-1: Operations supported by the Sessions API

| Operation name | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Session |  | POST | Creates an Session resource in the BAR. |
| Get Session | {sessionId} | GET | Used to retrieve a previously created Session resource in the BAR. |
| Update Session |  | PUT | Used to update Session resource corresponding to a Session ID. |
| Delete Session |  | DELETE | Removes and deletes the Session and revokes its associated access token. |
| Get Session Token | {sessionId}/token | GET | Returns current token metadata for the Session. |
| Rotate Session Token |  | POST | Issues a new short lived access token for the Session and invalidates the previous token. |
| Revoke Session Token |  | DELETE | Revokes the current Session token without deleting the Session. |

### B.2.4.3	Data model

#### B.2.4.3.1		Session resource

Table B.2.4.3.1-1: Definition of Session resource

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| sessionId | ResourceId | 1..1 | C: RO R: RW U: – | Unique identifier assigned by the BAR upon session creation. |
| avatarId | ResourceId | 1..1 | C: RO R: RW U: – | A unique identifier assigned to a base avatar model by the BAR. |
| avatarRepresentationId | ResourceId | 1..1 | C: RO R: RW U: – | A unique identifier assigned to an Avatar Representation by the BAR on creation. |
| ownerId | string | 1..1 | C: RW R: RO U: – | A unique identifier identifying the subscriber (owner) associated with the avatar representation specified by avatarId in this resource. |
| creatorId | string | 1..1 | C: RW R: RO U: – | A unique identifier identifying the creator of the session resource. |
| callStartTime | number | 1..1 | C: RW R: RO U: RW | The wall clock time at the start time of an avatar call period. |
| callEndTime | number | 1..1 | C: RW R: RO U: RW | Specifies the end time (in wall clock time) of an avatar call period. callEndTime may be extended while both the owner and intended user of the Base Avatar are online. NOTE: DC AS may collect ping within time out to determine connectivity. |
| publishTime | number | 1..1 | C: RO R: RO U: – | Describes the issue time or latest update time (in wall clock time) of the session resource. |
| accessToken | SessionAccess | 1..1 | C: RO R: RW U: – | Access token and validity for dereferencing BAR resources within the scope of this session. |

#### B.2.4.3.2		SessionAccess type

Table B.2.4.3.2-1: Definition of SessionAccess type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| token | SessionToken | 1..1 | The bearer token that authorizes session-scoped access to BAR resources referenced by this session. |
| issuedAt | number | 1..1 | Token issuance time (ms since epoch). |
| expiresAt | number | 1..1 | Absolute expiry (ms since epoch). Tokens are short-lived and shall not exceed operator policy TTL. |

#### B.2.4.3.3		SessionToken type

Table B.2.4..3.3-1: Definition of SessionToken type

| Property name |  |  | Data type | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| scheme |  |  | string | 1..1 |  | Token scheme; "bearer" shall be supported. |  |
| value |  |  | string | 1..1 |  | Opaque token value. Its internal structure is operator-specific. |  |

* * * End of Changes * * *