# Introduction

The floating-point code of the IVAS codec was standardized in Rel-18 in TS 26.258 [1]. The task to convert the floating-point implementation to fixed-point (BASOP) code is being done by Ittiam, under a contract issued by ETSI. The organization of this conversion is outlined in [2], which is also attached to the contract. A plan for the verification of the delivered code was agreed in [3]. This contribution aims to provide the input for SA4 to agree that the delivery of the first part, the IVAS BASOP Decoder and Renderer, has been fulfilled according to the contract. The Decoder and Renderer was delivered on August 31st, 2024, according to the time plan [3]. The verification started with a pre-test phase, where new issues were discovered. A version for subjective verification testing was tagged on October 23rd, 2024. This version still has issues such as crashes and quality problems, and work has continued to address these. For this reason, the verification is done on different versions of the code. Unless stated otherwise, the versions used for the verification are these:

- Floating point reference:

  - Branch: ivas-float-update

  - Commit: d18cbee3417bc331630e6dcf214908deacdc8167

  - Date: Wednesday Oct 23 11:00:11 2024 +0000

- BASOP verification candidate

  - Tag: 20241023_Ittiam_Decoder_Release_subjective_Testing

  - Branch: main

  - Commit: 074e9116eb4a6380f0fbc82b20bbfd7473c565ed

  - Date: Wednesday Oct 23 12:23:23 2024 +0000

# Objective measurement tools

The objective measurement tools are used for mono audio comparison. For output formats with more than one channel (i.e. all output configurations except MONO), the tools perform a pairwise comparison of each channel from the test and reference audio samples and present the worst scoring items. The objective measures are:

- Maximum loudness difference (MLD)

- Segmental Signal-to-Noise Ratio (SSNR)

For MLD the maximum number among the channels is reported and for SSNR the minimum value is shown.

## MLD

The Maximum Loudness Difference (MLD) tool was developed for the EVS floating point conformance, TS 26.843. The tool reports the maximum spectral deviation on a perceptually weighted spectrum. The first version used is available from [9]. It was later updated to handle JBM test cases, where the MLD is computed on the time-aligned portions of the test and reference signals. The time alignment is checked based on the JBM trace files that are input to the tool. For a multi-channel signal, the highest MLD score is reported.

## SSNR

The Segmental Signal-to-Noise Ratio is evaluated as defined in as defined in ISO/IEC 14496-4. The segment length is 20 ms, and segments below the threshold of -50 dBov is excluded from the calculation. If all segments are below the threshold, the result will be NaN (Not a Number).

## Test vector set

Three main sets of test vectors were used in the verification:

- Short test vector set (STV):

  - Input audio and accompanying metadata spanning all formats.

  - 8 kHz (EVS only), 16 kHz, 32 kHz and 48 kHz sampling rate

  - Speech and mixed speech/music content

  - Length: 1-30 seconds, average 10.5 seconds

  - 0/-10/+10 dB level scaling applied in scripts

- Long test vector set (LTV)

  - Input audio and accompanying metadata spanning all formats.

  - 16 kHz, 32 kHz and 48 kHz sampling rate

  - Speech and mixed speech/music content

  - Length: 115-206 seconds, average 152 seconds

  - 0/-10/+10 dB level scaling applied in scripts

- IVAS Selection test material

  - Input audio and accompanying metadata spanning all formats except OMASA and OSBA.

  - 48 kHz sampling rate

  - Speech and mixed speech/music content

  - Length: approximately 6-10 seconds per item, several items per category.

  - Nominal input level (-26 LKFS), although level scaling may be added in evaluation.

# Comparison of EVS BASOP and EVS float

## Complexity

Using the ITU-T WMC tool v.1.4 [9], the floating-point implementation of the 3GPP EVS codec (TS26.443, v.16.3.0), referred to as EVS FLOAT, has been instrumented for memory and complexity measurements. Similarly, the fixed-point implementation of the 3GPP EVS codec (TS26.452, v.16.4.0), referred to as EVS BASOP, has been modified to allow memory instrumentation with the WMC tool. The complexity counting mechanism in EVS BASOP, implemented in the `count.h` and `count.c` files, has not been altered, so the complexity measurements for EVS BASOP remain consistent with those reported in the Performance Characterization report TR 26.952 [11].

Complexity was measured using a 48kHz test signal comprising multiple short segments of clean speech, noisy speech, background noise, and generic audio, with a total duration of 38 seconds. Measurements covered 126 operating points across the EVS codec’s entire bitrate range, from 5.9 kbps (VBR) up to 128 kbps, and included all four supported bandwidths: NB, WB, SWB, and FB. Testing also included DTX operation, bitrate switching, and the 13.2 kbps channel-aware mode, but did not include error concealment or JBM among the tested operating points.

Complexity measurements were categorized into encoder complexity, decoder complexity, and total codec complexity (ENC + DEC per operating point). The 126 pairs of total codec complexity values for both EVS BASOP and FLOAT were then used to derive the complexity ratio as follows:

Rk=CMPLXBASOPCMPLXFLOAT,         k=1,…,126

The ratios Rk were subsequently used to generate the histogram of complexity ratios, as illustrated in Figure 1 below:

Figure 1: Histogram of EVS BASOP to FLOAT complexity ratios

The average complexity ratio is 0.76, with a standard deviation of 0.07.

Memory consumption was measured using the same test set as for computational complexity. It is categorized into RAM and ROM usage. RAM consumption is further broken down into HEAP (dynamically allocated memory) and STACK (automatic variable memory). RAM usage at each codec operating point is evaluated by measuring both the HEAP memory allocated dynamically using malloc() and the maximum STACK memory required.

ROM memory consumption is divided into PROM (program ROM) and TROM (global static variables and constants). PROM and TROM usage remain constant across all operating points and are calculated by summing the PROM or TROM consumption of the decoder and shared (common) modules.

Table 1 presents the memory consumption for the EVS decoder, with average STACK and HEAP memory values calculated across all operating points.

Table 1: EVS BASOP vs FLOAT decoder memory consumption.

| Memory [kBytes] |  | EVS BASOP | EVS FLOAT | EVS BASOP-TO-FLOAT RATIO |
| --- | --- | --- | --- | --- |
| RAM | HEAP | 125993 | 218840 | 0.58 |
|  | STACK | 68485 | 93598 | 0.73 |
| ROM | PROM | 200100 | 284456 | 0.70 |
|  | TROM | 266108 | 424000 | 0.63 |

The last column of Table 1 indicates the target ratio that should be reached with the IVAS codec.

## Objective quality measures

The requirements for the IVAS BASOP in comparison to the IVAS FLOAT are set in relation to how EVS BASOP compares to EVS FLOAT [11]. The objective measures were evaluated, comparing the EVS FLOAT to the EVS BASOP using the test vector set defined by 26.444, in the scripts Readme_EVS_dec.txt and Readme_JBM_dec.txt. These tests can be split into two categories:

- Static operation – Fixed bit rate, error free channel

- Flexible operation – DTX, PLC, Bitrate switching, JBM

The Static operation comprised of 141 test cases, with a maximum MLD of 4.01. The Flexible operation gave a wider range of MLD values, where the maximum MLD value in this category was 44.2. In general, the high MLD test cases were JBM tests. A summary of the MLD values can be found in Figure 2.

Figure 2: Histogram of MLD values for the EVS BASOP vs EVS FLOAT.

Figure 3: Histogram of MIN_SSNR values for the EVS BASOP vs EVS FLOAT.

Based on these findings, a rule-of-thumb target of MLD<5 was used for the IVAS BASOP conversion. This threshold cannot be applied strictly, since EVS BASOP vs EVS FLOAT does not meet this requirement, in particular when considering the test cases in Flexible operation. The performance of EVS BASOP vs EVS FLOAT in relation to this threshold is shown in Table 2.

Table 2: EVS BASOP vs EVS FLOAT test cases below threshold MLD 5 and above SSNR 60.

| Operation | MLD < 5 | Max MLD | MIN_SSNR > 60 | Min MIN_SSNR |
| --- | --- | --- | --- | --- |
| Static | 100% | 4.01 | 0% | 24.54 |
| Flexible | 91% | 44.2 | 0% | -1.66 |
| All | 92% | 44.2 | 0% | -1.66 |

# Summary of requirements for IVAS BASOP

## Objective quality measures

The objective measures of the IVAS Decoder and Renderer are measured daily by an automated test system. A snapshot of this measurement from commit 8f97e97c0121cc13d69a9ac096f88d148f89e39c (Fri Oct 25 11:09:47 2024 +0000) is presented below. A summary of the MLD scores for the IVAS Decoder can be seen in Figure 4, and a summary of the MLD scores for the Renderer can be seen in Figure 5. The scores are derived on the LTV test set for the decoder and renderer, using the nominal, +10 dB and -10 dB level scaling. The number of tests in the static operation category is 1209, the flexible category has 519 tests and the renderer category comprises 1974 tests.

Figure 4: Histogram of MLD values for the IVAS Decoder.

Figure 5: Histogram of MLD values for the IVAS Renderer.

Figure 6: Histogram of minimum SSNR values for the IVAS Decoder.

Figure 7: Histogram of minimum SSNR values for the IVAS Renderer.

Applying the target objective measures from [2] of MLD < 5 and SSNR > 60 on the IVAS BASOP Decoder and Renderer, we obtain the result in Table 3.

Table 3: IVAS BASOP Decoder test cases summary.

| Operation | MLD < 5 | Max MLD | MIN_SSNR > 60 | Min MIN_SSNR |
| --- | --- | --- | --- | --- |
| Static | 76% | 61.58 | 1% | 21.42 |
| Flexible | 58% | 73.55 | 0% | -9.43 |
| Renderer | 99% | 9.48 | 71% | 10.71 |
| All | 86% | 73.55 | 38% | -9.43 |

## Summary of formal requirements

| Requirement | Observation |
| --- | --- |
| 3GPP Forge will be used as the platform of FX development, similarly to the development of FL code in Public Collaboration. | 3GPP Forge has been used as platform for the development. |
| Inclusion of 3rd party code or open-source code is not allowed; inclusion of code from generative AI tools is not allowed. | None observed. |
| Structure of FL code shall be preserved to the maximum extent to allow comparison between FL and FX code and easier maintainability of the two code bases. | Structure is very similar. |
| This includes the best preservation of function prototypes and comments in the code; | Seems to be the case. |
| file names shall remain unchanged. | The use of the _fx suffix is inconsistent. However, this is also the case for the files inherited from the EVS BASOP in TS 26.452. |
| In addition, all variables converted to FX shall be marked with precision information (integer, fractional) in Q-notation; | This was found to be insufficient in the first delivery. An effort was made by Ittiam to add missing precision information. The review of this update is incomplete. |
| precision changes should be documented; | This was missing in the first delivery and addressed in an update by Ittiam (see note above). The review of this update is incomplete. |
| the goal is that the original functions could be used interchangeably during development; function names of converted functions shall avoid symbol clashes during the development; | This has not been reviewed. |
| the objective is to ultimately have maximum similarity to the FL code and eventually all variables and function names also in FX shall be unified to their FL counterpart. | Requirement somewhat conflicts with the previous requirement, it would be good though if functions in fixed have the _fx suffix so that there are no symbol clashes due to identical symbol name but different prototype (and even implementation). This may be for SA4 to discuss. |
| The performance of the FX code should match the performance of the FL code, both in terms of subjective quality and complexity. | See clause 5 and 6. |
| For decoding, the output synthesis of FX decoder (FL enc+ FX dec) compared to FL decoder (FL enc + FL dec) should be within the limit of segmental SNR (per frame and then averaged) > [60 dB], and MLD < 5. | See clause 4.1. |
| For bit-streams generated with the FX encoder, the output synthesis of the bit-stream decoded with the FL decoder compared to the synthesis of a bit-stream generated with the FL encoder and decoded with the FL decoder should be within the limit of MLD < [TBD], PEAQ (basic profile) < [0.1]. | Not applicable, this is a decoder-only release. |
| For non-EVS operation, the performance shall be measured using both the short and long test vector sets as specified in 3GPP Forge, with nominal level and +-10 dB. | This has been done. |
| Interoperability between FX and FL codes shall be guaranteed; for example, valid FL bitstreams shall be decodable by the FX- | Automated tests run the FL enc->FX dec and FX enc->FL dec. Seems to work in general. |

# Observations on the source code

Overall, the fixed-point code includes an excessive number of parameter precision conversions (Q-notation values) to match the varying precision requirements of different functions. Minimizing these intermediary conversions would enhance code readability and, more importantly, reduce the complexity overhead in the BASOP implementation. The relevant Q-notation values should also be documented consistently throughout the code (refer to the summary of formal requirements). Some inconsistencies between the actual and documented Q-notation values have been found, too.

The requirements state that floating-point and fixed-point functions can be used interchangeably. The fixed-point code still contains many original floating-point functions inside sections disabled with IVAS_FLOAT_FIXED macro. However, the fixed and floating-point functions cannot be interchanged directly in the fixed-point code, for instance by simply replacing #ifdef with #ifndef directive next to a function to be interchanged. The IVAS_FLOAT_FIXED macro was introduced during fixed-point code development and is intended to be removed along with inactive floating-point code towards completion of the code. It is not intended to ensure function interchangeability within the fixed-point code. In order to interchange functions between floating and fixed-point code one should copy a function from one codebase to another and provide with a dedicated adapter including parameter conversion based on Q-notation as documented. For instance, if in a BASOP code-base function ivas_func_fx(…) is to be replaced with its floating-point implementation, one should copy function ivas_func(…) from the floating-point codebase and replace the ivas_func_fx(…) implementation so that it would first covert input data to the fixed-point format, invoke ivas_func_fx(…), and convert output data from the fixed-point format to floating-point appropriately. It is proposed that this interchangeability mechanism should be documented. It should also be noted that multiple fixed-point functions names do not use the ‘_fx’ suffix, which might hamper interchangeability. These issues were filed in: https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1035.

Inconsistencies in the complexity analysis instrumentation have been observed, and the thorough analysis of the instrumentation seems required.

Some inconsistencies in the variable types are observed, for instance where uint16_t type was used instead of UWord16. This has been reported in https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1036. Also, some code still using floating-point operations was found, see https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1037.

In addition, there are multiple instances where the left-shift and right-shift operators, which are available in versions that either assert or do not assert on overflow, are not being used appropriately. The non-asserting versions are meant to be used only in situations where saturations are expected (and can be tolerated). Our inspection found many instances where saturation should not be allowed. Fixes to the code are ongoing and being tracked in issue #926 (https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/926).

# Quality verification of delivered Decoder and Renderer

## Stereo

### Stereo at 13.2 kbps and 16.4 kbps

#### Objective evaluation

VoiceAge evaluated the maximum MLD and the minimum segmental SNR for the following operating points of the fixed-point decoder.

| Format | Bit rates | DTX | FER | Level |
| --- | --- | --- | --- | --- |
| Stereo | 13200, 16400 bps | On and Off | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |

The databases used included the selection test database and some internal databases. The fixed-point decoder commit used was 6f1fdf53.

| Stereo : 13200 and 16400 bps, with DTX on and off, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | Max. MLD | Min. Seg.SNR |
| Nominal |  |  |
| +10 dB |  |  |
| -10 dB |  |  |

#### Subjective evaluation

VoiceAge run two stereo subjective tests using the material from the selection. All tests were done with ref-A-B comparison style and the listeners were a mix of experts and students from university. The stereo operating points tested were:

- 13.2 kbps

  - Based on p800-2

  - Mix of street and car noise

  - 5% frame of frame lost

  - DTX  on

- 16.4 k

  - Based on p800-1

  - Clean speech

  - No frame lost

  - DTX off

##### Stereo 13.2 kbps

- 13 Listeners

- 16 samples

| Number of votes | 208 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Average global | 0.087 |  | SEMD | t | Prob (TDIST) |  |
| STD global | 0.724 |  | 0.0502 | 1.7244 | 0.0431 |  |
| P(T<=t) one-tail | 0.0431 | Fixed point decoder slightly worse than floating point |  |  |  |  |

##### Stereo 16.4 kbps

- 13 Listeners

- 16 samples

| Number of votes | 208 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Average global | -0.067 |  | SEMD | t | Prob (TDIST) |  |
| STD global | 0.656 |  | 0.0455 | 1.4799 | 0.0702 |  |
| P(T<=t) one-tail | 0.0702 | Fixed point decoder not worse than floating point |  |  |  |  |

#### List of issues related to stereo and their status (as of 2024-11-06)

Since the first release of the decoder, VoiceAge has filed 91 issues from which 80 has been resolved. Here is a list of the remaining issues filed by VoiceAge and their estimated priority:

| Title | Issue ID | State | Priority |
| --- | --- | --- | --- |
| Bitrate switching quality issue | 895 | Open | Priority::Critical |
| Decoder crashes in Hilbert_transform during bit-rate switching for low-level content | 914 | Open | Priority::Critical |
| Presence of a tone during silence part | 881* | Open | Priority::Major |
| amplitude of downmixed signal (MC to SCE or MC to CPE) is too high for the EVS fixed point core | 823 | Open | Priority::Major |
| Discontinuities when switching from 16.4 kbps DFT-stereo to 80 kbps MDCT stereo | 883 | Open | Priority::Major |
| Noise burst during silence in ISM 2 at 48 kbps | 887 | Open | Priority::Major |
| Saturation when moving data from 32b to 16b | 893 | Open | Priority::Major |
| High frequency tone @ 48kbps stereo during frame concealment | 905 | Open | Priority::Major |
| BWE discontinuities when switching from ACELP to TCX in DFT stereo at 16.4 kbps | 909 | Open | Priority::Major |
| Crash in low level signal at 24.4 kbps with dtx enabled | 824 | Open | Priority::Minor |
| In stereo at 32kbps first frame after switching from DFT to TD is missing some HF energy | 908 | Open | Priority::Minor |

*Note that 881 could have also an important effect for ISM

### Stereo at 24.4 kbps and 32 kbps

#### Objective evaluation

To cover a wider range of operating points, a short input was used in combination with objective measures. The test file comprised of approximately 3 minutes of mixed speech and music items. The tests for objective verification and informal listening were:

- Test 1: Sampling rates and output configurations

- Test 2: JBM and FER conditions

- Selection tests (P.800-1, P.800-2, P.800-3, BS.1534-1a, BS.1534-1b)

Stereo test 1 – Sampling rates and output configurations

The parameters of stereo test 1 are listed in Table 4.

Table 4: Parameters of test 1: Sampling rates and output configurations. 126 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 3 | 16, 32, 48 |
| Output sampling rate | 3 | 16, 32, 48 |
| Output formats | 7 | MONO, STEREO, 5_1, 7_1, 5_1_2, 5_1_4, 7_1_4 |
| Bit rates | 2 | 24400, 32000 |
| Channel conditions | 1 | Clean channel |
| DTX | 1 | Off |

The results of the experiments in terms of objective scores can be found in Figure 8. The rendering of stereo to multichannel is a mapping of the left and right channels to the front left and front right, and the remaining channels are zero. Since the objective scores are only reported for the worst scoring channel, the scores are the same for the rendered loudspeaker configurations. From informal listening and inspection of the output files the following notes were made:

- Energy loss below 1 kHz occurring during one speech segment for MONO output. Not a drastic effect but can be heard as a loss of low frequencies. Recorded as https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/985

- Difference in high-frequency energy in 16 kHz – 20 kHz band. Not really audible, but visible in the spectrogram. Recorded as https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/986.

- Energy loss in onset of a few drumbeats. Not audible but visible in waveform. Recorded as https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/987.

Figure 8: Objective scores for stereo test 1.

Stereo test 2 – Jitter Buffer Manager and Packet Loss Concealment

The parameters of stereo test 2 are listed in Table 5.

Table 5: Parameters of test 2: Delay and loss profiles. 60 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 1 | STEREO |
| Bit rates | 2 | 24400, 32000 |
| Channel conditions | 15 | FER 0%, FER 3%, FER 10%, JBM dly_error_profile_{0..11}.dat |
| DTX | 2 | On/Off |

The objective scores for stereo test 2 can be found in Figure 9. The JBM test cases are difficult to measure since the objective measures are sensitive to differences in the timing of the output signals. Due to differences in the arithmetic precision, the timing is expected to differ to some extent. The MLD measurement tool only compares the segments that are time aligned, while the PEAQ ODG and Min SSNR will suffer from the misaligned output. Notes from informal listening:

- Difference in timing in output. Overall, no immediate problems spotted.

- BASOP has higher energy in 16-20 kHz band. Looks like the same issue as in https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/986.

- dly_error_profile_11.dat is an extreme profile which causes burst losses of ~10 seconds and ~180 seconds. The spectrograms of the silence regions look different in float and BASOP, but it is not audible.

Figure 9: Objective scores for stereo test 2.

Selection tests (P.800-1, P.800-2, P.800-3, BS.1534-1a, BS.1534-1b)

The stereo experiments of the IVAS selection test were processed using the float reference and the BASOP verification version, and the MLD scores were calculated for the decoded items. The items with the highest MLD scores were identified and reviewed with informal listening and inspection. The objective scores are shown show Figure 10. Notes from the informal listening of the highest MLD items:

- High frequency tone after frame concealment. An issue has been filed and a fix that fixes the issue has been proposed. High frequency tone @ 48kbps stereo during frame concealment (#905) · Issues · SA4 / Audio / IVAS BASOP · GitLab

Figure 10: Stereo MLD score

#### Subjective evaluation

The subjective quality was evaluated using a Ref-AB test, running stereo at 32 kbps.

- Method: Ref-AB

- Operating point: Stereo @ 32 kbps, 48 kHz input and output, error free channel

- Material: 40 Mixed critical items (speech, music, mixed speech/music)

- Listeners: 7

The material was selected by processing a set of 344 mixed speech and music items, and selecting the ones which had the worst objective scores. The objective measures used were MLD, SSNR and PEAQ. Two objective measurements were done to select the material. Firstly, the objective measures were evaluated using the output of the floating-point reference decoder as reference. Secondly, the objective measures were run comparing both the floating-point reference decoder and the BASOP decoder with the input signal and deriving a difference between the scores. This gave 6 different rankings to select the most critical samples. Selecting the 7 worst ranking items from each list gave a total of 40 items for the test. The result can be found in Figure 11, where positive scores indicate preference for the BASOP version. The result is that the BASOP is statistically worse, with a very small margin.

Figure 11: Result of Ref-AB test for stereo @ 32 kbps.

#### Issues identified

Table 6 List of issues identified following the tests for Stereo at bitrates 24.4 and 32 kbps.

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 732 | Missing energy on stereo CNG frames at 24k4 kbps | e6911975 | Stereo CNG | Major | Ongoing |
| 985 | Stereo to MONO decoding: energy drop below 1 kHz | 074e9116 | Stereo to mono | Medium | Pending |
| 986 | Stereo decoding, 16-20 kHz difference | 074e9116 | Bandwidth extension | Minor | Pending |
| 987 | Stereo decoding 16 kHz input, energy loss in drum onset | 074e9116 | Core coder, transients | Minor | Pending |
| 905 | High frequency tone @ 48kbps stereo during frame concealment | d4fc334d | IGF | Major | Resolved |

### Stereo from 48 kbps up to 256 kbps

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 387d70f2 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments P800-1, P800-2, P800-3, BS.1534-1a, BS.1534-1b.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 2 | STEREO, MONO |
| Bit rates | 8 | 48000, 64000, 80000, 96000, 128000, 160000, 192000, 256000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| DTX | 2 | On/Off |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots are presented separately for:

- clean channel and DTX off

- FER and DTX on

- JBM

##### Objective evaluation stereo output

For clean channel without DTX the MLD requirement is fully met and is for all test cases lower than 1. SSNR is higher than 44 dB for all test cases, with the majority of the test cases to fall within. 44-60 dB which does not meet the 60dB requirement but seems to be adequate performance comparing to Figure 3.

For FER and DTX the majority of test cases are well below 5 with a number of outliers with higher MLDs reaching 24. For SSNR, the range is similar to that of a clean channel, but there is a noticeable shift towards degradation. Comparing these two metrics with clean channel test cases, it can be concluded that FER and DTX remain occasional issues, which could potentially affect other operational points that these stereo bitrates serve as core coders. The main issue with the outliers is documented in issue 1029, see section 6.1.3.4.

For JBM the picture is similar to the FER cases. The SSNR measurements are not as reliable because of the differences in the time scaling of the signals being compared.

Tests with clean channel / DTX off

Tests with FER and/or DTX on

Tests with JBM

##### Objective evaluation mono output

For mono the overall picture is similar to the stereo output cases, however for FER/JBM the outliers are of higher MLD. Maybe this is in relation with issue reported in issue #975 of list in 6.1.3.4.

Tests with clean channel / DTX off

Tests with FER and/or DTX on

Tests with JBM

#### Subjective evaluation

For the subjective evaluation a Ref A-B test was conducted. From the objective evaluation it can be concluded that in terms of MLD and SSNR there is a degradation for conditions with FER and DTX. Therefore, these conditions were selected to be tested for the subjective evaluation.

Listening test conditions

- Stereo 48kbps at 48 kHz FB

- FER 5%

- DTX on

- Version tested SHA 6f1fdf539

- Items were selected critical items of  Fraunhofer internal database (speech and mixed music). Worst case items in terms of MLD were selected for the listening test.

Listening test results

The results from 6 expert listeners are shown below. It can be concluded that the perceptual quality is on par with a  tendency towards degradation. No systematic or worrying degradation was observed from the listeners, however, there was a clear degradation as can be seen for Item 9. Also, it was noticed that a regression was introduced in the BASOP decoder after the listening test was conducted. At the code version used for objective evaluation, the same items used with the same frame error pattern as in the listening test showed clearly audible clicks at concealed frame boundaries. The problem was documented in issue 1029, see section 6.1.3.4.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1041 | Decoder crash for Stereo at 32kbps JBM operation in stereo_dft_res_ecu_burst_att_fx() | Crash | JBM | Blocker | Open |
| 1030 | Strong artifact with Stereo JBM decoding to MONO output causes high MLD | Audio Quality | PLC | Critcial | Open |
| 1029 | New Clicks in MDCT-Stereo concealment | Audio Quality | PLC | Critical | Open |
| 981 | Stereo decoding with JBM: energy difference in right channel despite similar TSM | Audio Quality | JBM | Critical | Resolved |
| 975 | Higher MLD in MDCT-Stereo mono output than in stereo out | Audio Quality | Decoding | Major | Open |
| 956 | MDCT-Stereo concealment - discontinuity introduced by PLC | Audio Quality | PLC | Major | Resolved |
| 955 | MDCT-Stereo PLC - channel is attenuated in FD concealment | Audio Quality | PLC | Major | Resolved |
| 924 | Decoder crash for Stereo at 96kbps FER in IGF_appl_ivas() | Crash | PLC | Blocker | Resolved |

## Multichannel 5.1

### Bitrates 13.2 to 32 kbps (McMASA)

#### Objective evaluation

The 5.1 input operation on bitrate 13.2 to 32 kbps was evaluated objectively using material from the selection experiments. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core and with the fix not yet implemented, a constant gain of -8 dB was applied to the input signals before processing them with the reference encoder. This allows for more meaningful objective measures.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 5.1, Binaural, Mono, Stereo |
| Bitrates | 13.2 – 32 |
| Bitrate switching | Within 13.2 – 32 |
| FER | Clean and 10% (ep_10pct.g192) |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 16, 32, 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate, FER, DTX, bandwidth and output fs. A total of 1536 cases are presented below in the histogram figures.

In general, the MLD values are low for all tested cases. However, the minimum SSNR values are also somewhat low.

#### Subjective evaluation

No formal subjective evaluation was performed for 5.1 input for bitrates 13.2 to 32 kbps. Informal evaluation was performed for cases that had possible issues based on the objective measures.

#### Issues identified

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1013 | Crash in ivas_hq_core_dec_fx() for MC 5.1 at 32kbps JBM 16kHz out | Crash | Decoder, Multichannel | Blocker | Open |
| 954 | High frequency differences above 6 kHz, 7.1+4 MC with FER at 32kbps | Audio quality | Decoder, Multichannel | Major | Ongoing |

### Bitrates 48 to 80 kbps (ParamMC)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 38597c33 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-2a and BS1534-2b.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 5.1, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 3 | 48000, 64000, 80000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

The overall performance for 5.1 output for these bitrates is very good and fully met the MLD requirements. However, there were some outlier observed in SSNR in the range of 28-30dB, this has been reported in issue #1031 of the list in 6.2.2.4.

Although, MLD scores are lower than the threshold of 5 for FER, JBM, the picture is different comparing to the clean channel, implying that there may be differences that however, may not be worrisome (e.g. different PLC mode decision).

Also for binaural output the overall performance is very good and in line with the requirements. However, differences to the MLD scores for some outliers compared to the pass-through mode, may imply some issue with the binaural rendering for these bitrates. Especially for the outlier for FER cases.

For binaural room IR output the overall MLD scores for are lower compared to the pass-through or binaural HRIR output.

##### Objective evaluation 5.1 output

Tests with clean channel

Tests with FER

Tests with JBM

##### Objective evaluation BINAURAL output

Tests with clean channel

Tests with FER

Tests with JBM

##### Objective evaluation BINAURAL_ROOM_IR output

Tests with clean channel

Tests with FER

Tests with JBM

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1039 | Noise burst and clicks in MC5.1 decoding at 64kbps with FER | Audio Quality | PLC | Major | Open |
| 1031 | Differences for MC5.1 at 80kbps with FER | Audio Quality | PLC | Major | Open |
| 950 | High MLD for ParamMC 5.1 at 48/64/80kbps or 7.1+4 at 128kbps | Audio Quality | Decoding | Major | Open |
| 921 | Decoder crash for MC51 at 48kbps FER in decoder_tcx_noisefilling_fx() | Crash | PLC | Blocker | Resolved |

### Bitrates 96 kbps to 512 kbps (MCT)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 38597c33 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-2a and BS1534-2b.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 5.1, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 6 | 96000, 128000, 160000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

The overall performance for 5.1 output for these bitrates is very good and fully met the MLD requirements. There are some outliers for FER, JBM that may very well be related to the issues reported in issue #961 listed in 6.2.3.4

Also for binaural output the overall performance is very good and in line with the requirements. For binaural room IR output the overall MLD scores for are lower compared to the pass-through or binaural HRIR output.

##### Objective evaluation 5.1 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1034 | Click in MC5.1 BINAURAL output at 128kbps with concealment | Audio Quality | PLC | Medium | Open |
| 1002 | Decoder crash for MC51 at 128 kbps with FER in RefineThresholdsUsingPitch() | Crash | PLC | Blocker | Resolved |
| 961 | High energy differences for MC 5.1 for 3 first frames | Audio Quality | Decoding | Major | Resolved |
| 949 | High MLD for MC 5.1 with FER at 256 kbps | Audio Quality | Decoding | Major | Open |
| 948 | High MLD value due to high energy in transients for MC 5.1 in MCT | Audio Quality | Decoding | Major | Resolved |
| 927 | Audible artifacts for LFE with PLC for 5.1 MCT at 96kbps | Audio Quality | Decoding | Major | Resolved |
| 923 | Decoder crash for MC51 at 512kbps FER in LFE PLC | Crash | PLC | Blocker | Resolved |
| 922 | Decoder crash for MC51 at 128/256/384/512kbps FER in LFE PLC | Crash | PLC | Blocker | Resolved |

## Multichannel 7.1

### Bitrates 13.2 to 32 kbps (McMASA)

7.1 input was not formally evaluated for bitrates 13.2 to 32 kbps. Nevertheless, the automated verification system of BASOP development contains these input test cases while 5.1 and 7.1+4 evaluation covers most of the algorithmic operation within McMASA.

### Bitrates 48 to 96 kbps (ParamMC)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 38597c33 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b, after a format conversion using the selection test processing scripts [16].

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 7.1, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 4 | 48000, 64000, 80000, 96000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

##### Objective evaluation 7.1 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

### Bitrates 128 to 512 kbps (MCT)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 38597c33 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b, after a format conversion using the selection test processing scripts [16].

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 7.1, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 6 | 128000, 16000, 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

##### Objective evaluation 7.1 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

## Multichannel 5.1+2

### Bitrates 13.2 to 32 kbps (McMASA)

5.1+2 input was not formally evaluated for bitrates 13.2 to 32 kbps. Nevertheless, the automated verification system of BASOP development contains these input test cases while 5.1 and 7.1+4 evaluation covers most of the algorithmic operation within McMASA.

### Bitrates 48 to 96 kbps (ParamMC)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 916e6860 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b, after a format conversion using the selection test processing scripts [16].

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 5.1+2, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 4 | 48000, 64000, 80000, 96000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

##### Objective evaluation 5.1+2 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

### Bitrates 128 to 512 kbps (MCT)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 916e6860 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b , after a format conversion using the selection test processing scripts [16].

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 5.1+2, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 4 | 128000, 16000, 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

##### Objective evaluation 5.1+2 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

## Multichannel 5.1+4

### Bitrates 13.2 to 64 kbps (McMASA)

5.1+4 input was not formally evaluated for bitrates 13.2 to 64 kbps. Nevertheless, the automated verification system of BASOP development contains these input test cases while 5.1 and 7.1+4 evaluation covers most of the algorithmic operation within McMASA.

### Bitrates 96 to 128 kbps (ParamMC)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 916e6860 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b, after a format conversion using the selection test processing scripts [16].

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 5.1+4, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 2 | 96000, 128000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

##### Objective evaluation 5.1+4 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

### Bitrates 160 to 512 kbps (MCT)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 916e6860 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b , after a format conversion using the selection test processing scripts [16].

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 5.1+4, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 5 | 16000, 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

##### Objective evaluation 5.1+4 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

## Multichannel 7.1+4

### Bitrates 13.2 to 96 kbps (McMASA)

#### Objective evaluation

The 7.1+4 input operation on bitrates 13.2 to 96 kbps was evaluated objectively using material from the selection experiments. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core and with the fix not yet implemented, a constant gain of -11 dB was applied to the input signals before processing them with the reference encoder. This allows for more meaningful objective measures.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 7.1+4, Binaural, Mono, Stereo |
| Bitrates | 13.2 – 96 |
| Bitrate switching | Within 13.2 – 96 |
| FER | Clean and 10% (ep_10pct.g192) |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 16, 32, 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate, FER, DTX, bandwidth and output fs. A total of 2416 cases are presented below in the histogram figures.

In general, the MLD values are low for all tested cases. Minimum SSNR values are a bit better than with the 5.1 input. However, there seems to be outliers under 20 for SSNR which would need further inspection.

#### Subjective evaluation

No formal subjective evaluation was performed for 7.1+4 input for bitrates 13.2 to 96 kbps. Informal evaluation was performed for cases that had possible issues based on the objective measures.

#### Issues identified

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |

### Bitrates 128 kbps (ParamMC)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 31303809 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 7.1.4, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 1 | 128000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

In general the MLD requirements are met the majority of test cases with the exception of outliers for FER and JBM for binaural output. However, the low SNR scores for a number of cases for the pass-through mode need to be investigated.

##### Objective evaluation 7.1+4 output

##### Objective evaluation BINAURAL

##### Objective evaluation BINAURAL_ROOM_IR

#### Subjective evaluation

For the subjective evaluation a Ref A-B test was conducted. From the objective evaluation it can be concluded that in terms of MLD and SSNR there is a degradation for conditions with FER and DTX. Therefore, these conditions were selected to be tested for the subjective evaluation.

Listening test conditions

- 7.1+4 128 kbps at 48 kHz FB

- FER 0%

- DTX off

- Decoder output format: Binaural

- Version tested SHA 3facce7f

- Items were selected from the selection test database. Worst case items in terms of MLD were selected for the listening test.

Listening test results

The results from 6 expert listeners are shown below. It can be concluded that the perceptual quality is on par with a slight tendency towards degradation. No systematic or worrying degradation was observed from the listeners.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1010 | Complexity: High Complexity Overhead for ParamMC decoding | Complexity | BASOP | Critical | Open |

### Bitrates 160 kbps (ParamUpmix)

#### ParamUpmix Objective Test

MLD and SSNR objective metrics were run at 160kbps on 64 items from the IVAS BS1534-3a selection tests. Results indicate audio quality problems related to PLC as well as saturation problems with high input level. These issues have been reported and are pending resolution. Test parameters were as follows:

Table 6: Parameters of ParamUpmix Objective Test (12 combinations, release commit)

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Bit rates | 1 | 160000 bps |
| Level | 3 | {-16, -26, -36} LKFS |
| PLC | 4 | {0%, 5% (random), 10% (deterministic), 15% (random)} FER |

MLD and SSNR histograms are shown below. A severe increase in MLD is observed with increasing level of the input signal and in PLC operation.

Figure 12: MC714 @ 160kbps Objective Test Results (all conditions)

Figure 13: MC714 @ 160kbps Objective Test Results (no packet loss)

Figure 14: MC714 @ 160kbps Objective Test Results (packet loss)

Figure 15: MC714 @ 160kbps Objective Test Results (-36LKFS)

Figure 16: MC714 @ 160kbps Objective Test Results (-26LKFS)

Figure 17: MC714 @ 160kbps Objective Test Results (-16LKFS)

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1005 | Audio clipping at -16LKFS, 7.1.4 @160kbps | Audio Quality | ParamUpmix | Medium | Pending |
| 1006 | Very loud distortion with PLC, 7.1.4 @160kbps | Audio Quality | ParamUpmix, PLC, LFE | Blocker | Pending |

### Bitrates 192 - 512 kbps (MCT)

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 31303809 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiments BS1534-3a and BS1534-3b.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | 7.1.4, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 4 | 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

The general criteria for MLD are met with the exception of a few outliers for FER and JBM for all output formats and the SSNR criteria are adequate.

##### Objective evaluation 7.1+4 output

##### Objective evaluation BINAURAL output

##### Objective evaluation BINAURAL_ROOM_IR output

#### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 959 | Decoder crash for MCT-MC 7.1+4 > 192kbps JBM operation | Crash | JBM | Blocker | Resolved |
| 937 | Decoder crash for MC 7.1+4 at 192/256/384/512kbps JBM in idiv1616() | Crash | JBM | Blocker | Resolved |
| 930 | Decoder crash for MC 7.1+4 at 192 kbps JBM operation in IVAS_DEC_VoIP_reconfigure() | Crash | JBM | Blocker | Resolved |
| 925 | Decoder crash for MC 7.1+4 at 384kbps in IGF_replaceTCXNoise_1() | Crash | Decoding | Blocker | Resolved |

## ISM

### ISM from 13.2 kbps up 48 kbps

#### Objective evaluation

VoiceAge evaluated the maximum MLD and the minimum segmental SNR for the following operating points of the fixed-point decoder. Note that minimum Seg.SNR is not so meaningful in case of ISM as many objects have only sporadic content, thus for many frames the Seg.SNR is computed on almost silence leading to very low Seg.SNR numbers.

| Format | Bit rates | DTX | FER | Level |
| --- | --- | --- | --- | --- |
| ISM 1 | 13.2, 16.4, 24.4, 32 and 48 kbps | On and Off | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |
| ISM 2 | 24.4 and 32 kbps | On and Off | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |
| ISM 3 | 48 kbps | On and Off | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |
| ISM 4 | 48 kbps | On and Off | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |

The databases used included the selection test database and some internal databases. The fixed-point decoder commit used was 6f1fdf53.

##### ISM 1

| 13.2 kbsp up to 48 kbps, with DTX on and off, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | MLD | Seg.SNR |
| Nominal |  |  |
| +10 dB |  |  |
| - 10 dB |  |  |

##### ISM 2

| 24.4 kbsp and 32 kbps, with DTX on and off, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | Max. MLD | Min. Seg.SNR |
| Nominal |  |  |
| +10 dB |  |  |
| - 10 dB |  |  |

##### ISM 3

| 48 kbps, with DTX on and off, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | Max MLD | Min. Seg.SNR |
| Nominal |  |  |
| +10 dB |  |  |
| - 10 dB |  |  |

##### ISM 4

| 48 kbps, with DTX on and off, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | Max MLD | Min. Seg.SNR |
| Nominal |  |  |
| +10 dB |  |  |
| - 10 dB |  |  |

#### Subjective evaluation

VoiceAge run two ISM with binaural output subjective tests using the material from the selection. All tests were done with ref-A-B comparison style and the listeners were a mix of experts and students from university. The stereo operating points tested were:

- 1 object at 16.4 kbps

  - Based on p800-6

  - Clean content

  - 5% of frame lost

  - DTX on

- 4 objects at 48 kbps

  - Based on BS1534-6b

  - Clean content

  - No frame lost

  - DTX off

##### ISM-1 16.4 kbps

- 13 Listeners

- 16 samples

- For a confidence level of 95% :

| Number of votes | 208 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Average global | -0.101 |  | SEMD | t | Prob (TDIST) |  |
| STD global | 0.640 |  | 0.0444 | 2.2739 | 0.0120 |  |
| P(T<=t) one-tail | 0.0120 | Fixed point decoder not worse than floating point |  |  |  |  |

##### ISM-4 48 kbps

- 13 Listeners

- 16 samples

- For a confidence level of 95% :

| Number of votes | 208 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Average global | 0.034 |  | SEMD | t | Prob (TDIST) |  |
| STD global | 0.718 |  | 0.0498 | 0.6758 | 0.2500 |  |
| P(T<=t) one-tail | 0.2500 | Fixed point decoder not worse than floating point |  |  |  |  |

* The cases where REF  >> CuT are related to the issue 881

### ISM3 and ISM4 from 24.4 and 32 kbps

#### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 4e6c6d71 and floating point reference version d18cbee3.

The test case combinations are given in the table below. The inputs vectors are taken from the selection experiment P800-6.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | EXT, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 8 | 24400, 32000 |
| Number of objects | 2 | ISM3, ISM4 |
| Channel conditions | 15 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| DTX | 2 | On/Off |
| Loudnes Level |  | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER and/or DTX

- JBM

Overall the majority of the test cases lie under the MLD score of 5 threshold except for some outliers, and case with high MLD for FER (issues listed in 6.7.2.4). What needs attention and still needs to be investigated are the low SSNR outliers (23-31dB).

##### Objective evaluation EXT output

Tests with clean channel / DTX off

Tests with FER and/or DTX on

Tests with JBM

##### Objective evaluation BINAURAL output

Tests with clean channel / DTX off

Tests with FER and/or DTX on

Tests with JBM

##### Objective evaluation BINAURAL ROOM output

Tests with clean channel / DTX off

Tests with FER and/or DTX on

Tests with JBM

#### Subjective evaluation

For the subjective evaluation a Ref A-B test was conducted. From the objective evaluation it can be concluded that in terms of MLD and SSNR there is a degradation for conditions with FER and DTX. Therefore, these conditions were selected to be tested for the subjective evaluation.

Listening test conditions

- ISM3 32 kbps at 48 kHz FB

- FER 5%

- DTX on

- Decoder output format: Binaural

- Version tested SHA 3facce7f

- Items were selected from the selection test database. Worst case items in terms of MLD were selected for the listening test.

Listening test results

The results from 6 expert listeners are shown below. It can be concluded that the perceptual quality is on par with a slight tendency towards degradation. No systematic or worrying degradation was observed from the listeners.

#### Other observations

Void.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1020 | ParamISM: Binaural: HF click | Audio Quality | Decoding | Major | Open |
| 1019 | ParamISM: Ext-Output: Difference in high-band, pt. 02 | Audio Quality | Decoding | Major | Open |
| 1018 | ParamISM: Ext-Output: Difference in high-band | Audio Quality | Decoding | Major | Open |
| 1017 | ParamISM: Ext-Output: Difference in signal amplitude | Audio Quality | Decoding | Major | Open |
| 1016 | ParamISM: Signal leakage above stop-frequency | Audio Quality | Decoding | Major | Open |
| 1015 | ParamISM: Higher noise-floor, tonal lines | Audio Quality | Decoding | Major | Open |
| 009 | Complexity: High Complexity Overhead for ParamISM decoding to binaural | Complexity | BASOP | Critical | Open |
| 969 | ParamISM: For JBM, Noise-filling/comfort noise insertion missing | Audio Quality | JBM | Critical | Open |
| 964 | ParamISM: High MLD for ParamISM, 32 kbps, high frequency content missing | Audio Quality | Decoding | Major | Open |
| 933 | Decoder: High MLD for ParamISM, 32 kbps, DTX off | Audio Quality | Decoding | Major | Resolved |
| 929 | Decoder: High MLD for ParamISM3, 24.4 kbps, DTX on | Audio Quality | Decoding | Major | Open |

### ISM from 64 kbps up to 512 kbps

#### Objective evaluation

To cover a wider range of operating points, a short input (11 second) was used in combination with objective measures. The test files (ISM1, ISM2, ISM3 and ISM4) comprised of mixed speech, music, and noise items. The extended metadata files for the objects were also provided. The tests for objective verification and informal listening were:

- Bitrates, FER conditions, number of objects and DTX.

- Sampling rates and output configurations.

- Bitrates and bandwidth selection.

- Bitrate switching, head tracking, renderer configuration and renderer frame size.

- JBM conditions.

- Extensive testing: sampling rates, bitrates, DTX, FER, Bandwidth limitations.

- Selection test processing for ISM tests (P800-6, P800-7, BS1534-6a and BS1534-6b).

ISM test 1 - Bitrates, FER conditions, number of objects and DTX

The parameters for test 1 are summarized in Table 7. A total of 96 combinations are tested with 4 variable parameters. For ISM1, the applicable bitrates are 64, 96 and 128 kbps whereas for ISM2 and ISM3, 4 bitrates are applicable.

Table 7: Parameters of ISM test 1 Bitrates, loss profiles, Number of objects. 96 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 1 | BINAURAL |
| Bit rates | 5 | 64, 96, 128, 256, 512 kbps |
| DTX | 2 | On/Off |
| Channel conditions | 3 | FER 0%, FER 3%, FER 10% |
| Number of Objects | 4 | ISM1, ISM2, ISM3, ISM4 |

The objective measurement results of test 1 in terms of MLD and SSNR are provided in histogram plots in Figure 18. The scores did not present any problematic cases with highest MLD score of 2.861. An informal listening test from an earlier version of the code (1a1e5374) showed a distortion for ISM2 test vector with FER 10% loss profile at 96kbps. Issue #936  was filed and the issue is resolved with merge request !688.

Figure 18 Histograms of subjective measurements for ISM Test 1; a) MLD, b) Min SSNR.

ISM test 2 – Sampling rates and output configurations

The test parameters for test 2 are presented in Table 8, which resulted in 126 combinations for testing 3 different parameters.

Table 8: Parameters of ISM test 2: Sampling rates, output configurations. 126 combinations

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 3 | 16, 32, 48 |
| Output sampling rate | 3 | 16, 32, 48 |
| Output formats | 14 | MONO, STEREO, 5_1, 7_1, 5_1_2, 5_1_4, 7_1_4, FOA, HOA2, HOA3, BINAURAL, BINAURAL_ROOM_IR, BINAURAL_ROOM_REVERB, EXT |
| Bit rates | 1 | 256000 |
| DTX | 1 | Off |
| Channel conditions | 1 | Clean |
| Number of Objects | 1 | ISM4 |

The histograms of objective measurements for test 2 are provided in Figure 19. The highest MLD score for this test is 2.534 and all HOA3 output test cases resulted in MLD scores higher than 2.  A previous version of the code resulted a high MLD score for BINAURAL ROOM REVERB output which was reported as an issue in #920 and later resolved.

Figure 19 Histograms of subjective measurements for ISM Test 2; a) MLD, b) Min SSNR.

ISM test 3 – Bandwidth limitations

Different bandwidth limitations are tested for 3 different bitrates in test 3 and the test parameters are presented in Table 9.

Table 9: Parameters of ISM test 3: Bitrates, Bandwidth limitations. 12 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 1 | BINAURAL |
| Bit rates | 3 | 64, 96, 128 kbps |
| DTX | 1 | Off |
| Channel conditions | 1 | Clean |
| Number of Objects | 1 | ISM2 |
| Bandwidth | 4 | WB, SWB, FB, Bandwidth switching profile |

The histograms of objective measurements are presented in Figure 20. The highest MLD score in this test is 0.928.

Figure 20 Histograms of subjective measurements for ISM Test 3; a) MLD, b) Min SSNR.

ISM test 4 – Renderer conditions

Test 4 parameters are presented in Table 10 which resulted in 54 combinations. Two different head tracking files are provided with the movement being forward for the first case and rotating in the second. Two different renderer configurations are tested as well.

Table 10: Parameters of ISM test 4: Renderer conditions; head tracking, renderer configuration, frame size, bitrate, output format. 54 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 2 | BINAURAL, BINAURAL_ROOM_REVERB |
| Bit rates | 2 | 128 kbps, bitrate switching from 48 to 384 kbps. |
| DTX | 1 | Off |
| Channel conditions | 1 | Clean |
| Number of Objects | 1 | ISM3 |
| Head Tracking | 3 | None, moving forward, rotating |
| Render Frame Size | 3 | 5 ms, 10 ms, 20 ms |
| Renderer Configuration | 3 | None, object directivity, recreation |

The highest MLD score for test 3 is 1.665. The histograms of the objective measurements are presented in Figure 21. The code version 2b6dc1f3 introduced a crash for BINAURAL ROOM REVERB output with the renderer configuration ‘recreation’. The issue was reported in #935 and resolved in !692. The informal listening revealed minor differences, but no further issues were observed.

Figure 21 Histograms of subjective measurements for ISM Test 4; a) MLD, b) Min SSNR.

ISM test 5 – Jitter Buffer Manager profiles

The parameters for test 5 are presented in Table 11 which consists of 33 combinations. A total of 11 error profiles are tested for 3 different output configurations.

Table 11: Parameters of ISM test 5: Delay profiles, output format. 33 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | BINAURAL, BINAURAL_ROOM_IR, BINAURAL_ROOM_REVERB |
| Bit rates | 1 | 256 kbps |
| Number of Objects | 1 | ISM4 |
| DTX | 1 | Off |
| Channel conditions | 11 | JBM dly_error_profile_{0..11}.dat |

The objective measurement scores for JBM test cases do not provide a valuable insight since they are sensitive to the time alignment of the output signals. The timing is expected to differ between BASOP code output and floating-point code output to some extent. The histograms of the test scores are provided in Figure 22. The informal listening revealed the time alignment differences in some cases, but no major issues were observed.

Figure 22 Histograms of subjective measurements for ISM Test 5; a) MLD, b) Min SSNR.

ISM test 6 – Extensive ISM testing, Sampling rates, DTX, FER, Bitrates, Bandwidth limitations

The parameters for test 6 are presented in Table 12 which consists of 4320 combinations. All combinations are tested with extended metadata. Since the maximum bitrate supported for ISM1 is 128kbps, ISM1 is tested with bitrates 64, 96, 128 kbps and a bitrate switching profile from 48kbps to 128 kbps. This range allows the metadata switching between normal and extended metadata since extended metadata is supported for bitrates from 64kbps. The same principle applies to ISM2 where the tested bitrates are 64, 96, 128, 256 kbps and a bitrate switching profile from 48kbps to 128kbps. For ISM3, the bitrate parameters are the same with ISM2 with the only change in bitrate switching profile where the range is between 48 kbps and 384 kbps. For ISM4, the additional bitrate is 512 kbps, and the switching profile is changing in a range between 48kbps and 512 kbps.

Table 12: Parameters of ISM test 6: Sampling rates, Bitrates, DTX, FER, Bandwidth limitations, 4320 combinations.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input sampling rate | 3 | 48 |
| Output sampling rate | 3 | 48 |
| Output formats | 1 | BINAURAL |
| Bit rates | 6 | 64, 96, 128, 256, 512 kbps and bitrate switching |
| Number of Objects | 4 | ISM1, ISM2, ISM3, ISM4 |
| DTX | 2 | On, Off |
| Channel conditions | 3 | FER 0%, FER 3%, FER 10% |
| Bandwidth | 4 | WB, SWB, FB, Bandwidth switching profile |

The objective measurement scores for ISM test 6 are summarized in histogram plots in Figure 23. While most of the cases have low MLD scores, there are around 1140 test cases with MLD higher than 20. A closer look at these test cases revealed that a combination of FER and bandwidth limitation at higher bitrates results in serious clicks for these cases. An issue is filed in #1008. In addition to that, a crash is observed for 42 ISM 3 cases with FER10% at 128kbps and bandwidth switching. An issue is filed for the crash in #1007 which is later resolved with MR !757.

Figure 23 Histogram plots of objective measures for ISM Test 6, extensive ISM testing. The colors represent the configurations as Blue: Normal operation, Orange: FER, Yellow: DTX on, Purple: Bitrate Switching. a) MLD, b) Min SSNR.

Selection test processing for ISM tests (P800-6, P800-7, BS1534-6a and BS1534-6b)

The ism experiments of the IVAS selection test were processed using the float reference and the BASOP verification version, and the MLD scores were calculated for the decoded items. Items with the highest MLD scores were identified and reviewed with informal listening and inspection. The objective scores are shown in Figure 23 . Notes from the informal listening of the highest MLD items:

- High energy in high frequency part of transient for ISM3. An issue has been filed: https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/994

- High energy in high frequency of speech onset for ISM2. An issue has been filed: https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/995

Figure 24: Histogram of MLD scores for selection test ISM material.

#### Subjective evaluation

A Ref-AB listening test is conducted with 7 experts for the subjective assessment of the BASOP decoder output for ISM. A total of 28 ISM4 items are used from the selection test material:

- Method: Ref-AB

- Operating point: ISM4 @96kbps, 48 kHz input and output, error free channel

- Material: 28 Mixed items (mixed speech/music, sound effects) with relevant metadata

- Listeners: 7

The objective measurements were evaluated for the selected material. The histogram for MLD scores is presented in Figure 24. We observe only 1 item with MLD>5.

Figure 25 Histogram of MLD scores for ISM listening test items.

The results of the subjective Ref-AB listening test reveals a slight preference of floating-point decoder output over the BASOP decoder output in Figure 25. A negative score means a preference for floating-point code output. Even though a simple t-test analysis shows the average score to be significantly different from ‘0’ (p-value=0.0345), the 95% confidence intervals are quite slim, [-0.0688, -0.0026]. In addition to that, for none of the 28 items, more than one listener perceived the quality of the BASOP to be worse than the floating-point code. It can also be seen from the Figure 25 that, the high MLD score does not necessarily result in subjectively different scores.

Figure 26 Ref-AB Listening test results for ISM4 @96kbps. MLD scores for test items are presented in orange on the top.

### Issues identified

The issues identified as a result of objective and subjective testing are summarized in Table 13 for ISM at higher bitrates (64 kbps and higher).

Table 13 List of issues identified following the tests for ISM at bitrates 64 kbps and higher.

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| #936 | Distortion for ISM2 + FER 10% loss profile at 96kbps | 1a1e5374 | PLC | Minor | Resolved (!688) |
| #920 | High MLD score for BINAURAL ROOM REVERB output | Earlier | Room Reverb | Major | Resolved |
| #935 | Crash for BINAURAL ROOM REVERB output with ‘recreation’ renderer configuration | 2b6dc1f3 | Room Reverb | Blocker | Resolved !692 |
| #994 | High energy in high frequency part of transient for ISM3 | 074e9116 | Decoder | Critical | Resolved |
| #995 | High energy in high frequency of speech onset for ISM2 | 074e9116 | Decoder | Critical | Open |
| #1007 | Decoder crash ISM3+FER, Bandwidth switching at 96kbps, 128 kbps. | 9bae8e35 | PLC | Blocker | Resolved !757 |
| #1008 | Serious clicks ISM+FER at higher bitrates | 9bae8e35 | PLC | Major | Open |

## SBA

### Computational Complexity

Because improvements have been made to fixed-point complexity in recent commits after the 23/10/24 decoder release, the results reported below correspond to fixed-point commit a41b2422 from 04/11/24. The tested configuration is HOA3 input decoded to HOA3 output, and we compare fixed point complexity against the floating-point ivas-float-update-cmplx branch commit de702f5c from 6/11/2024. At each bitrate, the comparison was performed for all supported bandwidths and DTX settings. The figure below shows the resulting WMOPS complexity ratios between BASOP and FLOAT. We see that in most operating modes, the fixed-point WMOPS are somewhat higher than in floating point. The average ratio is 1.12, which is 1.47 times the target of 0.76. The range of values is [0.89, 1.23].

Figure 27: SBA WMOPS Complexity Ratios (BASOP/FLOAT).

### Objective Evaluation

A first test (Test1) with objective quality metrics was run with pre-release commit 77241999 (21/10/2024). Test content was selected from IVAS Selection MUSHRA and P.800 tests (105 items, ~18.5 minutes). Input content was normalized using BS.1770-4 [14] so that the uncoded binaurally rendered reference was at a specific LKFS. Both tests measured MLD and SSNR between floating-point decoder and fixed-point decoder. Before measurement, decoded output was binaurally rendered (output type = BINAURAL) using the IVAS floating-point external renderer. All inputs were encoded using the IVAS floating-point encoder. Input and output sampling rates were 48 kHz. JBM operation was not tested.

Two further objective tests were carried out with the Fixed-point Decoder Release [4]: Test2, to measure general quality, and Test3 to specifically measure the effect of DTX operation. For Test2, test items consisted of speech, music, and mixed content (21 items, ~3.0 minutes). Most of the items were previously used in the IVAS Selection Phase MUSHRA tests. For Test3, speech content from P800-4 and P800-5 IVAS Selection Tests was used in order to trigger the DTX function (84 items, ~9.2 minutes). Test parameters are listed below:

Table 14: Parameters of SBA Test1 (567 combinations, pre-release commit 77241999)

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Bit rates | 14 | {13200, 16400, 24400, 32000, 48000, 64000, 80000, 96000,128000,160000, 192000, 256000, 384000, 512000} bps |
| Order | 3 | {FOA, HOA2, HOA3} |
| Level | 3 | {-16, -26, -36} LKFS |
| DTX | 2 | {Off, On} (DTX On only supported at up to 80kbps bit rate) |
| PLC | 3 | {0%, 5% (random), 10% (deterministic)} FER |

Table 15: Parameters of SBA Test2 (48 combinations)

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Bit rates | 8 | {13200, 16400, 24400, 48000, 96000 256000, 384000, 512000} bps |
| Order | 1 | HOA3 |
| Level | 3 | {-16, -26, -36} LKFS |
| DTX | 1 | {Off} |
| PLC | 2 | {0%, 5%} FER (random pattern) |

Table 16: Parameters of SBA Test3 (60 combinations)

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Bit rates | 5 | {13200, 16400, 24400, 48000, 80000} bps |
| Order | 1 | FOA |
| Level | 3 | {-16, -26, -36} LKFS |
| DTX | 2 | {Off, On} |
| PLC | 2 | {0%, 5%} FER (random pattern) |

The following 3 figures show overall histograms for each test.

Figure 28: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test1)

Figure 29: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2)

Figure 30: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3)

The remainder of the analysis will focus on Test2 and Test3, which ran on the release commit.

The 3 figures below show degradation in the test metrics when the input signal level rises. Examination of high MLD and/or low SSNR test cases at -16LKFS led to the discovery and reporting of several issues (see section 5.8.5).

Figure 31: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2, -36LKFS)

Figure 32: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2, -26LKFS)

Figure 33: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2, -16LKFS)

PLC is another codec feature where several problems were identified, especially in combination with high amplitude inputs. For -16LKFS inputs with packet loss, MLD was above the 5.0 target level in over 10% of test cases.

Figure 34: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2, No packet loss)

Figure 35: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2, packet loss)

Figure 36: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test2, -16LKFS and packet loss)

DTX operation shows a negligible effect on the objective measures:

Figure 37: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, DTX off)

Figure 38: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, DTX on)

The bimodal distribution in SSNR can be attributed to bitrate differences, as shown below, with lower SSNR at lower bitrates.

Figure 40: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, 13.2 kbps)

Figure 41: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, 16.4 kbps)

Figure 42: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, 24.4 kbps)

Figure 43: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, 48 kbps)

Figure 44: Histogram of IVAS BASOP to FLOAT Objective Metrics (Test3, 80 kbps)

### Subjective Evaluation

9 listening experiments were carried out in accordance to ITU-R BS.1534 (MUSHRA) [13] to evaluate the audio quality of the IVAS fixed-point decoder against the floating-point reference. All tests were performed on headphones. The input SBA content was coded by the floating-point encoder and decoded to SBA, then rendered to binaural using the floating-point IVAS external renderer (output type BINAURAL). In each test, the 7 conditions consisted of the uncoded reference (directly rendered to binaural), a hidden reference, a 7kHz low-pass filtered uncoded anchor, and 4 conditions under test (CuTs) consisting of two floating-point/fixed-point condition pairs at the same bitrate. This is illustrated in the following table:

| Test condition | Description |
| --- | --- |
| C1 | Reference |
| C2 | Hidden Reference |
| C3 | 7 kHz anchor |
| C4 | Bitrate A, floating point decoder |
| C5 | Bitrate A, fixed point decoder |
| C6 | Bitrate B, floating point decoder |
| C7 | Bitrate B, fixed point decoder |

Each test covered two similar bitrates, chosen to minimize the range of audio quality that was presented in a single test. The sampling rate was 48 kHz and the default bandwidth setting was used. There were 8 experienced listeners per test.

For continuous transmission (DTX off) tests, content consisted of a subset of the items used for the Test2 objective test (see section 6.8.2). Content for the DTX tests consisted of a subset of the content used for the Test3 objective test. All input content was normalized using BS.1770-4 [14] so that the uncoded binaurally rendered reference was at -26 LKFS.

The floating-point reference binaries (encoder, decoder, renderer) were built from commit d9fdb374 in the 3GPP Forge IVAS BASOP git repository (ivas-float-update branch) [6]. Subjective testing was performed in the period between 25/09/2024 to 6/11/2024. During this time, bug fixes were being continually made to the fixed-point code, such that it was not possible to run all tests with a single version of the code. The table below summarizes the content of each test and includes the commit ID of the fixed-point decoder used in each test.

| Test ID | Input Format | Output Format | DTX (on/off) | Bitrates | FX Decoder Commit |
| --- | --- | --- | --- | --- | --- |
| SBADEC1 | FOA | FOA | Off | 13.2 and 16.4 kbps | 29b454bb |
| SBADEC2 | FOA | FOA | Off | 48 and 80 kbps | 29b454bb |
| SBADEC3 | FOA | FOA | Off | 24.4 and 32 kbps | 29b454bb |
| SBADEC4 | HOA3 | HOA3 | Off | 96 and 160 kbps | 912d574fc |
| SBADEC5 | HOA3 | HOA3 | Off | 256 and 512 kbps | 912d574fc |
| SBADEC6 | FOA | HOA31 | On | 13.2 and 24.4 kbps | 77241999 |
| SBADEC7 | FOA | HOA31 | On | 48 and 80 kbps | 77241999 |
| SBADEC8 | HOA3 | HOA3 | Off | 13.2 and 24.4 kbps | 77241999 |
| SBADEC9 | HOA3 | HOA3 | Off | 48 and 64 kbps | 77241999 |

1 HOA3 output was selected in these tests to verify the operation of DirAC-based upmixing from FOA to higher-order.

The results of these tests were used to direct focused listening by experts to identify audio quality problems that were not flagged by the objective quality assessment of section 6.8.2.

### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 896 | PLC crash at 16.4kbps on one item. | Crash | PLC | Blocker | Resolved |
| 926 | Incorrect use of overflow left-shift operators throughout the code. | Code quality | BASOP | Medium | Ongoing |
| 931 | Crash at 16.4kbps on one test item. | Crash | Bandwidth extension (core coder) | Blocker | Resolved |
| 934 | Spectral distortion at 32kbps | Audio Quality | SBA | Major | Resolved |
| 951 | Audibly higher noise floor across bitrates (even at 512kbps) on one item | Audio Quality | Unknown | Major | Pending |
| 952 | Unexpected ~2kHz tone at 13.2kbps on many test vectors | Audio Quality | Unknown | Major | Pending |
| 970 | Loss of signal amplitude on some received frames | Audio Quality | PLC | Major | Resolved |
| 971 | Waveform discrepancy at transition to received frames, at low bitrates | Audio Quality | PLC | Major | Resolved |
| 972 | Fast amplitude fadeout during packet lost | Audio Quality | PLC | Major | Resolved |
| 973 | Waveform discrepancy on received frames | Audio Quality | PLC | Major | Pending |

## MASA

### Objective evaluation

Objective evaluation of the MASA format was performed on the fixed-point decoder commit with SHA 9bae8e35. Evaluation was performed for two separate sets of input. The first set contains input vectors taken from the selection experiments. The second set contains the long test vectors used in the automated testing of the BASOP conversion, namely ltv48_MASA1TC.wav/.met and ltv48_MASA2TC.wav/.met available in [16]. These long test vectors have been designed specifically to test MASA codec as widely as possible.

The following combinations of operation mode variations were used.

| Parameter | First set (selection experiments) | Second set (LTV) |
| --- | --- | --- |
| Output format | EXT, Binaural, Mono, Stereo, 7.1+4 | EXT, Binaural, Binaural Room IR, Mono, Stereo, 7.1+4, FOA |
| Bitrates | 13.2 – 512 (all IVAS bitrates) | 13.2 – 512 (all IVAS bitrates) |
| Bitrate switching | All bitrates | All bitrates |
| FER | Clean and 10% (ep_10pct.g192) | Clean and 10% (ep_10pct.g192) |
| DTX | On/off | On/off |
| Bandwidth | WB, SWB, FB | WB, SWB, FB |
| Output FS | 16, 32, 48 | 16, 32, 48 |
| Transport channels | 2 TC | 1 TC and 2 TC |
| Number of directions | 1dir and 2dir | Switching 1dir/2dir in one file |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate, FER, DTX, bandwidth and output fs. A total of 8873 cases for first set and 530 results for the second set are presented below in the histogram figures. There were also combinations that did not process successfully, and these have been or will be inspected further. According Gitlab issues are filed for these.

As is seen in the figures, a significant number of cases produce low MLD values. However, there are also high MLD and low Min SSNR cases present, especially in the second set. The high MLD values are often explained with an audible artifact (e.g., in switching or concealment for FER) present as a difference.

The MASA format EXT output contains also MASA format metadata output. These were also analyzed to see if there are any problems in the metadata coding path. The result is that of all above cases pooled, the worst case had metadata difference in 49 frames total. The difference in all frames was in the direction metadata and is under 5 degrees. This is considered to be a good result. It should be noted that there were observed more problems in metadata coding in earlier versions but that stated evaluated version has these problems fixed.

### Subjective evaluation

Subjective evaluation of the MASA format was performed on the fixed-point decoder commit with SHA 46f1fdf5. Two separate listening tests were conducted for MASA 1TC and MASA 2TC. Test material contained clean and noisy speech in Finnish language. Some background noises contained music signals. Input signals originated from FoA, HoA2 and Eigenmike sources. Both MASA 1Dir and 2Dir material was included to the listening tests. Some of the test signals were produced with SRIR processing like in IVAS selection testing, rest of the test material were true spatial audio recordings (all Eigenmike material). Overall it can be said that the test material was relatively difficult. Listening was conducted with binaural rendering using Sennheiser HD650 headphones. DCR test methodology was used. Majority of the listeners were experts. Total number of listeners was 14 for MASA 1TC and MASA 2TC listening tests.

During processing some crashes occurred with MASA 1TC processing, when DTX was used together with bitrate switching. These conditions were changed to bitrate switching with DTX disabled.

For MASA 1TC listening no significant differences between float-float and float-fixed coding was found. Some particularly bad looking samplewise results were additionally manually checked and listened to, but no clear problems were identified with the subjective test material.

For MASA 2TC listening test one significant difference between float-float and float-fixed coding was found (MASA 2TC rate switching 16.4k - 96k with DTX enabled and FER5). Some particularly bad looking samplewise results were manually inspected (spectrogram) and listened to, but no clear problems were identified with the subjective test material. Overall, this condition sounds quite bad and the difference in DCR MOS score is likely statistical variance.

### Other observations

In general, the “simple modes” of constant bitrates and full band operation seem to work quite well. This is expected as the automated verification framework contains these cases. However, more issues are noticed when lower bandwidths and output sample rates are combined with FER, DTX, and rate switching. These should be a clear focus of further improvement within MASA format.

Objective evaluation was also tested for JBM operation points but objective comparison between then output signals is less meaningful as differences in decoded audio signal may produce different JBM decision and cause significant objective measure differences while being completely inaudible. However, some issues were noticed in the JBM operation with it not being as stable as the non-JBM operation.

### Issues identified

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1040 | High frequency "modulation" in rate switching MASA to MONO output | Audio quality | Decoder, MASA | Minor | Open |
| 1003 | Switching artifacts worse in LTV MASA 2TC DTX FER WB to BINAURAL | Audio quality | Decoder, MASA, DTX + PLC | Medium | Open |
| 1000 | Serious clicks in MASA 1TC LTV at 96 kbps using DTX + FER + WB and output to EXT at 32 kHz | Audio quality | Decoder, MASA, DTX + PLC | Major | Ongoing |

## OSBA

### Low bitrates 13.2 to 128 kbps

All the objective measurements presented in this section are based on the release version of the decoder. The content consisted of 32 items from the BS1534-6b IVAS selection tests.

The test case combinations are given in the table below (384 combinations). Measures were run on SBA outputs after being rendered to BINAURAL using the floating-point external IVAS renderer.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 1 | EXT |
| Bit rates | 8 | {13.2, 16.4, 24.4, 32, 64, 80, 96 128} kbps |
| Channel conditions | 4 | {0%, 5%, 10% (deterministic), 15%} FER |
| SBA | 1 | HOA3 |
| ISM | 4 | ISM4 |
| Loudness Levels | 3 | {-16, -26, -36} LKFS |

The MLD and SSNR histograms are shown below. MLD results indicate a strong degradation in quality when PLC is enabled, and an even stronger degradation when the input signal level is increased. At -16LKFS, 18% of test cases fail to meet the MLD criterion.

Figure 45: Histogram of IVAS BASOP to FLOAT OSBA ISM4, 13.2-128kbps

Figure 46: Histogram of IVAS BASOP to FLOAT OSBA ISM4, 13.2-128kbps, packet loss

Figure 47: Histogram of IVAS BASOP to FLOAT OSBA ISM4, 13.2-128kbps, no loss

Figure 48: Histogram of IVAS BASOP to FLOAT OSBA ISM4, 13.2-128kbps, -36LKFS

Figure 49: Histogram of IVAS BASOP to FLOAT OSBA ISM4, 13.2-128kbps, -26 LKFS

Figure 50: Histogram of IVAS BASOP to FLOAT OSBA ISM4, 13.2-128kbps, -16 LKFS

The results shown above are generally worse than what is reported for SBA operation in section 6.8.2. This is unexpected because at bit rates at or below 128kbps, the OSBA encoder is simply pre-rendering OSBA into SBA, such that the decoder is expected to behave very similarly (if not identically) to the SBA decoder. However, the audio content used in OSBA experiments is different than what was used for SBA, and this could be a source of difference in the above results.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1023 | Large spectral differences (up to 15dBFS) | Audio Quality | OSBA | Major | Open |
| 1024 | Time shift between float and fixed point output | Audio Delay | OSBA | Medium | Open |
| 1038 | High MLD scores in OSBA at <= 128kbps | Audio Quality | OSBA | Major | Open |

### High bitrates 160 to 512 kbps

#### HOA3 with ISM1, ISM2, ISM3 and ISM4

##### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 4eaf9a37 and floating point reference version d18cbee3.

The test case combinations are given in the table below.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | EXT, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 5 | 160000, 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| SBA | 1 | HOA3 |
| ISM | 4 | ISM1, ISM2, ISM3, ISM4 |
| Loudness Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

For OSBA HOA3 EXT output although the majority of test cases MLD scores fall under the MLD 5 threshold with the exception of outliers for both clean channel and FER and JBM, the performance wrt to SSNR is worrying low. As can be seen from the histograms below the majority of the test cases fall in to the range of 12-30 dB SSNR. It should be noted, that these numbers are not replicated for binaural or binaural room output. That may be an indication that underlying issues may be masked for binaural/ binaural room IR output especially for input formats with a large number of channels like OSBA.

##### Objective evaluation for EXT output

##### Objective evaluation for BINAURAL output

##### Objective evaluation for BINAURAL_ROOM_IR output

##### Subjective evaluation

For the subjective evaluation a Ref A-B test was conducted. From the objective evaluation it can be concluded that in terms of MLD and SSNR there is a degradation for conditions with FER. Therefore, these conditions were selected to be tested for the subjective evaluation.

Listening test conditions

- OSBA (ISM4 + HOA3) 320 kbps at 48 kHz FB

- FER 5%

- DTX off

- Decoder output format: Binaural

- Version tested SHA 3facce7f

- Items were selected from an Fraunhofer internal test database. Worst case items in terms of MLD were selected for the listening test.

Listening test results

The results from 6 expert listeners are shown below. Statistically significant degradations were found for Item 12 and over all items.

##### Other observations

Binaural render to mode “BINAURAL_ROOM_IR“ is not working for both, floating-point reference code and BASOP implementation (severe distortion for any output signal), and thus couldn’t be evaluated correctly.

##### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1014 | Clicks for OSBA at 384kbps with PLC | Audio Quality | PLC | Major | Open |
| 980 | OSBA 160 kbps JBM - Saturations | Audio Quality | JBM | Critical | Resolved |
| 997 | OSBA HOA3+ISM4 Crash at 256kbps | Crash | OSBA | Blocker | Closed |
| 977 | OSBA 160 kbps: Clicks in Concealment | Audio Quality | PLC | Major | Open |
| 976 | OSBA 160 kbps (pre-rendering): Spectral Holes below 4 kHz for several components | Audio Quality | Decoding | Major | Open |
| 974 | OSBA 160 kbps (pre-rendering): Discrepancy for SPAR part for several components | Audio Quality | Decoding | Major | Resolved |
| 963 | Decoder crash for OSBA_ISM3_PlanarHOA3 at 24.4kbps FER in lsp_reorder()/TCX core | Crash | PLC | Blocker | Resolved |
| 962 | Decoder crash for OSBA_ISM1_HOA3 at 80kbps FER in ivas_dirac_dec_binaural_formulate_input_and_target_covariance_matrices_fx() | Crash | PLC | Blocker | Resolved |
| 960 | Decoder crash for OSBA_2ISM_HOA3 at 32kbps with FER in pitch_pred_linear_fit() | Crash | PLC | Blocker | Resolved |

#### HOA2 with ISM1, ISM2, ISM3 and ISM4

##### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 916e6860 and floating point reference version d18cbee3.

The test case combinations are given in the table below.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | EXT, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 5 | 160000, 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| SBA | 1 | HOA2 |
| ISM | 4 | ISM1, ISM2, ISM3, ISM4 |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

Same overall observations as for OSBA with HOA3 in 6.10.2.1.1.

##### Objective evaluation for EXT output

##### Objective evaluation for BINAURAL output

##### Objective evaluation for BINAURAL_ROOM_IR output

##### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

##### Other observations

Binaural render to mode “BINAURAL_ROOM_IR“ is not working for both, floating-point reference code and BASOP implementation (severe distortion for any output signal), and thus couldn’t be evaluated correctly.

##### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1022 | OSBA 256 kbps: Various concealment clicks for 2nd object | Audio Quality | PLC | Critical | Open |
| 1021 | OSBA 384kbps PLC: Concealment introduces click | Audio Quality | PLC | Critical | Open |

#### FOA with ISM1, ISM2, ISM3 and ISM4

##### Objective evaluation

All the objective measurements presented in this section are based on BASOP version SHA 916e6860 and floating point reference version d18cbee3.

The test case combinations are given in the table below.

| Parameter | Number of combinations | Combinations |
| --- | --- | --- |
| Input sampling rate | 1 | 48 |
| Output sampling rate | 1 | 48 |
| Output formats | 3 | EXT, BINAURAL, BINAURAL_ROOM_IR |
| Bit rates | 5 | 160000, 192000, 256000, 384000, 512000 |
| Channel conditions | 14 | FER 0%, FER 5%, JBM dly_error_profile_{0..11}.dat |
| SBA | 1 | FOA |
| ISM | 4 | ISM1, ISM2, ISM3, ISM4 |
| Loudnes Level | 1 | -26 |

For better assessment of the performance the MLD and SSNR measurement plots for each output format are presented separately for:

- clean channel

- FER

- JBM

Same overall observations as for OSBA with HOA3 in 6.10.2.1.1.

##### Objective evaluation for EXT output

##### Objective evaluation for BINAURAL output

##### Objective evaluation for BINAURAL_ROOM_IR output

##### Subjective evaluation

No systematic subjective evaluation for these operation points have been carried out. Experts have only informally listened to a limited number of samples, especially when the objective measurements have indicated a significant difference from the floating-point decoder.

##### Other observations

Binaural render to mode “BINAURAL_ROOM_IR“ is not working for both, floating-point reference code and BASOP implementation (severe distortion for any output signal), and thus couldn’t be evaluated correctly.

##### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [6].

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1028 | OSBA 192 kbps: Concealment introduces clicks | Audio Quality | PLC | Critical |  |
| 1026 | OSBA 512 kbps: Differences in low-band | Audio Quality | Decoding | Medium | Open |
| 1025 | OBSA prerendering: High MLD for SBA component | Audio Quality | Decoding | Medium | Open |

## OMASA

### Objective evaluation

VoiceAge evaluated the maximum MLD and the minimum segmental SNR for the following OMASA operating points of the fixed-point decoder. Both the maximum MLD and the minimum segmental SNR have been computed on the synthesis rendered to the binaural domain.

| Format | Bit rates | Bandwidth switching | FER | Level |
| --- | --- | --- | --- | --- |
| ISM 1 | 24.4, 32, 48, 64, 96 kbps | 32, 64, 96 kbps | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |
| ISM 2 | 48, 64, 96 kbps | 48, 64, 96 kbps | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |
| ISM 3 | 96, 128, 192 kbps | 96 kbps | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |
| ISM 4 | 96, 128, 160, 192, 256, 384, 512 kbps | 96, 192, 512 kbps | Clean and 10% of frame lost | -10 dB, nominal, +10 dB |

The fixed-point decoder commit used was 6f1fdf53.

#### OMASA 1

| 24.4 kbsp up to 96 kbps, with and without bandwitdth switching, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | MLD | Seg.SNR |
| Nominal |  |  |
| + 10 dB |  |  |
| -10dB |  |  |

#### OMASA 2

| 48 kbsp up to 96 kbps, with and without bandwitdth switching, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | MLD | Seg.SNR |
| Nominal |  |  |
| + 10 dB |  |  |
| -10dB |  |  |

#### OMASA 3

| 96, 128 and 192 kbps, with and without bandwitdth switching, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | MLD | Seg.SNR |
| Nominal |  |  |
| + 10 dB |  |  |
| -10dB |  |  |

#### OMASA 4

| 96, 128, 160, 192, 256, 384 and 512 kbps, with and without bandwitdth switching, clean and 10 % frame lost |  |  |
| --- | --- | --- |
| Level | MLD | Seg.SNR |
| Nominal |  |  |
| + 10 dB |  |  |
| -10dB |  |  |

#### OMASA LTV

A further objective evaluation of OMASA was performed by Nokia to evaluate the performance of OMASA using the long test vectors used in the automated testing of the BASOP conversion, namely ltv48_OMASA*.wav/.met variants available in [16]. These inputs contain 1-4 ISMs and 1-2 transport channels for MASA. The input has been specifically designed to test different paths of the OMASA codec.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | Binaural, Mono, Stereo, 7.1+4 |
| Bitrates | 13.2 – 512 (all IVAS bitrates) |
| Bitrate switching | All bitrates |
| FER | Clean and 10% (ep_10pct.g192) |
| DTX | Off (not supported) |
| Bandwidth | FB |
| Output FS | 48 |
| Transport channels | 1 TC and 2 TC |
| Number of directions | Switching 1dir/2dir in one file |
| Number of ISM | 1 – 4 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate, FER. A total of 688 cases are presented below in the histogram figures. There were also combinations that did not process successfully, and these have been or will be inspected further. According Gitlab issues are filed for these.

As seen in the figures, many cases have low MLD scores but there are also cases with rather high values. Moreover, these values do not include JBM test cases which, in general, have more issues via crashes.

### Subjective evaluation

Subjective evaluation of the OMASA format was performed on the fixed-point decoder commit with SHA 46f1fdf5. Test material contained noisy Finnish speech and some musical OMASA scenes. Some background noises contained music signals. Background MASA signals originated from FoA, HoA2 and Eigenmike sources. Both MASA 1Dir and 2Dir material was included to the listening tests. All MASA background (sound bed) material were true spatial audio recordings. Regarding number of objects all 1, 2, 3, and 4 ISM cases were covered. All had four different samples. Object paths included static and moving objects. Some of the objects were over-lapping each other. For some samples all objects were active all the time. Overall it can be said that the test material was extremely difficult. Listening was conducted with binaural rendering using Sennheiser HD650 headphones without head-tracking. DCR test methodology was used. Majority of the listeners were experts. No processing problems were found with the listening test material.

No significant differences between floating-floating and floating-fixed versions were found.

### Issues identified

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1033 | OMASA-4 at 192 kbps exhibits discontinuity | Audio quality | Decoder, OMASA | Minor | Open |
| 1032 | OMASA-4 at 512 kbps shows glitches during bandwidth switching | Audio quality | Decoder, OMASA | Major | Ongoing |
| 1004 | Modulation artifact in OMASA at 24 kbps using LTV 3ISM 2TC input and output to MONO | Audio quality | Decoder, OMASA | Major | Open |
| 992 | Crash in ivas_dirac_dec_render_fx() in OMASA JBM decoding | Crash | Decorer, OMASA, rendering | Blocker | Open (crash fixed, under verification) |
| 991 | Crash in stereo_dft_dec_fx() for OMASA JBM rateswitch | Crash | Decoder, OMASA, JBM | Blocker | Open |
| 990 | Crash in acelp_core_dec_ivas_fx() for OMASA JBM rateswitch pt2 | Crash | Decoder, OMASA, JBM | Blocker | Open |
| 989 | Crash in acelp_core_dec_ivas_fx() for OMASA JBM rateswitch | Crash | Decoder, OMASA, JBM | Blocker | Open |
| 968 | Decoder crash for OMASA ISM4_1TC / ISM4_2TC with bitrate switching and FER in enhancer_ivas_fx2() | Crash | Decoder, OMASA | Blocker | Open |
| 958 | Assert error in stereo DFT decoding of CPE in OMASA rate switching | Assert | Decoder, OMASA, Core | Blocker | Open |

## Verification of JBM time-scaling behavior

Although MLD values have been reported for JBM test cases in the above subsections, these do not give a full overview of the performance of the JBM, as MLD is calculated for time-aligned portions of the test and reference signals. In order to verify that the fixed-point decoder applies time-scaling similarly to the floating-point reference, JBM trace files written by the decoder executables have been analyzed.

### Test conditions

Git revision used for testing: SHA 8a35289e

One test condition was generated for a unique valid combination of the following parameters, with the exception of JBM error profiles, where one profile was assigned at random to one combination of input file and bitrate, to limit the total number of test cases.

| Parameter | Number of configurations | Configurations |
| --- | --- | --- |
| Input audio file | ISM: 64 MC: 128, OSBA: 177, STEREO: 171 | Inputs vectors from the selection experiments P800-1, P800-2, P800-3, BS.1534-1a, BS.1534-1b |
| Input sampling rate | 1 | 48 kHz |
| Bit rate | ISM: 2 MC: 10 OSBA: 7 STEREO: 12 | ISM: 24.4 - 32 kbps MC: 48 - 512 kbps OSBA: 13.2 - 512 kbps STEREO: 13.2 - 256 kbps |
| DTX | ISM: 2 MC: 1 OSBA: 1 STEREO: 2 | ISM: on, off MC: off OSBA: off STEREO: on, off |
| Channel conditions | 12 | JBM dly_error_profile_{0..11}.dat |
| Output format | 6 | 7_1_4, BINAURAL, EXT, HOA3, MONO, STEREO |
| Output sampling rate | 1 | 48 kHz |

Only a subset of IVAS formats was used in testing due to the availability of test material, however this subset can be considered representative of other modes of operation. A wide range of properties relevant to the performance of the IVAS JBM were covered, including all possible transport channel counts, transport channel domains and parametric modes.

Each test condition consisted of a chain of executables, ending with the IVAS decoder running in VoIP mode. For each test condition, two variants were executed: CUT - using the FX decoder and REF - using the FL decoder.

The decoder stage in each case generated a JBM trace file, which lists the following properties for each frame of output audio:

- RTP sequence number*

- RTP timestamp (rtp_ts)*

- Receive time*

- Play time (play_time)

- Active speech/DTX

*(set to -1 for PLC (lost) frames, where these values are unknown due to frame not being received)

### Definitions of tested metrics

For each CUT/REF pair of trace files the following metrics were deemed useful in assessing the performance of CUT with respect to REF:

#### Maximum absolute play delay difference

Computed as follows:

- Play delay is calculated for each frame in CUT/REF trace:

play_delay = play_time - rtp_ts

Play delay of lost frames is unknown and is therefore excluded from calculations.

- Absolute play delay difference is calculated at each point in time where a CUT or REF frame is played:

abs_diff_play_delay = abs(cut_play_delay - ref_play_delay)

- Maximum absolute play delay difference (max_abs_diff_play_delay) is taken as the maximum of abs_diff_play_delay over the common play_time axis.

#### Difference in the number of scaled frames

Computed as follows:

- Number of scaled frames (num_scaled) is counted for CUT and REF trace.

- Final metric is calculated:

diff_num_scaled = cut_num_scaled - ref_num_scaled

### Results

The following sections discuss test results for the defined metrics.

#### Maximum absolute play delay difference

Figure 51 Histogram of the maximum absolute play delay difference in all tested conditions

In an overwhelming majority of test conditions, play delay of the fixed-point decoder remains within a tolerable range from that of the floating-point decoder. Some test conditions with large differences (>100 ms as a rule of thumb) were present for all tested IVAS formats with delay profile 6, which features very high (>400 ms) initial delay. These test cases were further inspected as signs of potential issues.

Plotted in Figure 52 and Figure 53 below are two examples of extreme outliers observed with delay profile 6, where fixed-point is plotted in blue and floating-point in orange. Maximum absolute play delay difference is the maximum vertical distance between the two play delay curves. Quality of performance can be judged by how soon play delay is decreased after a delay spike.

The key observation from the two plots is that, depending on the test case, either the floating- or the fixed-point implementation performs worse. In total, out of 1617 test cases with delay profile 6, there were 270 where the play delay of the fixed-point decoder was over 100 ms larger that of the floating-point decoder and only 16 test cases where the inverse was true. Based on this asymmetry, it can be concluded that in the presence of extreme delay peaks, the fixed-point JBM sometimes tends to perform significantly worse than the floating-point implementation. These edge cases require further inquiry.

Figure 52 Example test case where fixed-point JBM performs worse than floating-point

Figure 53 Example test case where fixed-point JBM performs better than floating-point

#### Difference in the number of scaled frames

Figure 54 Histogram of the difference in the number of scaled frames in all tested conditions

In most test cases, the number of scaled frames is very similar between the fixed- and floating-point decoders, suggesting no critical problems in the fixed-point implementation. Many outliers on this metric are the same as in the case of maximum absolute play delay difference, since the two metrics are correlated. The slight tendency of the fixed-point implementation to do less scaling is also visible.

In some extreme cases, the fixed-point implementation scales no frames where the floating-point implementation applies scaling to many frames in a short time span, as shown in Figure 55. No test cases were observed where the inverse would be true.

Extreme outliers require further examination.

Figure 55 Example test case where fixed-point JBM scales no frames despite high play delay

## External renderer

    - Evaluation

No formal evaluation of the external renderer (application IVAS_rend) was performed as part of this verification process. However, informal evaluation was performed, and some issues were noted in the external renderer that have not been caught by the automated test system. It is possible that other such issues are present and further evaluation of the non-covered operation modes is recommended.

Section 4.1 presents MLD and minimum SSNR scores for the external renderer based on the automated daily CI pipeline. This shows that for many operation points, then MLD scores are low and the minimum SSNR scores are high. This is expected as the input is full quality. However, there are some cases that seem to be outliers (higher MLD or lower minimum SSNR) and should be further studied.

    - Issues identified

| Issue # | Summary | Type | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1012 | IVAS_rend OMASA to MASA2 rendering crashes | Crash | Renderer, MASA | Blocker | Open |
| 1011 | IVAS_rend OMASA BINAURAL rendering has very low gain for ISM | Audio quality | Renderer, MASA, ISM | Critical | Open |

# Complexity verification of IVAS Decoder and Renderer

The complexity of the current development versions of IVAS Float and IVAS BASOP, daily measurements are run. This was obtained for the BASOP main version a41b242295d670affc6091ff69e90655084886c8 (Mon Nov 4 15:49:53 2024 +0000) and ivas-float-update-cmplx version de702f5cf01e71b2c1dbfaadd5b2bd3a94670fb5 (Wed Nov 6 21:02:58 2024 +0100). The total number of test cases is 3532, out of which 18 cases triggered a crash in the BASOP decoder and 192 cases triggered a crash in the float reference. The ratio of the WMOPS for each operating point is shown in Figure 44, with the average complexity ratio is 1.95 and a standard deviation of 0.83. The average ratio is 2.57 times the target complexity ratio 0.76 of EVS.

Figure 56: Histogram of IVAS BASOP to FLOAT complexity ratios

In Figure 57 - Figure 74 below, the highest WMOPS are shown for three bit rate ranges:

- bit rate <= 80 kbps

- 80 kbps < bit rate <= 192 kbps

- 192 kbps < bit rate <= 512 kbps

For the stereo down-mix operation in Figure 57, the decoder does not operate above 128 kbps and there are no cases in the last bitrate range. For the OSBA to binaural cases in Figure 69 and Figure 70 the high bit rate cases are also left out, since the floating-point reference crashed for these cases.

Figure 57: WMOPS complexity of BASOP vs FLOAT for Stereo Downmix EVS in, mono out.

Figure 58: WMOPS complexity of BASOP vs FLOAT for ISM in, binaural out.

Figure 59: WMOPS complexity of BASOP vs FLOAT for ISM in, binaural room IR out.

Figure 60: WMOPS complexity of BASOP vs FLOAT for ISM in, EXT out.

Figure 61: WMOPS complexity of BASOP vs FLOAT for MASA in, binaural out.

Figure 62: WMOPS complexity of BASOP vs FLOAT for MASA in, EXT out.

Figure 63: WMOPS complexity of BASOP vs FLOAT for MASA in, HOA3 out.

Figure 64: WMOPS complexity of BASOP vs FLOAT for MC in, 7.1-4 out.

Figure 65: WMOPS complexity of BASOP vs FLOAT for MC in, binaural out.

Figure 66: WMOPS complexity of BASOP vs FLOAT for MC in, binaural room IR out.

Figure 67: WMOPS complexity of BASOP vs FLOAT for OMASA in, binaural out.

Figure 68: WMOPS complexity of BASOP vs FLOAT for OMASA in, HOA3 out.

Figure 69: WMOPS complexity of BASOP vs FLOAT for OSBA in, binaural out.

Figure 70: WMOPS complexity of BASOP vs FLOAT for OSBA in, Binaural room IR out.

Figure 71: WMOPS complexity of BASOP vs FLOAT for SBA HOA3 in, Binaural out.

Figure 72: WMOPS complexity of BASOP vs FLOAT for SBA HOA3 in, Binaural room IR out.

Figure 73: WMOPS complexity of BASOP vs FLOAT for SBA HOA3 in, HOA3 out.

Figure 74: WMOPS complexity of BASOP vs FLOAT for Stereo in, Stereo out.

# Memory verification of IVAS Decoder and Renderer

Memory consumption is measured daily for both the IVAS FLOAT and BASOP code bases. It is divided into RAM and ROM usage. RAM consumption is further broken down into HEAP (dynamically allocated memory) and STACK. The total RAM usage at each operating point of the codec is calculated by summing the encoder and decoder HEAP memory and adding the maximum value between the encoder STACK and decoder STACK.

For ROM, memory consumption is separated into PROM (Program ROM) and TROM (global static variables and constants). The total PROM consumption remains constant across all operating points and is calculated by summing the PROM usage for the encoder, decoder, renderer, and shared components. Similarly, total TROM consumption remains consistent for each operating point and is the sum of the encoder, decoder, renderer, and common TROM.

In this report, only decoder and renderer modules are considered. Measurements were taken for 3,532 operating points, out of which 719 cases (OSBA) were excluded due to processing errors. The average memory consumption for the decoder is presented in Figure 75.

Figure 75: IVAS BASOP to FLOAT Average Decoder Memory Consumption Ratios

The total PROM memory ratio for the decoder is 0.73, while for the renderer, it is 0.67. The total TROM memory ratio for the decoder is 0.92. The average HEAP memory consumption for the decoder stands at 1.09, and the average STACK memory consumption is 1.86.

A detailed breakdown of memory consumption for each input format is provided in Figure 76 to Figure 81.

Figure 76: IVAS BASOP to FLOAT STEREO Decoder Memory Consumption Ratios

Figure 77: IVAS BASOP to FLOAT ISM+N Decoder Memory Consumption Ratios

Figure 78: IVAS BASOP to FLOAT HOA3 Decoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 79: IVAS BASOP to FLOAT Average MASA Decoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 80: IVAS BASOP to FLOAT Average MC Decoder Memory Consumption Ratios (sorted by input format and bitrate)

Figure 81: IVAS BASOP to FLOAT Average OMASA Decoder Memory Consumption Ratios (sorted by output format and bitrate)

# Summary of identified issues

This is a list of the main problematic areas to be addressed.

| Issue category | Description |
| --- | --- |
| Complexity and memory | The IVAS BASOP codec is, on average, 1.85 times more complex than the IVAS FLOAT codec, whereas the target ratio, based on the EVS codec, is 0.76. In the worst-case scenario, the IVAS BASOP codec is 4.88 times more complex than the IVAS FLOAT codec. The corresponding number for the EVS BASOP vs EVS FLOAT is 0.90. The average decoder HEAP memory consumption of the IVAS BASOP codec is 1.09 times higher compared to the IVAS FLOAT codec, while the target value derived from the EVS codec is 0.58. Similarly, the maximum decoder STACK memory consumption of the IVAS BASOP codec is 1.86 times higher than that of the IVAS FLOAT codec, with the target value from the EVS codec being 0.73. |
| Packet loss concealment | Significant quality issues for packet loss concealment have been observed (clicks, discontinuities), e.g. #949, #954, #977, #978, #1000, #1008, #1014, #1021, #1022, #1028, #1031, #1039 |
| High noise floor compared to float | Issue present even at 512 kbps. See #951, #1015. |
| Saturation | Several operating modes show audible distortion with high-level input signals. See e.g. Figure 17 and the daily reports in [6]. |
| Crashes | Increased number of crashes were found during verification, especially for high-level input. The codec should be robust also for these inputs. Related to high-level input. |
| Regressions | For a few cases regressions were noticed during the verification process, e.g. for packet loss concealment, where new artifacts were appearing as fixes were brought in. |
| Test coverage | For non-mainstream operation, more issues have been found, e.g. rate-switching, bandwidth switching, other input bandwidths (not 48 kHz). |
| High MLD issues | - Some test cases were observed to give high MLDs, indicating audible artifacts which were later confirmed by manual inspection of the signals. - OSBA at low bitrates should decode near-identically to SBA if the content is equivalent. However, higher MLD in OSBA decoder is observed. |
| Degradations found in listening tests | - Stereo at 48kbps with FER and DTX, significant degradation for one item and degradation for overall score (see 6.1.3.2) - OSBA (HOA3 with ISM4) at 320kbps with FER, statistical significant degradation for one item and overall score (see 6.10.2.1.2) |
| Low SSNR issues | Very low SSNR cases observed for the following operating points: - MC 7.1+4 @128kbps (ParamMC) - range 26-30dB (see 6.6.2.1) - ISM 3,4 @32,48 kbps (ParamISM) – range 23 – 30dB (see 6.7.2.1) - OSBA (HOA3, HOA2, FOA) – range 12-30dB (see 6.10.2.1.1, 6.10.2.2.1, 6.10.2.3.1) |

# Summary

A verification of the delivered IVAS BASOP Decoder and Renderer has been done, including all supported formats. Considering the daily run tests presented in 4.1, the percentage of test cases below MLD 5 is 86%. The threshold of SNR > 60 dB proved not to be so useful, since EVS has 0% of the test cases above this limit. The corresponding figure for the IVAS BASOP Decoder and Renderer is 38% of the test cases above 60 dB, mainly due to high SSNR scores in the outputs of the Renderer. The computational complexity is on average 2.57 times the target ratio of EVS BASOP vs EVS FLOAT.

# References

- TR 26.258 Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)

- SA4aA240038, Organization of IVAS Characterization Phase rev 1, https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_Audio/Docs/SA4aA240038.zip

- SA4aA240056, On the status of the BASOP conversion, https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_Audio/Docs/S4aA240056.zip

- IVAS_BASOP_Release_Notes_31082024.pdf, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/releases/20240831_Ittiam_fixed_point_decoder_renderer_release (also attached)

- Verification time plan, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/wikis/Verification-time-plan

- IVAS BASOP repository on 3GPP Forge, https://forge.3gpp.org/rep/sa4/audio/ivas-basop

- Verification assignment table, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/wikis/Verification-Assignment-table

- MLD Tool, https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/TSGS4_102_Bruges/Docs/S4-190114.zip

- ITU-T WMC Tool, https://github.com/openitu/STL/tree/dev/src/wmc_tool

- Performance Characterization report TR 26.952, https://www.etsi.org/deliver/etsi_tr/126900_126999/126952/12.05.00_60/tr_126952v120500p.pdf

- Measure EVS FX/FL with MLD, SSNR and MAX ABS DIFF, 3GPP Forge BASOP issue #891, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/891

- ITU-R BS.1534, “Method for the subjective assessment of intermediate quality level of audio systems, 2015

- ITU-R BS.1170, “Algorithms to measure audio programme loudness and true-peak audio level”, 2015

- 3GPP IVAS Processing Scripts repository, https://forge.3gpp.org/rep/ivas-codec-pc/ivas-processing-scripts