3GPP TSG-SA5 Meeting #162	S5-253497

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1583 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.541 Add solution to construct conditions on performance metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2, CCLM |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently it is not possible to construct conditions based on performance metrics. This requirement is discussed in MADCOL_ph2, see current Editor’s Note in TS 28.622 clause 4.3.47.1 as well as in FS_CCLM, see TR 28.867 clause 5.2 Triggered CCL. In TR 28.867 clause 5.2.3.4 the solution of pseudo attribute was evaluated and recommended for normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add pseudo attributes for GNBCUCPFunction IOC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Pseudo attributes clause for GNBCUCPFunction IOC is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.5 (new), 4.3.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of the package. It has dependency on TS 32.156 CR0117 and TS 32.160 CR 0078. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

#### 4.3.2.5	State diagram

None.

#### 4.3.2.X	Pseudo Attributes

| Name(s) | Definition |
| --- | --- |
| avgPrepTimeForInterSNCPC | Average Time Interval for Preparation of the SN initiated inter-SN CPC as defined in TS 28.552[X] clause 5.1.1.6.10 |
| pAG.ReceivedNbrCnInitiated | Number of CN Initiated paging records received by the gNB-CU as defined in TS 28.552[X] clause 5.1.1.27.1 |
| pAG.ReceivedNbrRanIntiated | Number of NG-RAN Initiated paging records received by the gNB-CU as defined in TS 28.552[X] clause 5.1.1.27.2 |
| pAG.DiscardedNbrCnInitiated | Number of CN Initiated paging records discarded at the gNB-CU as defined in TS 28.552[X] clause 5.1.1.27.4 |
| pAG.DiscardedNbrRanInitiated | Number of NG-RAN Initiated paging records discarded at the gNB-CU as defined in TS 28.552[X] clause 5.1.1.27.5 |
| pAG.SentNbrRanIntiated | Number of NG-RAN Initiated paging records sent by the gNB-CU as defined in TS 28.552[X] clause 5.1.1.27.7 |
| pAG.SentNbrRanIntiatedSucc | Number of successful NG-RAN Initiated paging sent by the gNB-CU as defined in TS 28.552[X] clause 5.1.1.27.8 |
| pAG.PagingLoadInstantCore.Bin | Distribution of Instantaneous Paging Load for Core Initiated Page in NTN Deployments as defined in TS 28.552[X] clause 5.1.1.27.9 |
| pAG.PagingLoadInstantRAN.Bin | Distribution of Instantaneous Paging Load for RAN Initiated Page in NTN Deployments as defined in TS 28.552[X] clause 5.1.1.27.10 |
| oEU.CellInServiceTotal.NCGI | Total cell In-Service duration as defined in TS 28.552[X] clause 5.1.4.1.1 |
| oEU.CellOOSTimeTotal.NCGI | Total cell outage duration as defined in TS 28.552[X] clause 5.1.4.1.2 |
| oEU.CellOOSTimeAverage.NCGI | Average cell outage duration as defined in TS 28.552[X] clause 5.1.4.1.3 |
| vR.VCpuUsageMean | Mean virtual CPU usage as defined in TS 28.552[X] clause 5.7.1.1.1 |
| vR.VMemoryUsageMean | Mean virtual memory usage as defined in TS 28.552[X] clause 5.7.1.2.1 |
| vR.VDiskUsageMean | Mean virtual disk usage as defined in TS 28.552[X] clause 5.7.1.3.1 |

End of changes