3GPP TSG-SA5 Meeting #164	S5-255108

Dallas, Texas, USA, 17 – 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Dynamic CCL for resource optimization

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TS28.889

Version:	0.1.0

Work Item:	Closed Control Loop Management phase 2

Comments

This pCR is to add use case and requirements for CCL for network slice resource optimization

- WT-3: Study the need for CCL enhancements enabling automated, efficient network management.

Proposed Changes

* * * First Change * * * *

# 4. 			Use Cases

### 4.X	Dynamic CCL for resource optimization

4.X.1	Description

A CCL may require different kinds of input to address its requirements. 3GPP TS 28.104 [X], specified MDA with reports on different kinds of analysis of network issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands..

A closed control loop for network slice resource optimization can be created using MDA analytics. The management system should enable the MnS consumer to compose the network slice resource optimization closed control loop.

Note: relation of this use case with the assurance closed control if FFS.

#### 5.X.2	Potential requirements

REQ- CCL-ES-1: The management system should enable the MnS consumer to compose a network slice resource optimization closed control loop using MDA capabilities.

### 5.x.3	Possible solutions

This solution proposes to enhance the existing CCL information models defined in 3GPP TS 28.567 [2].

To support the composition of a CCL on network slicing resource optimization, the following can be introduced:

-	an IOC for network slice resource optimization closed control loop.

-	The network slice resource optimization closed control loop inherits the capabilities of the closed control loop and the assurance closed control loop.

-	attributes on the network slice resource optimization CCL for analysis insight and decision recommendation on resource optimization components. The attributes can be configured by the MnS consumer to point to specific analytics outputs.

5.X.4			Evaluation of solutions

.

* * * End of Changes * * * *