3GPP TSG-SA5 Meeting #164	S5-255113

Dallas, Texas, USA, 17 – 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Assisting intent decomposition

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR28.881

Version:	0.2.0

Work Item:	Intent driven management services for mobile network phase 4

Comments

TS 28.312 discussed the idea of intent decomposition, where an intent is decomposed into multiple derivative intents. However, the MnS consumer might possess some knowledge, which is necessary to fulfil the main intent and subsequent derivative intents. This knowledge needs to be conveyed to all subsequent intent handlers, who are going to handle all the subsequent derivative intents. There should be a support/enablement for the MnS consumer to do so.

WT-2	New intent driven management scenarios:

WT-2.2	Investigate what intent handling functionality is needed to better support deployment scenario#2 defined in TS 28.312. (e.g. intent decomposition).

[1] 	3GPP TR 28.881 “Study on intent driven management services for mobile network phase 4”.

Proposed Changes

* * * First Change * * * *

## 4.3 Use case #3:  Assisting and reporting intent decomposition across intent handling functions

### 4.3.1	Description

An intent-driven MnS producer can translate an intent and decompose it into one or more new intents. If these newly generated intent(s) cannot be handled by the intent-driven MnS producer, it can submit them to another intent-driven MnS producer(s). In this case, the former intent-driven MnS producer acts as the consumer of the latter intent driven MnS producer(s), to which the newly generated intents are submitted.

The issue arises when the decomposition of an intent into new intents is carried out by an intent driven MnS producer, these decomposed intents may be submitted to intent-driven MnS producers of managed domains that are not desired by the original intent-driven MnS consumer. The intent-driven MnS consumer needs to be able to assist the intent driven MnS producer regarding the decomposition of intents to other MnS producers. Furthermore, the intent driven MnS consumer needs to receive a report regarding this decomposition. This is also important for troubleshooting an intent-driven MnS producer, to check whether it is acting as expected.

The decomposition may be to other management functions such as digital twins and closed control loops that are requested to accomplish specific aspects of the intent. Where such decomposition is undertaken, the intent handler's report should indicate the derivative intentsthat are decomposed form the intent.

The intent may be decomposed into requirements or goals to be fulfilled by automation functionality such as closed control loops or automation functions. The goals express specific targets to be fulfilled and priorities among those targets. The intent handler's report should indicate the goals desired to be fulfilled by the automation functionality.

The intent handler may need to apply specific ml capabilities to fulfil an intent, i.e., the intent may be mapped and decomposed into a set of required ML capabilities. The intent handler's report should indicate the set of ML capabilities that are required to fulfil an intent.

To assist intent decomposition, an operator may wish to provide a logic for intent decomposition, e.g., the logic as a set of rules and sequence of execution of those rules. The intent handler may combine can then combine that logic with its existing logic to decide how to decompose or fulfil the intent. The Mns consumer should be  enabled to provide assistance logic for intent handling.

### 4.3.2	Potential requirements

REQ-Intent_DCMP-1: The intent driven MnS producer should have the capability to allow an MnS consumer to specify a context to be able to assist the intent-driven MnS producer regarding the decomposition of an intent to other MnS producers.

REQ-Intent_DCMP-2: The intent driven MnS producer should have the capability to allow an intent-driven MnS consumer to receive a report regarding the decomposition of an intent to other MnS producers.

REQ-Intent_DCMP-3: The intent driven MnS producer should have the capability to provide a report indicating non-intent-handler entities to which an intent is decomposed.

REQ-Intent_DCMP-4: The intent driven MnS producer should have the capability to provide a report indicating goals desired to be fulfilled by automation functionality such as closed control loops or SON functions.

REQ-Intent_DCMP-5: The intent driven MnS producer should have the capability to provide a report indicating the set of ML capabilities that are required to fulfil the intent.

REQ-Intent_DCMP-6: The intent driven MnS producer should have the capability enabling an MnS consumer to provide assistance logic for intent handling.

Note: the nature of assistance logic for intent handling is FFS

### 4.3.3	Potential solutions

This solution proposes to reuse and enhance the existing intent expectations and intent report information models defined in 3GPP TS 28.312 [1].

Intent expectations are enhanced with:

- A new expectation context that specifies the constraints applicable in the intent decomposition performed by the intent-diven MnS producer, such as a list of Intent Handling Functions identity among those under MnS consumer’s authorization to which the intent decomposition is not recommended by the intent-driven MnS consumer.

- assistance logic for intent handling

The IntentReport IOC in 3GPP TS 28.312 [1] is enhanced with following information:

- The identity of Intent-Handling Functions and non-Intent-Handling Functions to which the decomposed intents are sent.

- The identity of each intent and expectation resulting from the decomposition.

- The goals requested to be fulfilled by non-Intent-Handling Functions

- ML capabilities that are required to fulfil the intent

### 4.3.4	Evaluation of potential solutions

TBD

* * * End of Changes * * * *