3GPP TSG-SA5 Meeting #164	S5-255145

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1643 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.541 YANG stage-3 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1983 at commit 4d4a190705a0e4870d80fde65936be3f4e79a32c |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1983 at commit 4d4a190705a0e4870d80fde65936be3f4e79a32c

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-5g-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines common types for 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-07 { reference CR-1640 ; }

revision 2025-07-25 { reference CR-1558 ; }

revision 2025-03-25 { reference CR-1489 ; }

revision 2024-11-01 { reference CR-1405; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-05-10 { reference CR-0916; }

revision 2021-08-05 { reference S5-214053/CR-0518; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 { reference "Initial version."; }

grouping AreaScopeGrp {

description "This <<dataType>> defines an area scope.";

choice AreaScopeChoice {

case eutran-only { // choice 1

leaf-list eutraCellIdList {

type types3gpp:EutraCellId;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

}

case tac { // choice 3

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

leaf-list cAGIdList {

type types3gpp:CagId;

max-elements 256;

description "It identifies a CAG list containing up to

256 CAG-identifiers per UE or up to 12 CAG-identifiers

per cell, see TS 38.331 [38]. CAG ID is used to combine

with PLMN ID to identify a PNI-NPN.AG ID is a hexadecimal

range with size 32 bit.";

}

}

case ntnGeoAreaList{ // choice 6

list ntnGeoAreaList {

must 'not(../nrCellIdList)';

description "geographical areas for NTN MDT";

key idx;

max-elements 8;

leaf idx { type string; }

uses types3gpp:GeoAreaGrp;

}

}

case else {

list nPNIdentityList { // choice 2, 4 , 5

description "list of NPN IDs of in NR. It is either

a list of PNI-NPNs identified by CAG ID with

associated plmn-Identity or a list of SNPN

identified by Network ID with associated plmn-Identity";

key idx;

min-elements 1;

uses types3gpp:NpnIdGrp;

leaf idx { type string; }

}

leaf-list nrCellIdList {

must 'not(../taiList)';

type types3gpp:NrCellId;

max-elements 32;

description "List of NR cells identified by NG-RAN CGI";

}

list taiList {

must 'not(../nrCellIdList)';

description "Tracking Area Identity list";

key idx;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

}

list sliceIdList {

description "Network Slice Id list";

key idx;

uses PLMNInfo;

min-elements 1;

max-elements 16384;

leaf idx { type string; }

}

}

grouping IpInterfaceGrp {

leaf-list ipv4EndpointAddresses {

description "Available endpoint IPv4 address(es) of

the User Plane interface.";

type inet:ipv4-address;

must '../ipv4EndpointAddresses or ../ipv6EndpointAddresses or ../fqdn';

}

leaf-list ipv6EndpointAddresses {

description "Available endpoint IPv6 address(es) of

the User Plane interface.";

type inet:ipv6-address;

must '../ipv4EndpointAddresses or ../ipv6EndpointAddresses or ../fqdn';

}

leaf fqdn {

description "This parameter defines FQDN of the Network Function

(See TS 23.003).";

type inet:domain-name;

must '../ipv4EndpointAddresses or ../ipv6EndpointAddresses or ../fqdn';

}

}

grouping SdRangeGrp {

leaf start {

type string {

pattern "[A-Fa-f0-9]{6}";

}

mandatory true;

description "First value identifying the start of an SD range.

This string shall be formatted as specified for the sd attribute of the

Snssai data type in clause 5.4.4.2 of TS 29.571.";

}

leaf end {

type string {

pattern "[A-Fa-f0-9]{6}";

}

mandatory true;

description "Last value identifying the end of an SD range.

This string shall be formatted as specified for the sd attribute of the

Snssai data type in clause 5.4.4.2 in TS 29.571";

}

}

grouping SnssaiExtensionGrp {

list sdRanges {

min-elements 1;

key "start end";

description "It shall contain the range(s) of Slice Differentiator values

supported for the Slice/Service Type value indicated in the sst

attribute of the Snssai data type (see clause 5.4.4.2 in TS 29.571).";

uses SdRangeGrp;

}

leaf wildcardSd {

type boolean;

default false;

description "It indicates that all SD values are supported for the

Slice/Service Type value indicated in the sst attribute of the Snssai

data type (see clause 5.4.4.2 in TS 29.571).";

}

}

grouping ExtSnssaiGrp {

list snssai {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003 ";

min-elements 1;

max-elements 1;

key "sd sst";

uses SNssai;

}

list snssaiExtension {

description "It represents extensions to the Snssai.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses SnssaiExtensionGrp;

}

}

grouping sNssaiSmfInfoItem {

list sNssai {

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses SNssai;

}

list dnnSmfInfoList {

description "List of parameters supported by the SMF per DNN.

The absence indicates the DNN can be selected for any DNAI.";

min-elements 1;

key dnn;

uses DnnSmfInfoItem;

}

}

grouping MappedCellIdInfoGrp {

description "This data type represents the mapping relationship between

Mapped Cell IDs and geographical areas (see clause 16.14.5 of TS 38.300";

list ntnGeoArea {

description "This attribute indicates a specific geographical location

mapped to Mapped Cell ID(s).";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

uses types3gpp:GeoAreaGrp;

}

list mappedCellId  {

description "This attribute is in format of NCGI to indicate a fixed

geographical area (See subclause 16.14.5 in TS 38.300)";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

uses NcgiGrp;

}

}

grouping NcgiGrp {

description "Represents the Ncgi datatype";

list plmnId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf nrCellId {

type string;

mandatory true;

description "This attribute represents NR Cell Identity.

It's a 36-bit string identifying an NR Cell Id as specified in

clause 9.3.1.7 of TS 38.413, in hexadecimal representation. Each

character in the string shall take a value of

'0' to '9', 'a' to 'f' or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most significant

bits of the Cell Id shall appear first in the string, and the

character representing the 4 least significant bit of the

Cell Id shall appear last in the string.

Pattern: '^[A-Fa-f0-9]{9}$'

Example:

An NR Cell Id 0x225BD6007 shall be encoded as '225BD6007'.";

}

leaf nId {

type string;

mandatory true;

description "Network Identity; Shall be present if PlmnIdNid identifies

an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in

3GPP TS 23.501.";

}

}

typedef NRTAC {

type string;

description "This holds the identity of the common Tracking Area Code

for the PLMNs.

allowedValues:

a) It is the TAC or Extended-TAC.

b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300,

subclause 10.1.7 (PLMNID and TAC relation).

c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003

and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473.

d) For a 5G SA (Stand Alone), it has a non-null value.";

}

grouping SNssai {

description

"Single Network Slice Selection Assistance Information(S-NSSAI)";

reference "3GPP TS 23.003";

leaf sd {

description "Slice Differentiator

If not needed, the value can be set to ff:ff:ff.";

type yang:hex-string {

length 8;

}

reference "3GPP TS 23.003";

}

leaf sst {

type uint8;

description "Slice/Service Type.

Values 0 to 127 belong to standardized SST range and are defined in

3GPP TS 23.501. Values 128 to 255 belong to operator-specific range.";

}

}

grouping PlmnIdNid {

description "Represents the SCP domain specific information as defined

in TS 29.510 ";

uses types3gpp:PLMNId;

leaf nid {

type string;

description "This attribute represents network Identity;

Shall be present if PlmnIdNid identifies an SNPN.

(see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in TS 23.501";

}

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific

PLMNId, and it have two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI).

The PLMNId represents a data type that is comprised of mcc

(mobile country code) and mnc (mobile network code), (See TS 23.003

subclause 2.2 and 12.1) and S-NSSAI represents an data type, that is

comprised of an SST (Slice/Service type) and an optional

SD (Slice Differentiator) field";

uses types3gpp:PLMNId;

uses SNssai;

}

typedef CommModelType {

reference "3GPP TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered

by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through

SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through SCP

as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping TaiRangeGrp {

list plmnId {

description "PLMN ID related to the TacRange.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list tacRangeList {

description "The range of the TACs.";

min-elements 1;

key "start end";

uses TacRange;

}

}

grouping TacRange {

leaf start {

description "First value identifying the start of a TAC range,

to be used when the range of TAC's can be represented

as a hexadecimal range (e.g., TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6}$)';

}

}

leaf end {

description "Last value identifying the end of a TAC range,

to be used when the range of TAC's can be represented as

a hexadecimal range (e.g. TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$';

}

}

leaf nRTACpattern {

description "Pattern (regular expression according to the ECMA-262)

representing the set of TAC's belonging to this range.

A TAC value is considered part of the range if and only if the

TAC string fully matches the regular expression.";

type string;

}

}

grouping DnnSmfInfoItem {

leaf dnn {

description "Supported DNN.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported by

the SMF for this DNN. The absence of this attribute indicates that

the SMF can be selected for this DNN for any DNAI.";

min-elements 1;

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

typedef EnergySavingLoadThresholdT {

type uint32 {

range 0..10000;

}

units 1/10000;

}

typedef EnergySavingTimeDurationT {

type uint32 {

range 0..900;

}

units seconds;

}

typedef PhysCellID {

type uint32 {

range "0..1007";

}

reference "clause 7.4.2 of TS 38.211";

}

typedef UTC24TimeOfDayT {

description "Time of day in HH:MM or H:MM 24-hour format per UTC

time zone.";

type string {

pattern "(([01]?[0-9])|(2[0-3])):([0-5][0-9])";

}

}

typedef DayOfWeekT {

type enumeration {

enum Monday;

enum Tuesday;

enum Wednesday;

enum Thursday;

enum Friday;

enum Saturday;

enum Sunday;

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** yang-models/_3gpp-5gc-nrm-aiot-ep.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-aiot-ep {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5gc-nrm-aiot-ep";

prefix "aiotep3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-5gc-nrm-amffunction { prefix amf3gpp; }

import _3gpp-5gc-nrm-nrffunction { prefix nrf3gpp; }

import _3gpp-5gc-nrm-neffunction { prefix nef3gpp; }

import _3gpp-5gc-nrm-aiot { prefix aiot3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines AIOT related endpoint Classes (IOCs).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-01 { reference CR-1640; }

grouping EP_AIOT2Grp {

description "Represents the EP_AIOT2 IOC.";

uses eprp3gpp:EP_Common;

}

grouping EP_AIOT3Grp {

description "Represents the EP_AIOT3 IOC.";

uses eprp3gpp:EP_Common;

}

grouping EP_AIOT4Grp {

description "Represents the EP_AIOT4 IOC.";

uses eprp3gpp:EP_Common;

}

grouping EP_AIOT5Grp {

description "Represents the EP_AIOT5 IOC.";

uses eprp3gpp:EP_Common;

}

grouping EP_AIOT6Grp {

description "Represents the EP_AIOT6 IOC.";

uses eprp3gpp:EP_Common;

}

grouping EP_AIOT7Grp {

description "Represents the EP_AIOT7 IOC.";

uses eprp3gpp:EP_Common;

}

grouping EP_AIOT8Grp {

description "Represents the EP_AIOT8 IOC.";

uses eprp3gpp:EP_Common;

}

feature EP_AIOT2UnderAMFFunction {

description "EP_AIOT2 shall be contained under AMFFunction.";

}

augment /me3gpp:ManagedElement/amf3gpp:AMFFunction {

if-feature EP_AIOT2UnderAMFFunction;

list EP_AIOT2 {

description "This IOC represents the AIOT2 interface between NG-RAN and

AIOTF, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT2Grp;

}

}

}

feature EP_AIOT5UnderNRFFunction {

description "EP_AIOT5 shall be contained under NRFFunction.";

}

augment /me3gpp:ManagedElement/nrf3gpp:NRFFunction {

if-feature EP_AIOT5UnderNRFFunction;

list EP_AIOT5 {

description "This IOC represents the AIOT5 interface between AIOTF and

NRF, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT5Grp;

}

}

}

feature EP_AIOT4UnderNEFFunction {

description "EP_AIOT4 shall be contained under NEFFunction.";

}

feature EP_AIOT8UnderNEFFunction {

description "EP_AIOT8 shall be contained under NEFFunction.";

}

augment /me3gpp:ManagedElement/nef3gpp:NEFFunction {

if-feature EP_AIOT4UnderNEFFunction;

list EP_AIOT4 {

description "This IOC represents the AIOT4 interface between AIOTF and

NEF, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT4Grp;

}

}

if-feature EP_AIOT8UnderNEFFunction;

list EP_AIOT8 {

description "This IOC represents the AIOT8 interface between NEF and ADM,

which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT8Grp;

}

}

}

feature EP_AIOT3UnderAIOTFFunction {

description "EP_AIOT3 shall be contained under AIOTFFunction.";

}

feature EP_AIOT4UnderAIOTFFunction {

description "EP_AIOT4 shall be contained under AIOTFFunction.";

}

feature EP_AIOT5UnderAIOTFFunction {

description "EP_AIOT5 shall be contained under AIOTFFunction.";

}

feature EP_AIOT6UnderAIOTFFunction {

description "EP_AIOT6 shall be contained under AIOTFFunction.";

}

augment /me3gpp:ManagedElement/aiot3gpp:AIOTFFunction {

if-feature EP_AIOT3UnderAIOTFFunction;

list EP_AIOT3 {

description "This IOC represents the AIOT3 interface between AMF and

AIOTF, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT3Grp;

}

}

if-feature EP_AIOT4UnderAIOTFFunction;

list EP_AIOT4 {

description "This IOC represents the AIOT4 interface between AIOTF and

NEF, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT4Grp;

}

}

if-feature EP_AIOT5UnderAIOTFFunction;

list EP_AIOT5 {

description "This IOC represents the AIOT5 interface between AIOTF and

NRF, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT5Grp;

}

}

if-feature EP_AIOT6UnderAIOTFFunction;

list EP_AIOT6 {

description "This IOC represents the AIOT6 interface between AIOTF and

ADM, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT6Grp;

}

}

}

feature EP_AIOT6UnderADMFunction {

description "EP_AIOT6 shall be contained under ADMFunction.";

}

feature EP_AIOT7UnderADMFunction {

description "EP_AIOT7 shall be contained under ADMFunction.";

}

feature EP_AIOT8UnderADMFunction {

description "EP_AIOT8 shall be contained under ADMFunction.";

}

augment /me3gpp:ManagedElement/aiot3gpp:ADMFunction {

if-feature EP_AIOT6UnderADMFunction;

list EP_AIOT6 {

description "This IOC represents the AIOT6 interface between AIOTF and

ADM, which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT6Grp;

}

}

if-feature EP_AIOT7UnderADMFunction;

list EP_AIOT7 {

description "This IOC represents the AIOT7 interface between UDR and ADM,

which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT7Grp;

}

}

if-feature EP_AIOT8UnderADMFunction;

list EP_AIOT8 {

description "This IOC represents the AIOT8 interface between NEF and ADM,

which is defined in 3GPP TS 23.369.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_AIOT8Grp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** yang-models/_3gpp-5gc-nrm-aiot.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-aiot {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5gc-nrm-aiot";

prefix "aiot3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping AIOT (Ambient power-enabled Internet

of Things).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541

3GPP TS 23.369";

revision 2025-11-01 { reference CR-1640; }

grouping ServedAIOTAreaIDGrp {

description "ServedAIOTAreaID datatype";

list pLMNId {

description "It defines which PLMN that can be served by the AIOT reader.";

min-elements 1;  // error not well defined on stage 2 clause 4.4.1

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

// error nID is only defined in 5.4.1 not 4.4.1> The text states it is not

// always needed, but the multiplicity is strictly 1.

leaf nID {

type string {

pattern '[A-Fa-f0-9]{11}';

}

description "Network Identity; Shall be present if PlmnIdNid identifies

an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in 3GPP

TS 23.501";

}

leaf aIotAreaCode {

type string;

mandatory true;

description "This specifies the identity of the A-IoT Area Code which is

one of the components of A-IoT Area ID. It's a 3-octet string defined

in TS 38.413.";

}

}

grouping ServedReaderInfoGrp {

description "ServedReaderInfo datatype";

leaf readerId {

type int32;

mandatory true;

description "It defines the reader identifier to uniquely identify a

reader within a gNB.";

}

list servedAIOTAreas {

min-elements 1;

key idx;

leaf idx { type uint32; }

uses ServedAIOTAreaIDGrp;

description "This attribute is used to specify the A-IoT areas supported

by the A-IoT reader. It contains one or multiple A-IoT Area ID, which

is used to uniquely identify an A-IoT Area.

A-IoT Area ID = PLMN ID +NID (optional) + A-IoT Area Code

(OCTET STRING (SIZE(3))), which is defined in TS 38.413.";

}

leaf readerLocation {

type string;

mandatory true;

description "This specifies the geographical location of a A-IoT reader.

Reader Location may represent any of latitude/longitude, or any

geographical location/coordinate/area polygon.";

}

}

grouping AIoTgNBInfoGrp {

description "AIoTgNBInfo datatype";

leaf gNBId {

type int64 { range "0..4294967295"; }

mandatory true;

description "It identifies a gNB within a PLMN";

}

list servedReaderInfoList {

description "Represents the information of served Readers of a gNB,

which includes the reader ID (indexes), served A-IoT areas of the RAN

and Reader and optionally the Reader location.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses ServedReaderInfoGrp;

}

}

grouping AIOTFFunctionGrp {

description "Attributes of the IOC AIOTFFunction";

uses mf3gpp:ManagedFunctionGrp;

list pLMNId {

description "Represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "It is used to indicate the FQDN of the registered NF instance

in service-based interface, for example, NF instance FQDN structure is:

nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org";

type inet:domain-name;

mandatory true;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

uses mnfp3gpp:ManagedNFProfileGrp;

description "This parameter defines profile for managed NF";

reference "3gpp TS 23.501";

}

list aIOTgNBInfo {

key idx;

leaf idx { type uint32; }

min-elements 1;

description "It represents the information that a AIOTF needs for

selecting the NG-RAN i.e.of gNB supporting Ambient-IoT service,

which includes gNB ID, served NG-RAN A-IoT area and the information

of served Readers of the gNB.";

uses AIoTgNBInfoGrp;

}

}

feature AIOTFFunctionUnderManagedElement {

description "AIOTFFunction shall be contained under ManagedElement.";

}

augment /me3gpp:ManagedElement {

if-feature AIOTFFunctionUnderManagedElement;

list AIOTFFunction {

description "This IOC represents the AIOTF function defined in TS 23.369";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AIOTFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

//////////////////////////////////////////////////////////////////////

grouping ADMFunctionGrp {

description "Attributes of the IOC ADMFunction";

uses mf3gpp:ManagedFunctionGrp;

list pLMNId {

description "Represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "It is used to indicate the FQDN of the registered NF instance

in service-based interface, for example, NF instance FQDN structure is:

nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org";

type inet:domain-name;

mandatory true;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

uses mnfp3gpp:ManagedNFProfileGrp;

description "This parameter defines profile for managed NF";

reference "3gpp TS 23.501";

}

}

feature ADMFunctionUnderManagedElement {

description "ADMFunction shall be contained under ManagedElement.";

}

augment /me3gpp:ManagedElement {

if-feature ADMFunctionUnderManagedElement;

list ADMFunction {

description "This IOC represents the ADM function defined in TS 23.369";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ADMFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 3 ***

*** START OF CHANGE 4 ***

*** yang-models/_3gpp-5gc-nrm-neffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NEF function in 5GC. For more

information about the NEF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-07 { reference CR-1640 ; }

revision 2025-07-25 { reference CR-1558 ; }

revision 2025-03-25 { reference CR-1489 ; }

revision 2025-01-25 { reference CR-1442 ; }

revision 2024-05-18 { reference CR-1273 ; }

revision 2024-04-12 { reference CR-1218 ; }

revision 2023-09-18 { reference CR-1103 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-20 { reference "initial revision"; }

grouping DnnInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

leaf dnn {

type string;

mandatory true;

description "String representing a Data Network as defined in clause 9A

of 3GPP TS 23.003; it shall contain either a DNN Network Identifier,

or a full DNN with both the Network Identifier and Operator Identifier,

as specified in 3GPP TS 23.003 clause 9.1.1 and 9.1.2. It shall be

coded as string in which the labels are separated by dots

(e.g. 'Label1.Label2.Label3').

Whether the dnn data type contains just the DNN Network Identifier, or

the Network Identifier plus the Operator Identifier, shall be

documented in each API where this data type is used.";

}

}

grouping SnssaiInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

list sNssai {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:SNssai;

}

list dnnInfoList{

description "It represents list of parameters supported by the

NF per DNN.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses DnnInfoItemGrp;

}

}

grouping AfEventExposureDataGrp {

description "This data type represents the AF Event Exposure data managed

by a given NEF Instance. (See clause 6.1.6.2.50 TS 29.510";

leaf-list afEvents {  // stage 2 double defined

type string;

min-elements 1;

description "It represents AF Event(s) exposed by the NEF after

registration of the AF(s) at the NEF.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

}

grouping PfdDataGrp {

description "This data type represents the list of Application IDs

and/or AF IDs managed by a given NEF Instance.

(See clause 6.1.6.2.49 TS 29.510)";

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

}

grouping UnTrustAfInfoGrp {

description "This data type represents information of an untrusted AF

Instance. (See clause 6.1.6.2.95 TS 29.510)";

leaf afId {

type string;

mandatory true;

description "It represents associated AF id.";

}

list sNssaiInfoList {

description "It represents S-NSSAIs and DNNs supported by the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses SnssaiInfoItemGrp;

}

leaf mappingInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute indicates whether the

AF supports mapping between UE IP address (IPv4 address or IPv6 prefix)

and UE ID (i.e. GPSI).

True: the AF supports mapping between UE IP address and UE ID;

False: the AF does not support mapping between UE IP address

and UE ID.";

}

}

grouping NefInfoGrp {

description "This data type represents information of an NEF Instance.

(See clause 6.1.6.2.48 TS 29.510).";

list taiList {

config false;

description "List of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

config false;

description "Range of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:TaiRangeGrp;

}

leaf nefId {

type string;

mandatory true;

description "It represents the NEF ID.

(see clause 6.1.6.3.2 of TS 29.510)";

}

list pfdData {

config false;

description "It represents PFD data, containing the list of internal

application identifiers and/or the list of application function

identifiers for which the PFDs can be provided.

Absence of this attribute indicates that the PFDs for any internal

application identifier and for any application function identifier

can be provided.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses PfdDataGrp;

}

list afEeData {

config false;

description "It represents the AF provided event exposure data. The NEF

registers such information in the NRF on behalf of the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses AfEventExposureDataGrp;

}

list gpsiRanges {

description "It represents list of ranges of GPSIs whose profile data

is available.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses mnfp3gpp:IdentityRange;

}

list externalGroupIdentifiersRanges {

description "It represents list of ranges of external groups whose

profile data is available.";  // stage 2 definition is shaky

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses mnfp3gpp:IdentityRange;

}

leaf-list servedFqdnList {

type string;

min-elements 1;

description "It represents pattern (regular expression according to

the ECMA-262 dialect [75]) representing the Domain names served by

the NEF.";

}

leaf-list dnaiList {

type string;

min-elements 1;

description "It represents list of Data network access identifiers

supported by the NEF. The absence of this attribute indicates that

the NEF can be selected for any DNAI.";

}

list unTrustAfInfoList {

description "It represents list of information corresponding to the AFs.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses UnTrustAfInfoGrp;

}

leaf uasNfFunctionalityInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute shall indicate whether the ,

NEF supports UAS NF functionality:

- True: UAS NF functionality is supported by the NEF.

- False (default): UAS NF functionality is not supported by the NEF";

}

leaf multiMemAfSessQosInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports

Multi-member AF session with required QoS functionality:

- TRUE: Multi-member AF session with required QoS functionality is

supported by the NEF

- FALSE (default): Multi-member AF session with required QoS

functionality is not supported by the NEF.";

}

leaf memberUESelAssistInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports member

UE selection assistance functionality:

- TRUE: member UE selection assistance functionality is supported by

the NEF

- FALSE (default): member UE selection assistance functionality is

not supported by the NEF.";

}

}

grouping AIoTNEFMappingGrp {

description "Represents the data type AIoTNEFMapping";

list targetAreaAF {

description "It represents the external target area provided by an AF

to NEF for triggering A-IoT services. It could refer to a geographical

location.";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list internalTargetArea  {

description "This is the (internal) target area mapped to external

target area. It is provided to NEF by NRF.

It could refer to any of TAC/TAI(list), PLMN or any geographical

location/coordinate/area polygon";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:AreaScopeGrp;

}

}

grouping NEFFunctionGrp {

description "Represents the NEFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

description "This parameter defines profile for managed NF(See TS 23.501)";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32;}

uses mnfp3gpp:ManagedNFProfileGrp;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "3GPP TS 23.003";

type string;

}

leaf isCAPIFSup {

type boolean;

yext3gpp:inVariant;

}

list nefInfo {

description "This attribute represents information of an NEF NF Instance.";

key idx;

leaf idx { type uint32 ; }

uses NefInfoGrp;

}

list AIoTNEFMapping {

description "It represents mapping information between external target

area (provided by AF) and (5G core internal) target area that is to

be provided to NRF";

config false;

min-elements 1;

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses AIoTNEFMappingGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 4 ***

*** START OF CHANGE 5 ***

*** yang-models/_3gpp-5gc-nrm-nrffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nrffunction;

prefix nrf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-5gc-nrm-managed-nfprofile { prefix mnfp3gpp; }

import _3gpp-5gc-nrm-udrfunction { prefix udr3gpp; }

import _3gpp-5gc-nrm-udmfunction { prefix udm3gpp; }

import _3gpp-5gc-nrm-ausffunction { prefix ausf3gpp; }

import _3gpp-5gc-nrm-amffunction { prefix amf3gpp;}

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp;}

import _3gpp-5gc-nrm-upffunction { prefix upf3gpp;}

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp;}

import _3gpp-5gc-nrm-nwdaffunction { prefix nwdaf3gpp;}

import _3gpp-5gc-nrm-gmlcfunction { prefix gmlc3gpp;}

import _3gpp-5gc-nrm-lmffunction { prefix lmf3gpp;}

import _3gpp-5gc-nrm-aiot { prefix aiot3gpp;}

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NRF function in 5GC.

For more information about the NRF, see 3GPP TS 23.501 [2].

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-01 { reference CR-1640 ; }

revision 2025-07-25 { reference CR-1558 ; }

revision 2025-01-25 { reference CR-1442 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 { reference "initial revision"; }

grouping bsfInfo {

list ipv4AddressRanges {

description "List of ranges of IPv4 addresses handled by BSF.

If not provided, the BSF can serve any IPv4 address.";

key "start end";

uses types3gpp:Ipv4AddressRange;

}

leaf-list dnnList {

description "List of DNNs handled by the BSF

If not provided, the BSF can serve any DNN.";

min-elements 1;

type string;

}

leaf-list ipDomainList {

description "List of IPv4 address domains, as described in

subclause 6.2 of 3GPP TS 29.513, handled by the BSF.

If not provided, the BSF can serve any IP domain.";

min-elements 1;

type string;

}

list ipv6PrefixRanges {

description "List of ranges of IPv6 prefixes handled by the BSF.

If not provided, the BSF can serve any IPv6 prefix.";

key "start end";

uses types3gpp:Ipv6PrefixRange;

}

}

grouping chfInfo {

list supiRangeList {

description "List of ranges of SUPIs that can be served by

the CHF instance. If not provided, the CHF can serve any SUPI.";

key "start end pattern";

min-elements 1;

uses mnfp3gpp:SupiRangeGrp;

}

list gpsiRangeList {

description "List of ranges of GPSI that can be served

by the CHF instance. If not provided, the CHF can serve any GPSI.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses mnfp3gpp:IdentityRange;

}

list plmnRangeList {

description "List of ranges of PLMNs (including the PLMN

IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf primaryChfInstance {

description "This attribute represents the NF Instance Id of the

primary CHF instance. This attribute shall be absent if the

secondaryChfInstance is present.";

type string;

}

leaf secondaryChfInstance {

description "This attribute represents the NF Instance Id of the

secondary CHF instance.This attribute shall be absent if the

primaryChfInstance is present.";

type string;

}

}

grouping pcscfInfo  {

description "This data type represents the information of a P-CSCF

NF Instance.

For more information, see clause 6.1.6.2.53 of TS 29.510.";

leaf-list accessType {

description "It provides the condition of access type of the UE when

the session AMBR shall be enforced, see TS 29.512 [60].If this

attribute is included in SmfInfo, it shall contain the access type

(3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF.If not

included, it shall be assumed the both access types are supported.";

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

min-elements 1;

max-elements 2;

}

leaf-list dnnList {

description "It represents the DNNs supported by the PCF. The DNN,

as defined in clause 9A of TS 23.003 [13], shall contain the Network

Identifier and it may additionally contain an Operator Identifier,

as specified in TS 23.003 [13] clause 9.1.1 and 9.1.2. If the Operator

Identifier is not included, the DNN is supported for all the PLMNs in

the plmnList of the NF Profile.If not provided, the PCF can serve any

DNN.";

min-elements 1;

type string;

}

leaf-list gmFqdn {

description "This attribute represents FQDN of the P-CSCF for the Gm

interface.";

max-elements 1;

type string;

}

leaf-list gmIpv4Addresses {

description "This attribute represents list of IPv4 addresses of of

the P-CSCF for the Gm interface.";

type inet:ipv4-address;

}

leaf-list gmIpv6Addresses {

description "This attribute represents list of IPv6 addresses of of the

P-CSCF for the Gm interface.";

type inet:ipv6-address;

}

leaf mwFqdn {

description "This attribute represents FQDN of the P-CSCF for the Mw

interface.";

type string;

}

leaf-list mwIpv4Addresses {

description "List of ranges of PLMNs (including the PLMN

IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

type inet:ipv4-address;

}

leaf-list mwIpv6Addresses {

description "This attribute represents list of IPv4 addresses of

the P-CSCF for the Mw interface.";

type inet:ipv6-address;

}

list servedIpv4AddressRanges {

description "List of ranges of PLMNs (including the PLMN

IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping HssInfoGrp {

description " Information of an HSS NF Instance";

leaf groupID{

description "This attribute defines the identity of the HSS group

that is served by the HSS instance.If not provided, the HSS instance

does not pertain to any HSS group.";

type string;

}

list imsiRanges {

description "This attribute defines the list of ranges of IMSIs whose

profile data is available in the HSS instance.";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses ImsiRange;

}

list imsPrivateIdentityRanges{

description "This attribute defines the list of ranges of IMS Private

Identities whose profile data is available in the HSS instance.";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses mnfp3gpp:IdentityRange;

}

list imsPublicIdentityRanges{

description "This attribute defines the list of ranges of MSISDNs whose

profile data is available in the HSS instance. ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses mnfp3gpp:IdentityRange;

}

list msisdnRanges{

description "This attribute defines the list of ranges of IMS Public

Identities whose profile data is available in the HSS instance ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses mnfp3gpp:IdentityRange;

}

list externalGroupIdentifiersRanges{

description "This attribute defines the list of ranges of external

group IDs that can be served by this HSS instance.If not provided,

the HSS instance does not serve any external groups. ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses mnfp3gpp:IdentityRange;

}

list hssDiameterAddress{

description "This attribute defines the Diameter Address of the HSS ";

max-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses NetworkNodeDiameterAddress;

}

list additionalDiamAddresses{

description "This attribute defines the Additional Diameter Addresses

of the HSS;may be present if hssDiameterAddress is present ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses NetworkNodeDiameterAddress;

}

}

grouping NrfInfoGrp {

list servedUdrInfo {

description "This attribute contains all the udrInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udr3gpp:UdrInfoGrp;

}

list servedUdmInfo {

description "This attribute contains all the udmInfo

attributes

locally configured in the NRF or the NRF

received during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udm3gpp:UdmInfoGrp;

}

list servedAusfInfo {

description "This attribute contains all the

ausfInfo attributes

locally configured in the NRF or the NRF

received during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses ausf3gpp:AusfInfoGrp;

}

list servedAmfInfo {

description "This attribute contains all the amfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses amf3gpp:AmfInfoGrp;

}

list servedSmfInfo {

description "This attribute contains all the smfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses smf3gpp:SmfInfoGrp;

}

list servedUpfInfo {

description "This attribute contains all the upfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses upf3gpp:UpfInfoGrp;

}

list servedPcfInfo {

description "This attribute contains all the pcfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcf3gpp:pcfInfoGrp;

}

list servedBsfInfo {

description "This attribute contains all the bsfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId{

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses bsfInfo;

}

list servedChfInfo {

description "This attribute contains all the bsfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses chfInfo;

}

list servedNwdafInfo {

description "This attribute contains all the nwdafInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId of which

the nwdafInfo belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses nwdaf3gpp:NwdafInfoGrp;

}

list servedPcscfInfoList {

description "This attribute contains all the pcscfInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId to which

the map entry belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcscfInfo;

}

list servedGmlcInfo {

description "This attribute contains all the gmlcInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId of which

the nefInfo belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses gmlc3gpp:GmlcInfoGrp;

}

list servedLmfInfo {

description "This attribute contains all the lmfInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId of which

the lmfInfo belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses lmf3gpp:LmfInfoGrp;

}

list servedNfInfo {

description "This attribute contains information of other NFs

without corresponding NF type specific Info extensions locally

configured in the NRF or the NRF received during NF registration.

The key of the map is the nfInstanceId of the NF. The map entry

is the NfInfo as defined in clause 5.3.229 representing the

information of a generic NF instance.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses nfInfoGrp;

}

list servedHssInfoList {

description "This attribute contains list of HssInfo attribute

locally configured in the NRF or that the NRF received during NF

registration. The key of the map is the nfInstanceId to which the

map entry belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses HssInfoGrp;

}

list served5gDdnmfInfo {

description "This attribute contains all the 5gDdnmfInfo

attribute locally configured in the NRF or that the NRF

received during NF registration. The key of the map is

the nfInstanceId to which the map entry belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses FiveGDdnmfInfo ;

}

}

grouping ImsiRange{

leaf start{

description "This attribute indicates the first value identifying

the start of a IMSI range. ";

type string{

pattern "^[0-9]+$";

}

}

leaf end{

description "This attribute indicates the last value identifying

the end of a IMSI range.";

type string{

pattern "^[0-9]+$";

}

}

leaf pattern{

description "This attribute indicates pattern (regular expression

according to the ECMA-262 dialect [75]) representing the set of

IMSIs belonging to this range. An IMSI value is considered part

of the range if and only if the IMSI string fully matches the

regular expression.Either the start and end attributes, or the

pattern attribute, shall be present.";

type string;

}

}

grouping NetworkNodeDiameterAddress {

leaf-list name{

description "This attribute indicates the Diameter

name of the network node diameter address.";

reference " See TS 29.571 [61]. String contains a Diameter Identity

(FQDN).";

type string;

min-elements 1;

max-elements 1;

}

leaf-list realm{

description "This attribute indicates the Diameter realm of the

network node diameter addres. See TS 29.571 [61]. String contains

a Diameter Identity (FQDN).";

type string;

min-elements 1;

max-elements 1;

}

}

typedef PduSessionType {

type enumeration {

enum IPV4;

enum IPV6;

enum IPV4V6;

enum UNSTRUCTURED;

enum ETHERNET;

}

}

grouping DnnUpfInfoItem {

leaf dnn {

description "String representing a Data Network.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported

by the UPF for this DNN.

The absence of this attribute indicates that the UPF

can be selected for this DNN for any DNAI.";

min-elements 1;

type string; //dnai is the type but its only a string with

//desc: DNAI (Data network access identifier),

//is this needed as its own typedef or string is ok

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by

the UPF for a specific DNN.";

min-elements 1;

type PduSessionType;

}

}

grouping nfInfoGrp {

description "This data type represents the information of a

generic NF Instance. (See clause 6.1.6.2.56 TS 29.510 [23]).";

leaf nfType {

description "This parameter defines type of Network Function";

type uint16;

}

}

grouping FiveGDdnmfInfo {

description "This data type represents the specific data for the 5G DDMF NF.

See clause 6.1.6.2.74 of TS 29.510.";

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s)

is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping AIoTNRFMappingGrp {

description "Data type AIoTNRFMapping";

leaf aIOTFdN {

type types3gpp:DistinguishedName;

mandatory true;

description "Represents the distinguished name (DN) identifier of the

AIOTF that serves the (internal) target area provided by NEF to NRF";

}

list internalTargetArea {

description "It is the (internal) target area used by NRF (as provided

by NEF) to accurately locate the AIOTF instance. It is mapped to aIOTFdN.

It could refer to a list of AIoT Areas.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses aiot3gpp:ServedAIOTAreaIDGrp;

}

}

grouping NRFFunctionGrp {

description "Represents the NRFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

description "It defines the PLMN(s) of a Network Function.";

min-elements 1;

key "mcc mnc sd sst";

uses types5g3gpp:PLMNInfo;

}

leaf sBIFQDN {

description "It is used to indicate the FQDN of the registered NF instance

in service-based interface, for example, NF instance FQDN structure is:

nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org";

type inet:domain-name;

mandatory true;

}

list nFProfileList {

description "It is a set of NFProfile(s) to be registered in the NRF

instance. NFProfile is defined in 3GPP TS 29.510 ";

key idx;

leaf idx { type uint32; }

uses mnfp3gpp:ManagedNFProfileGrp;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list nrfInfo {

description "Represents information of an NRF NF Instance.";

max-elements 1;

key idx;

leaf idx {type uint32;}

uses NrfInfoGrp;

}

list managedNFProfile {

key idx;

leaf idx { type uint32; }

min-elements 1;

max-elements 1;

uses mnfp3gpp:ManagedNFProfileGrp;

description "This parameter defines profile for managed NF";

reference "3gpp TS 23.501";

}

list aIoTNRFMapping {

config false;

yext3gpp:inVariant;

yext3gpp:notNotifyable;

description "Represents mapping information between (Internal) target

area (provided by NEF) and AIOTF DN.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses AIoTNRFMappingGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NRFFunction {

description "5G Core NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 5 ***

*** START OF CHANGE 11 ***

*** yang-models/_3gpp-nr-nrm-aiotreader.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-aiotreader {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-aiotreader";

prefix "aiotread3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-aiot { prefix aiot3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping the AIoT Reader.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-11-07 { reference CR-1640; }

grouping AIOTReaderGrp {

description "Attributes of the IOC AIOTReader";

leaf readerId {

type int32;

mandatory true;

description "It defines the reader identifier to uniquely identify a

reader within a gNB.";

}

leaf administrativeState {

type types3gpp:BasicAdministrativeState;

default LOCKED;

description "It indicates the administrative state of the AIOTReader.

It describes the permission to use or prohibition against using the

AIOT reader, imposed through the OAM services.";

}

leaf-list supportedAIOTServices {

type enumeration {

enum INVENTORY;

enum COMMAND;

}

min-elements 1;

description "It indicates the supported AIOT service type for an

AIOT reader.";

}

list pLMNId {

description "Defines which PLMN that can be served by the AIOT reader.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list servedAIOTAreas {

description "This attribute is used to specify the A-IoT areas supported

by the A-IoT reader. It contains one or multiple A-IoT Area ID, which

is used to uniquely identify an A-IoT Area.

A-IoT Area ID = PLMN ID +NID (optional) + A-IoT Area Code

(OCTET STRING (SIZE(3))), which is defined in TS 38.413.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses aiot3gpp:ServedAIOTAreaIDGrp;

}

leaf readerLocation {

type string;

mandatory true;

description "This specifies the geographical location of a A-IoT reader.

Reader Location may represent any of latitude/longitude, or any

geographical location/coordinate/area polygon.";

}

leaf-list nRSectorCarrierRef {

type types3gpp:DistinguishedName;

description "This attribute contains the DN of the referenced

NRSectorCarrier.";

}

}

augment /me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction  {

list AIOTReader {

description "This IOC represents AIoT Reader which supports AIOT

services.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AIOTReaderGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 11 ***

*** START OF CHANGE 12 ***

*** yang-models/_3gpp-nr-nrm-nrfreqrelation.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-nrfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrfreqrelation";

prefix "nrfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-nr-nrm-redcapaccess { prefix redcap3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-11-02 { reference CR-1643; }

revision 2025-01-25 { reference CR-1442; }

revision 2024-02-24 { reference CR-1218; }

revision 2023-09-18 { reference CR-1043; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-04-23 { reference CR-0281; }

revision 2019-10-28 { reference S5-193518; }

revision 2019-06-17 {

reference "Initial version";

description "Initial revision";

}

grouping NRFreqRelationGrp {

description "Represents the NRFreqRelation IOC.";

reference "3GPP TS 28.541";

container offsetMO {

description "A set of offset values applicable to all measured cells

with reference signal(s) indicated in corresponding MeasObjectNR. It

is used to indicate a cell, beam or measurement object specific offset

to be applied when evaluating candidates for cell re-selection or when

evaluating triggering conditions for measurement reporting. It is

defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB,

rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS.";

reference "offsetMO in MeasObjectNR in 3GPP TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb {

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs {

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf-list blockListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blocklisted in NR measurements.";

reference "3GPP TS 38.331";

type uint16 { range "0..503"; }

}

leaf-list blockListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blocklisted in SIB4 and SIB5.";

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in subclause 5.2.4.1 of 3GPP TS 38.304.";

reference "CellReselectionPriority in 3GPP TS 38.331, priority in

3GPP TS 38.304";

type uint32;

default 0;

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "3GPP TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

list cellReselectionRedcap {

description "This attribute indicates the configuration parameters

to allow relaxation of RRM measurement requirements for redcap UE

cell reselection";

key idx;

leaf idx{ type uint32;}

reference "see clause 6.3.1 TS 38.331 ";

uses redcap3gpp:CellReselectionRedcapGrp;

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in 3GPP TS 38.304), at cell reselection to a cell.";

reference "PEMAX in 3GPP TS 38.101-1";

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

type types3gpp:QOffsetRange;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "3GPP TS 38.304";

type int32 { range "-34..-3 | 0"; }

units dB;

default 0;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the NR frequency for cell reselection.

Broadcast in SIB3 or SIB5, depending on whether the related frequency

is intra- or inter-frequency. Resolution is 2.";

reference "3GPP TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionNR {

description "Cell reselection timer for NR.";

reference "TreselectionRAT for NR in 3GPP TS 38.331";

mandatory true;

type int32 { range "0..7"; }

units s;

}

leaf tReselectionNRSfHigh {

description "The attribute tReselectionNr (parameter TreselectionNR in

3GPP TS 38.304) is multiplied with this scaling factor if the UE is

in high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for high

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionNRSfMedium {

description "The attribute tReselectionNr (parameter TreselectionNR in

3GPP TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for medium

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf nRFrequencyRef {

description "Reference to a corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement" +

"/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

list NRFreqRelation {

description "Together with the target NRFrequency, it represents the

frequency properties applicable to the referencing NRFreqRelation.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFreqRelationGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 12 ***

*** START OF CHANGE 13 ***

*** yang-models/_3gpp-nr-nrm-redcapaccess.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-redcapaccess {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-redcapaccess";

prefix "redcap3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-11-02 { reference CR-1643; }

revision 2025-07-07 { reference CR-; }

feature redcapaccesscriteriasubnertwork {

description "The RedCap acsess criterea shall be contained under

SubNetwork";

}

grouping CellReselectionRedcapGrp {

description "This data type defines configuration parameters

to allow relaxation of RRM measurement requirements for (e)RedCap UE

cell reselection. ";

reference "TS 38.331 , clasue 6.3.1 and TS 38.304 , clause 5.2.4.9";

leaf sSearchDeltaPStationary {

type uint8 {

range "2 | 3 | 6 | 9 | 12 | 15";

}

mandatory true;

description "This specifies the threshold (in dB) on Srxlev variation

to evaluate stationary criterion for relaxed measurement. It

corresponds to the SSearchDeltaP-Stationary in TS 38.304 [49].

Its unit is 1 dB. Where Srxlev is the cell selection received

signal level value. ";

// optional

}

leaf tSearchDeltaPStationary {

type uint16 {

range "5 | 10 | 20 | 30 | 60 | 120 | 180 | 240 | 300";

}

mandatory true;

description "TThis specifies the time period over which the Srxlev

variation is evaluated for stationary criterion for relaxed measurement.

It corresponds to the TSearchDeltaP-Stationary in TS 38.304 [49]. Its

unit is seconds. Where Srxlev is the cell selection received signal

level value. ";

//optional

}

leaf sSearchThresholdP2 {

type uint16 {

range "0..31";

}

mandatory true;

description "This specifies the Srxlev threshold (in dB) to evaluate

not at cell edge criterion for relaxed measurement. It corresponds

to the SSearchThresholdP2 in TS 38.304 [49]. Its unit is 1 dB.

Where Srxlev is the cell selection received signal level value.  ";

//optional

}

leaf sSearchThresholdQ2 {

type uint16 {

range "0..31";

}

mandatory true;

description "This specifies the Squal threshold (in dB) to

evaluate not at cell edge criterion for relaxed measurement. It

corresponds to the SSearchThresholdQ2 in TS 38.304 [49]. Its unit

is 1 dB. Where Srxlev is the cell selection quality level value. ";

//optional

}

}

grouping RedCapAccessCriteriaGrp {

description "Represents the RedCapAccessCriteria <<IOC>> ";

leaf nRCellDURef {

type types3gpp:DistinguishedName;

mandatory true;

description "This attribute contains the DN of a NR Cell (NRCellDU) ";

}

leaf criteriaConditionRef {

type types3gpp:DistinguishedName;

mandatory true;

description "This specifies the DN of the ConditionMonitor MOI.The

attribute condition will contain information on the condition to be

satisfied to restrict Redcap UE access. This means that the value of

attribute uECellBarredAccess of NRCellDU IOC will be set to

REDCAP_1RX and REDCAP_2RX if this condition is met.The condition

will be created providing following information:

The performance metrics (KPIs and performance measurements) that are

to be considered in the criteria for deciding whether the cell in a

RAN node is barred or allowed for (e)RedCap UEs

The direction (up and down) that is to be considered for crossing

the threshold value of the given performance metrics for taking a

decision whether the RAN node is barred or allowed for

(e)RedCap UEs.

The threshold level of performance metrics value which when crossed

the RAN node is barred or allowed for (e)RedCap UEs";

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature redcapaccesscriteriasubnertwork;

list RedCapAccessCriteria {

description "The RedCapAccessCriteria <<IOC>> contains attributes

to support the controlling of RedCap access in a NR Cell.

It provides information of a condition (for example, KPIs such as

Total DRB accessibility, DRB Retainability , Utilization KPI,

NG-RAN data Energy Efficiency, Downlink/Uplink delay in gNB-DU etc.

and  Performance measurements such as DL/UL Total PRB Usage,

Average DL/UL UE throughput in gNB etc.) which can be used to

control decision of barring or allowing access of RedCap/eRedCap

UEs to a NR Cell. This information can be used to decide whether

RedCap/eRedCap UEs are barred or allowed at the time of new UE

access attempt in NR cell during applicable RAN procedures.";

key id;

max-elements 1;

uses top3gpp:Top_Grp;

container attributes {

uses RedCapAccessCriteriaGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 13 ***