3GPP TSG-SA5 Meeting #164	S5-255224

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0172 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.533 Update Annex F to add missing R19 management capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA5#163: The following management capabilities defined in R19 as missing in the Table F-1: Overview of management capabilities and corresponding solution sets in Annex F in TS 28.533. - NDT lifecycle management defined in TS 28.561. - Cloded control loop management defined in TS 28.567. - Signalling traffic monitoring management defined in TS 28.560. - Management of planned configurations defined in TS 28.572. SA5#164: Duplicated references for TS 28.561 and TS 28.567 in clause 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SA5#163: Add following management capabilities and corresponding solution sets in Annex F: - STM provisioning - STM streaming - Configuration plan management - Closed control loop management - NDT Lifecycle Management SA5#164: Remove the duplicated references for TS 28.561 and TS 28.567 in clause 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA5#163: Several R19 manageemnt capabilities are missing in the Table F-1: Overview of management capabilities and corresponding solution sets in Annex F in TS 28.533 SA5#164: Duplicated references in clause 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1. S5-255224 is the revision of S5-254276 |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void

[3]	3GPP TS 28.530: "Management and orchestration of networks and network slicing; Concepts, use cases and requirements".

[4]	3GPP TS 28.541: "Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3".

[5]	3GPP TS 28.552: "Management and orchestration of 5G networks; Performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration of 5G networks; 5G End to end Key Performance Indicators (KPI)".

[7]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[8]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[9]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[10]	3GPP TS 28.500: "Telecommunication management; Management concept, architecture and requirements for mobile networks that include virtualized network functions"

[11]	3GPP TS 28.510: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Requirements".

[12]	3GPP TS 28.511: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[13]	3GPP TS 28.512: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 2".

[14]	3GPP TS 28.513: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 3".

[15]	3GPP TS 28.515: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Requirements".

[16]	3GPP TS 28.516: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Procedures".

[17]	3GPP TS 28.517: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Stage 2".

[18]	3GPP TS 28.518: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Stage 3".

[19]	3GPP TS 28.520: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Requirements".

[20]	3GPP TS 28.521: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.522: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2".

[22]	3GPP TS 28.523: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3".

[23]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[24]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[25]	3GPP TS 28.527: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 2".

[26]	3GPP TS 28.528: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 3".

[27]	ETSI GS NFV 003: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV V1.3.1 (2018-01)".

[28]	Void.

[29]	ETSI GS ZSM 002: "Zero-touch Network and Service Management (ZSM); Reference Architecture V.1.1.1 (2019-08)".

[30]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[31]	3GPP TS 23.501: "System Architecture for the 5G system".

[32]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[33]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[34]	IETF RFC 4253: "The Secure Shell (SSH) Transport Layer Protocol".

[35]	3GPP TS 28.100: "Management and orchestration; Levels of autonomous network".

[36]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[37]	3GPP TS 28.535: "Management services for communication service assurance; Requirements".

[38]	3GPP TS 28.536: "Management services for communication service assurance; Stage 2 and stage 3".

[39]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[40]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[41]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1".

[42]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[43]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[44]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[45]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[46]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[47]	3GPP TS 28.557: "Management and orchestration; Management of Non-Public Networks (NPN); Stage 1 and stage 2".

[48]	3GPP TS 28.404: "Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements".

[49]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[50]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[51]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[52]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[53]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[54]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[55]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements".

[56]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[57]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[58]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[59]	3GPP TS 28.314: "Management and orchestration; Plug and Connect; Concepts and requirements".

[60]	3GPP TS 28.315: "Management and orchestration; Plug and Connect; Procedure flows".

[61]	3GPP TS 28.316: "Management and orchestration; Plug and Connect; Data formats".

[62]	3GPP TS 28.555: "Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

[63]	3GPP TS 28.556: "Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3".

[64]	ETSI GS NFV-IFA 008 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[65]	ETSI GS NFV-IFA 013 (V4.3.1): "Network Function Virtualisation (NFV) Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[66]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management".

[67]	3GPP TS 28.317: "Management and orchestration;Self-configuration of Radio Access Network Entities (RAN NEs)".

[68]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[69]	3GPP TS 28.318: "Management and Orchestration; Network and services operations for energy utilities".

[70]	3GPP TS 28.319: "Management and orchestration; Access Control for Management services".

[71]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)"

[72]	3GPP TS 28.558: "Management and Orchestration UE level measurements for 5G system"

[73]	3GPP TS 28.561: "Management and orchestration; Management aspects of Network Digital Twins"

[74]	3GPP TS 28.567: "Management and orchestration; Management aspects of closed control loops"

[75]	3GPP 29.501: "5G System; Principles and Guidelines for Services Definition"

[76]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2"

[77]	3GPP TS 28.579: "Management and orchestration; Management services exposure to external consumers through CAPIF"

[X]	3GPP TS 28.560: "Management and orchestration; Signalling traffic monitoring management (Stage 1, stage 2, and stage 3)".

[Y]	3GPP TS 28.572: "Management and orchestration; Management of planned configurations".

2nd Change

Annex F (informative):
Overview of management capabilities and corresponding solution sets in SBMA

The model driven approach (i.e. usage of CRUD operations specified in TS 28.532 [9] and NRM fragments) can be used to support various types of management capabilities in SBMA. In addition, there are also several management capabilities which are implemented by non-CRUD operations. In the context of SBMA, Management Capability refers to the functional abilities that a management system can offer to monitor, control, and optimize network and service. These management capabilities are offered via standardized service interfaces called MnS. MnS allows these management capabilities to be consumed by internal and external consumers through well-defined APIs. Each MnS instance can combine these SBMA MnS component types A, B and C to deliver a specific management capability (see clause 4.3 combination of MnS components). Table F-1 shows an overview of management capabilities, MnS and corresponding solution sets.

Table F-1: Overview of management capabilities and corresponding solution sets

| Management Feature | Management Capability | MnS definition | Solution Sets |
| --- | --- | --- | --- |
| Network and network slicing management | NR Provisioning | CRUD operations/notifications (3GPP TS 28.532 [9]) + NR NRM fragment (3GPP TS 28.541 [4]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | 5GC Provisioning | CRUD operations/notifications (TS 28.532 [9]) + 5GC NRM fragment (3GPP TS 28.541 [4]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Network Slicing Provisioning | CRUD operations/notifications (3GPP TS 28.532 [9]) + Network Slicing NRM fragment (3GPP TS 28.541 [4]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | Network slicing provisioning service (3GPP TS 28.531 [8]) | RESTFUL |
| Edge Computing Management | Edge Computing Provisioning | CRUD operations/notifications (3GPP TS 28.532 [9]) + Edge NRM fragment (3GPP TS 28.538 [40]) | RESTFUL |
| Performance Assurance | Performance Metric Collection Control | CRUD operations/notifications (3GPP TS 28.532 [9]) + PM control NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | CRUD operations/notifications (3GPP TS 28.532 [9]) + ManagementDataCollection control NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | Performance measurement job control (3GPP TS 28.550 [42]) | RESTFUL |
|  | Performance Metric Data Report | Streaming data reporting service (3GPP TS 28.532 [9]) + Performance data stream units (3GPP TS 28.550 [42]) | RESTFUL+WebSocket+(GPB/ASN.1) (also used by the NETCONF/YANG solution set) |
|  |  | File data reporting service (3GPP TS 28.532 [9]) + Performance data file format (3GPP TS 28.532 [9]) | RESTFUL+( SFTP/FTPES/HTTPS)+XML |
|  | Performance Metric Threshold Monitor Control | CRUD operations/notifications (3GPP TS 28.532 [9]) + Threshold monitoring control NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Performance Metric Threshold Notification | notifyThresholdCrossing notification (3GPP TS 28.532 [9]) | RESTFUL (also used by the NETCONF/YANG solution set) |
| Fault Management | Fault control | CRUD operations/notifications (3GPP TS 28.532 [9]) + FM control NRM fragment (3GPP TS 28.111 [68]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Fault Notification | Fault Notifications (3GPP TS 28.111 [68]) | RESTFUL (also used by NETCONF/YANG) |
| Trace and MDT management | Trace/MDT data collection control | CRUD operations/notifications (3GPP TS 28.532 [9]) + Trace control NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  |  | CRUD operations/notifications (3GPP TS 28.532 [9]) + ManagementDataCollection control NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Trace/MDT data report | Streaming data reporting service (3GPP TS 28.532 [9]) + Trace/MDT stream date schema definition (3GPP TS 32.423 [45]) | RESTFUL+WebSocket+(GPB/ASN.1) (also used by the NETCONF/YANG solution set) |
|  |  | File data reporting service (3GPP TS 28.532 [9]) + Trace/MDT file date format definition (TS 32.423 [45]) | RESTFUL+( SFTP/FTPES/HTTPS)+XML |
| Signalling traffic monitoring management | STM provisioning | CRUD operations/notifications (3GPP TS 28.532 [9]) + STM control NRM fragment (3GPP TS 28.560 [X]) | NETCONF/YANG |
|  | STM streaming | Format of the STM reports (3GPP TS 28.560 [X]) | UDP/TCP transport protocols +( UDP-GRE-PCAPNG / Type - Length - Value (TLV)) |
| QoE management | QoE data collection control | CRUD operations/notifications (3GPP TS 28.532 [9]) + QoE Measurement Collection control NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | QoE data report | File data reporting service (3GPP TS 28.532 [9]) + QoE data file format (3GPP TS 26.247 [71]) | RESTFUL+(SFTP/FTPES/HTTPS)+XML |
| File Management | File Retrieval | CRUD operations/notifications (3GPP TS 28.532 [9]) + File retrieval NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | File Download | CRUD operations/notifications (3GPP TS 28.532 [9]) + File download NRM fragment (3GPP TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
| Plan management | Configuration plan management | Information model for configuration plan management (3GPP TS 28.572[Y]) | RESTFUL |
| Notification subscription and Heartbeat notification control | Subscription Control | CRUD operations/notifications (3GPP TS 28.532 [9]) + Notification subscription and heartbeat notification control NRM fragment (3GPP TS 28.622 [32) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Heartbeat Control | CRUD operations/notifications (3GPP TS 28.532 [9]) + Heartbeat notification control NRM fragment (3GPP TS 28.622[32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | Heartbeat Notification | notifyHeartbeat notification (3GPP TS 28.532 [9]) | RESTFUL |
| MDA | Management Data Analytic | CRUD operations/notifications (3GPP TS 28.532 [9]) + NRM fragment for MDA request and MDA report (3GPP TS 28.104 [57]) | RESTFUL |
| SON | RANSC Management | CRUD operations/notifications (3GPP TS 28.532 [9]) + RANSC NRM Fragment 3GPP TS 28.317 [67]) | RESTFUL |
|  | SON policy | CRUD operations/notifications (3GPP TS 28.532 [9]) + NRM Fragment for DANR/DES/DRACH/DMRO/DPCI/CES/CPCI/DLMO/CCO Management (3GPP TS 28.541 [4]) | RESTFUL |
|  |  |  | NETCONF/YANG |
| Closed-loop | Communication Service Assurance Control | CRUD operations/notifications (3GPP TS 28.532 [9]) + Assurance management NRM fragment (3GPP TS 28.536 [38]) | RESTFUL |
|  | Closed control loop management | CRUD operations/notifications (3GPP TS 28.532 [9]) + NRM fragment for CCL management (3GPP TS 28.567 [74]) | RESTFUL |
| Intent driven management | Intent Driven Management | CRUD operations/notifications (3GPP TS 28.532 [9]) + NRM fragment for intent driven management (3GPP TS 28.312 [46]) | RESTFUL |
| AI/ML management | ML model Management | CRUD operations/notifications (3GPP TS 28.532 [9]) + NRM fragment for ML model training/testing/ inference emulation control/ ML model loading / inference (3GPP TS 28.105 [66]) | RESTFUL |
| NDT management | NDT Lifecycle Management | CRUD operations/notifications (3GPP TS 28.532 [9]) + NRM fragment for NDT management (3GPP TS 28.561 [73]) | RESTFUL |
| MnS Registry and Discovery | MnS Registry and Discovery | CRUD operations/notifications (3GPP TS 28.532 [9]) + MnS Registry NRM fragment (TS 28.622 [32]) | RESTFUL |
|  |  |  | NETCONF/YANG |
|  | MgmtData Registry and Discovery | CRUD operations/notifications (3GPP TS 28.532 [9]) + MnS Registry NRM fragment (TS 28.622 [32]) | RESTFUL |
| MSAC | MnS Access Control | CRUD operations/notifications (3GPP TS 28.532 [9]) + Information model for role based access control (3GPP TS 28.319 [70]) | RESTFUL |
| NSOEU | DSO Rapid Recovery and Threshold Monitoring | CRUD operations/notifications (3GPP TS 28.532 [9])+ DSO Rapid Recovery NRM fragment (3GPP TS 28.318 [12]) + DSO Rapid Recovery NRM fragment(3GPP TS 28.318 [69]) | RESTFUL |
| External Data Management | External Data Discovery and Request | CRUD operations/notifications (3GPP TS 28.532 [9]) + External data type NRM fragment (TS 28.622 [32]) | RESTFUL |

End of Changes