3GPP TSG-SA5 Meeting #164	S5-255232

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0749 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add N6 delay measurement for EAS and local UPF (re)selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Taking N6 delay into account by SMF will help the SMF to (re)select/(re)discover more suitable (L-)PSA UPF and optionally EAS in order to reduce the latency for Edge Computing services. Therefore, it would be necessary for the operators to monitor the performance of the N6 delay measurement procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add N6 related measurements： - Number of requested N6 delay measurements - Number of successful N6 delay measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature for Enhancement of EAS and local UPF (re)selection is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.X (new), 5.4.2.Y (new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.4.2.X	Number of requested N6 delay measurements

a)	This measurement provides the number of N6 delay measurement requests received by the UPF.

b)	CC.

c)	On receipt of N4 association setup request or N4 association update request message (see TS 23.502 [7]) containing one or more "N6 Delay Measurement Control Information" IE(s) (see TS 29.244 [16]) by the UPF from SMF. Each "N6 Delay Measurement Control Information" IE with non-null "Measurement Endpoint Address" IE and "Network Instance" IE (see table 7.4.4.1.4-1 in TS 29.244 [16]) indicating the requested N6 delay measurement triggers the counter to increment by 1.

d)	A single integer value.

e)	SM.N6DelayReq.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.4.2.Y	Number of successful N6 delay measurements

a)	This measurement provides the number of successful N6 delay measurements at the UPF.

b)	CC.

c)	On transmission of N4 report message (see TS 23.502 [7]) containing one or more "N6 Delay Measurement Report" IE(s) (see TS 29.244 [16]) by the UPF to SMF. Each "N6 Delay Measurement Report" IE indicating the successful N6 delay measurement triggers the counter to increment by 1.

d)	A single integer value. In case of measurement reporting type is periodic or event triggered reporting, multiple N4 report messages containing "N6 Delay Measurement Report" IE with the same "Measurement Endpoint Address" IE and "Network Instance" IE (see table 7.4.5.1.8-1 in TS 29.244 [16]) are counted as one.

e)	SM. N6DelaySucc

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

A.X	Monitoring of N6 delay measurement procedures for EAS and local UPF (re)selection

The candidate UPF(s) may be configured by the SMF, as described in clause 4.4.3 of TS 23.502 [3], to measure the N6 delay for the connection between the UPF and measurement endpoint(s) in the DN (represented by one or more IP address(es)). The SMF may (re)select the PSA UPF(s) or trigger EAS(es) (re)discovery by considering N6 delay measurement result, see TS 23.501 [4].

The performance of N6 delay measurement procedures reflect the quality between the UPF and measurement endpoint(s) in the DN. The monitoring of N6 delay measurement request and success is needed for EAS and local UPF (re)selection evaluation.

==============End of change==============