3GPP TSG-SA5 Meeting #162	S5-254062

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0717 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR 28.552 Add a new measurement related to the number of roaming data collection service subscriptions received by RE-NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_Ph3-OAM |  |  |  |  |  | Date: |  |  | 2025-8-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.288 (SA2), the “Nnwdaf_RoamingData_Subscribe service operation” is specified under “NWDAF Roaming Data” that enables the consumer to subscribe for roaming data. By measuring the number of roaming data collection service subscriptions received by RE-NWDAF, the operator can estimate the potential workload on NWDAF and take management actions for configuration and resource allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Number of subscriptions related to roaming data collection service received by RE-NWDAF” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the performance of “Nnwdaf_RoamingData_Subscribe service operation” and to take RE-NWDAF management actions for configuration, resource allocation, load balancing and placement purposes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-254062 was S5-253417 |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.X    Measurements related to RE-NWDAF Data Collection

5.18.X.1   Number of subscriptions related to roaming data collection service received by RE-NWDAF

a)	This measurement provides the number of subscriptions related to roaming data collection service received by an RE-NWDAF.

b) CC

c)	On receiving the subscription of roaming data collection service by the RE-NWDAF from an RE-NWDAF in other PLMN, each subscription is added to the corresponding counter. The measurement can be split into sub-counters per S-NSSAI and per PLMN.

In the scenario where the H-RE-NWDAF in the HPLMN as service consumer to subscribe to notifications about input data from the V-RE-NWDAF in VPLMN, when the subscription for the roaming data collection service is received by the V-RE-NWDAF, each subscription is added to the corresponding counter.

In the scenario where the V-RE-NWDAF in the VPLMN as service consumer to subscribe to notifications about input data from the H-RE-NWDAF in HPLMN, when the subscription for the roaming data collection service is received by the H-RE-NWDAF, each subscription is added to the corresponding counter.

d)	It is an integer value

e)	DANS. RoamingDataSerSubReceived

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

Start of Second change

Annex A (informative):

Use cases for performance measurements

A.X	Monitoring of RE-NWDAF data collection

In TS 23.288, three service operations are specified under "NWDAF Roaming Data" that enables the consumer to subscribe for roaming data. These service operations include the following:

•	Nnwdaf_RoamingData_Subscribe service operation

•	Nnwdaf_RoamingData_Unsubscribe service operation

•	Nnwdaf_RoamingData_Notify service operation

The number of roaming data subscriptions and notifications are some of the basic statistic information to monitor the performance of RE-NWDAF (Roaming Exchange Network Data Analytics Function). These basic measurements can be used to derive the other performance of RE-NWDAF such as roaming data service successful and/or failed rate, etc. With these measurements, operators can analyze and evaluate the performance of the RE-NWDAF. Operators can use these measurements to take management actions for configuration, resource allocation, load balance and placement purposes.

End of Changes