3GPP TSG-SA5 Meeting #163
S5-254633
Wuhan, China, 13 - 17 October 2025
Source:
Ericsson, AT&T, Nokia, Rakuten, Docomo
Title:
Pseudo-CR on TR 28.869 Recommendation for LCM of NF Deployment 
Document for:
Approval

Agenda item:
6.19.6

Spec:
3GPP TR 28.869
Version:
v1.6.0

Work Item:
FS_Cloud_OAM 

Comments

This contribution proposes the recommendations for LCM of NF Deployment using industry solutions.
Proposed Changes

[image: image1.emf]* * * First Change * * * *  


6 


Conclusions and recommendations


6.x
Recommendations for LCM of NF Deployment

The study has identified use cases, requirements, and solutions in clause 5.2 for 3GPP management system to support LCM of NF Deployment using industry solutions including the use of both ETSI NFV MANO and non ETSI NFV MANO.

To enable 3GPP the use of non ETSI NFV MANO for LCM of NF Deployment in addition to existing use of ETSI NFV MANO solution, it is recommended to start further normative work according to clauses 4.2, 5.2.1, 5.2.3, 5.2.4 and 5.2.5 and enhance TS 28.531 to specify the following in Rel-20:
· Specify 3GPP management architecture to support LCM of NF Deployment including the requirement for deployment management reference point as described in clause 5.2.1.
· Specify the requirements and use cases for 3GPP management system to support LCM of NF Deployment as described in clauses 5.2.3, 5.2.4 and 5.2.5.
· 
Enhance the LCM of NF procedures in TS 28.531 clause 7.10, 7.11 and 7.12 to support both NFV-ΜΑΝΟ and Non-ETSI NFV MANO for LCM of NF Deployment, including procedures that decouple the LCM of NF Deployment from LCM of NF. 
The normative work preserves the following:
· The 3GPP management system is not responsible for the management of the non-application parameters of the NF Deployment. 
· 
· In the context of the present document, 3GPP management system does not specify non-application parameters of the NF Deployment.
· Cloud infrastructure management aspects are out of scope of the 3GPP management system.
· The realization of the deployment management reference point (described in clause 5.2.1.3) is implementation specific and the specification of the interfaces is out of scope of 3GPP. 

* * * End of Changes * * *
