3GPP TSG SA WG5 Meeting #163	S5-254658

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei, China Mobile

Title:	Pseudo-CR on 28.881 Add use case for the investigation on the applicability and potential impacts to support natural language intents translation

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes to add use case for the Investigation on the applicability and potential impacts to support natural language intents translation based on corresponding use case documented in TR 28.914.

This is related to WT-6.

Proposed Changes

* * * First Change * * * *

4.X Use case #X: Investigation on the applicability and potential impacts to support natural language intents translation

4.X.1 Description

In TR 28.914 [X], clause 5.6 described the use case for investigating potential impacts to support natural language intents translation. It also states that there are some use cases where the use of natural language for intent expression would be preferred. For example, on the human-machine interface presenting and expressing intent using natural language can lead to a more intuitive experience for human users. A concrete example of natural language RAN ES intent can be “Reduce 10% RAN energy consumption without impact on user experience for Region X”. The use case also described that if an intent interpreter is introduced to process intents described in natural language, it is necessary to clarify its position within the existing intent management framework, especially its relationship with the MnS consumer.

Scnenario#1: Intent Interpreter as a separate function outside the intent handling function. In this scenario, Intent interpreter receive the natural language intent and translate it into a formal intent (the formal intent contains the attributes and corresponding values to enable the intent handling function to decide the actions to fulfil the intent expectation) according to intent model defined in TS 28.312 [1] and consume the 3GPP Intent driven MnS with formal intent model provided by intent handling function.

Scnenario#2: Intent Interpreter is a function integrated in intent handling function. In this scenario, Intent interpret receive the natural language intent from consumer  and translate it into detailed actions to be executed to fulfil the intent.

Based on above two deployment scenarios, this study can investigate whether there is a need for a potential solution to enhance the intent-driven management service to support natural language

.

4.X.2 Potential requirements

TBD

4.X.3 Potential solution

TBD

4.X.4 Evaluation of potential solutions

TBD

* * * End of Changes * * * *