3GPP TSG-SA5 Meeting #163	S5-254720

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0737 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add DL data transmission time per SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The proposed change aims to provide operators with beam-level visibility into downlink performance. In current 5G networks, especially those using higher frequencies, SSBs are not just for initial cell search but are crucial for beam management. It specifically counts the time associated with successful data transmissions, as indicated by the reception of feedback from the UEs. This reflects the total time of successfully transmitted data volume under the SSB beam. It allows operators to move beyond cell-level metrics and understand how efficiently each individual beam is delivering user data . This can reveal if certain beams, perhaps those serving cell-edge users, are congested or experiencing poor channel conditions. In advanced 5G deployments with a high number of beams, this metric is crucial for validating and optimizing beamforming strategies, ensuring network capacity and user experience are balanced across all coverage areas |  |  |  |  |  |  |  |  |
| Summary of change: |  | This measurement provides the downlink data transmission time for UEs under an SSB beam. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of beam-level visibility into downlink performance |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.28.x(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.1.28.x	DL data transmission time per SSB

a)	This measurement provides the downlink data transmission time for UEs under an SSB beam.

b)	CC.

c)	This measurement is incremented by duration of the slot in the time domain of gNodeB in TS 38.213[63] in milliseconds, where the gNodeB schedules PDSCH data for any UE(s) as the service beam. The value is incremented by the corresponding slot after the gNodeB receives feedback for the downlink-scheduled PDSCH data from UEs served by the SSB beam.

d)	Each measurement is a float value (in milliseconds).

e)	L1M.DLDataTrsTimeSSB.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

End of change