3GPP TSG-SA5 Meeting #163	S5-254733

Wuhan, China, 13. -17. October 2025

Source:	Ericsson, Nokia

Title:	Rel-20 pCR TR 28.883 Defining the Lifecycle and Runtime Behaviour of NDT Jobs

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.0.0

Work Item:	FS_NDT_Ph2

Comments

The following pCR introduces issues which would help define the lifecycle and runtime behaviour of the NDT Jobs for the MnS consumer of the NDT MnS Producer.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 28.561: “Management and orchestration; Management aspects of Network Digital Twins”

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

# 5	Use cases

## 5.X	Use Case #<A> Defining the Lifecycle and Runtime Behaviour of NDT Jobs

### 5.X.1	Description

The NDTJob lifecycle is not clear, there are some items open to interpretation and some items missing which are described below.

Issue#1: The MnS Consumer should have a method to know which configuration of NDTJob has produced a given report. In the current specification NDTReport refer to the NDT job (i.e NDTJobRef). In the existing specification, it is possible to modify the NDTJob which may result in the different NDTReport. This will result in loosing the link between produced NDT Report and the original NDT Job. The MnS Consumer would benefit from understanding the implications of reconfiguring the NDTJob. At the moment, the NDTReports refer to the DN of the NDTJob – if a NDTJob is reconfigured, this means the same DN is applied to each report, even if the simulation has changed. Likewise, there is no clear procedure described in Clause 6.4, despite the “Modify” use-case being possible for the NDTJob Instances.

Excerpt: Table 6.2.1.3.8.2-1 describes the attributes associated with the NDTReport <<IOC>>

Excerpt: From 6.3 Attribute definitions, the explanation of the ndtJobRef of the NDTReport <<IOC>> refers to

### 5.X.2	Potential requirements

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of potential solutions

# 6	Conclusions and Recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *