3GPP TSG-SA5 Meeting #163	S5-254741

Wuhan, China, 13th Oct 2025 - 17th Oct 2025	(revision of S5-254528)

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case to study EC and EE per PLMNID in Network sharing scenario

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.0.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for "Enhancements to Energy Consumption and Energy Efficiency measurements and KPIs", related to WT-4.

This pCR introduces use case and potential requirements related to WT-4 of the SI FS_Energy_Ph4_OAM.

"

WT-4. Study enhancements to Energy Consumption and Energy Efficiency measurements and KPIs for Network Functions and Network Slice.

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[B]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[X]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[C]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[D]	3GPP TS 32.130: " Telecommunication management; Network sharing; Concepts and requirements".

* * * Next Change * * * *

### 5.4.A	Use case #<X>: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

#### 5.4.A.1	Description

When a NG-RAN is shared between multiple operators in the network sharing (see TS 32.130 [D]), the participating operators are interested in the operator-specific energy consumption (EC) and energy efficiency (EE) of the NG-RAN.

The NG-RAN can support and serve multiple operators (i.e., PLMN-IDs) in Network sharing scenarios. The existing EE and EC KPIs do not consider network sharing scenarios, and do not provide KPIs at operator-specific (i.e., PLMN-ID) granularity.

Following are the currently supported measurements and KPIs related to energy consumption and energy efficiency:

- Power, Energy and Environmental (PEE) measurements are defined in clause 5.1.1.19 of TS 28.552 [X], and PNF Energy consumption for ManagedElement is defined in clause 5.1.1.19.3 of TS 28.552 [X].

- Energy consumption and energy efficiency related KPIs are defined in clause 6.7 of TS 28.554 [C].

This use case is to study the enhancements to enable monitoring and/or estimation of energy consumption and energy efficiency KPIs and reporting of a NG-RAN at operator-specific (i.e., at a per PLMN-ID) granularity in network sharing scenarios.

#### 5.4.A.2	Potential requirements

PREQ-Energy_Consumption_Network_Sharing-1: The 3GPP management system should be able to report the energy consumption and energy efficiency of a NG-RAN at operator-specific, i.e., at a per PLMN-ID granularity.

#### 5.4.A.3	Potential solutions

#### 5.4.A.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.4.A.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.4.A.3.i.2	Description

Editor's Note:	This clause further details the potential solution, and any assumptions made.

#### 5.4.A.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

* * * End of Changes * * * *