3GPP TSG-SA5 Meeting #164	S5-255561

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Add use case and requirements on access control for data

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.2.0

Work Item:	FS_MADCOL_Ph3

Comments

This pCR proposes to add a new use case and requirements to enable access control on performance metrics.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y1]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration   Reference Point (IRP); Information Service (IS)".

[Y2]	3GPP TS 28.319: "Access control for management services".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 4.3 	Enhancement of Management Services Access Control (MSAC)

### 4.3.A	Use Case#<A>:  Access control for performance metrics

#### 4.3.A.1      Description

When an MnS consumer requests the collection of performance metrics using the PerfMetricJob IOC (see clause 4.3.31 of TS 28.622[Y1]), the MnS producer needs to be able to determine whether the MnS consumer is authorized to collect such data or not.

#### 4.3.A.2      Problem Statement

Clause 7.3 of TS 28.319[Y2] defines the AccessRule class associated to a given MnS consumer identified by their Identity and Role classes. The AccessRule class includes the following attributes as defined in clause 7.3.3 of TS 28.319[Y2]):

Table 4.3.A.2-1: AccessRule properties

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | M |
| operations | M |
| actions | O |
| componentCData | O |

The componentCData attribute of the AccessRule class is defined as an optional attribute which specifies notification types and performance metric names.

From the definition of the componentCData attribute of the AccessRule class, it’s not clear how the MnS producer can use this attribute to determine the performance metrics that an MnS consumer is allowed to collect on a set of managed object(s) (represented by the dataNodeSelector attribute of the AccessRule class).

* * * Next Change * * * *

#### 4.1.A.3      Potential Requirements

PREQ-FS_ MADCOL_Ph3-AccCtrl-01:  The security information model should support authorization of performance metrics.

* * * End of Changes * * * *