3GPP TSG-SA5 Meeting #164	S5-255566

Dallas, USA, 17 - 21 November 2025

Source:	Huawei

Title:	pCR TR 28.888 Add use case on transformation of MnS information for external MnS consumers

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.1.0

Work Item:	FS_EnExpo

Comments

Based on the following justification for WT-4: Investigate new management services to support exposure to external MnS consumers of the FS_EnExpo:

According to Reply LS on Internal 5G Core information expose to trusted AF (tdoc number S5-250295) from SA3 to SA6 with SA2, CT3 and SA5 in cc, the following is stated:

"As far as information exposure to an AF is of concerns, it is allowed to expose "Internal 5G Core information" to an AF within the operator’s domain. Besides, it is also possible to expose internal 5G Core information to an AF outside the operator’s domain after proper security protection or privacy protection. For example, an "S-NSSAI" can be mapped to an AF-Service-Identifier before exposing to an AF outside the 3GPP operator domain (i.e., the "S-NSSAI" shall not be exposed directly in this case to be in line with the security requirements specified in the TS 33.501)."

Further when exposing management services to external MnS consumers which is outside the PLMN trust domain, it might be necessary to hide the NRM name-containment hierarchy which is captured by default as part of the "distinguished name (DN)" of the managed object resources or attributes. Accordingly, when publishing management services to external MnS consumers, the management services must be abstracted to avoid exposing directly internal network information to external MnS consumers, e.g., the "DNs" of the MOIs or attributes could be replaced with other identifiers.

Hence, there is a need to identify what kind of management services require abstraction before exposing it to external MnS consumers outside the PLMN trust domain from the 3GPP management system.

This pCR proposes to add use case and potential requirements on transformation of MnS information for external MnS consumers.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.579: "Management services exposure to external consumers through CAPIF".

[4]	3GPP TS 28.319: "Access control for management services".

…

[A]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[AA]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[B]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[C]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[D]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Management Service (MnS): defined in TS 28.533 [2].

External MnS consumer: defined in TS 28.533 [2].

Internal MnS consumer: defined in TS 28.533 [2].

* * * Next Change * * * *

5	Use cases and potential solutions

5.X	Use case #<X>: Transformation of MnS information for external MnS consumers

5.X.1	Description

When an external MnS consumer which is located outside the PLMN trust domain requests access to network information, 3GPP management system needs to ensure that internal 5G information is not revealed. For instance, the relevant "S-NSSAI" need to be transformed/mapped to an AF-Service-Identifier before exposure, rather than being exposed to an untrusted AF directly. 3GPP 5GC control plane uses the same transformation/mapping approach for internal 5GC information. For example, to report the number of UEs and PDU Sessions per network slice to AF, the S-NSSAI is used in case of a trusted AF and AF-Service-Identifier is used for an untrusted AF, see " Translation of internal-external information" in clause 6.2.5.0 of TS 23.501 [A] and "Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration" in clause 4.3.6.1 of TS 23.502 [AA] for further detail.

To manage the untrusted AFs, the related NRM information has been specified in TS 28.541 [B], which includes the NRM supporting for AF-Service-Identifier for an untrusted AF, see the attribute unTrustAfInfoList of NefInfo in clause 5.3.164 of TS 28.541 [B] for further detail.

The MnS consumer can get the performance measurements of a network slice. Performance measurements specified in TS 28.552 [C] can be split into sub-counters per S-NSSAI. 3GPP management system can use these sub-counters to distinguish performance measurements for different consumers/trusted AFs, which might be required when performance measurements are exposed as part of Network Slice as a Service (NSaaS) specified in TS 28.530 [D]. So far, no transformation/mapping approach of an S-NSSAI to an AF-Service-Identifier is specified for the case of an external MnS consumer/untrusted AF in performance management specifications. This use case is to study potential solutions to support transformation/mapping of an S-NSSAI to an AF-Service-Identifier for network slicing performance managements.

5.X.2	Potential requirements

REQ-EnExpo-Trans-01: The 3GPP management system should support a transformation to replace S‑NSSAI with AF‑Service‑Identifier before exposure to external MnS consumers.

5.X.3	Potential solutions

5.X.4	Evaluation of potential solutions

* * * End of Changes * * * *