3GPP TSG-SA5 Meeting #164	S5-255575

Dallas, USA, 17 – 21 November 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.889 Add potential solution for Network Maintenance CCL

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TR 28.889

Version:	0.1.0

Work Item:	FS_CCLM_Ph2

Comments

This pCR provides solution for Clause 4.1 Network Maintenance CCL.

Proposed Changes

* * * First Change * * * *

## 4.1	Use case1: Closed Control Loop for Network Maintenance

### 4.1.1	Description

Network maintenance operations (e.g., software upgrades, downgrades, patching of network functions, license renewals, certificate renewals etc.) can significantly affect service continuity, performance, and user experience. Because of this, they need to be planned, validated, executed, and monitored with minimal disruption. CCLs can help in assisting and automating the maintenance operations, in particular when maintenance needs to be performed in complex scenarios with multiple network functions with dependencies among them, in a large-scale network deployment.

This use case describes a scenario in which an MnS consumer requests a CCL for network maintenance, for example, a software upgrade, to achieve automation for network maintenance. The request may include the type of maintenance required ( e.g., software upgrade, software downgrade), target version for maintenance, any constraints for maintenance (e.g., time-window when the maintenance should take place), as well as any policies required for network maintenance the order of NFs for maintenance, whether isolation of NFs is required for network maintenance work, etc.

The intent-driven MnS producer may be MnS consumer of a Network Maintenance CCL MnS producer if the intent-driven MnS producer handles a Network Maintenance Expectation as specified in 3GPP TS 28.312 [3], clause 6.2.2.1.6.

A CCL for network maintenance may also use MDA reports in 3GPP TS 28.104 [4], for example, for the maintenance use-cases described in clause 7.2.6, to decide and execute necessary actions for network maintenance. A CCL for network maintenance may also analyse the impact of requested software update, for example, the impact of upgrading an NF software version. CCL for network maintenance delivering software updates may also coordinate with Conflict Management and Coordination Entity as specified in TS 28.567 [2], clause 6.3.6 whether requested network maintenance cause any conflicts to existing operations. Based on the analysis, CCL may execute required actions to deliver software updates or execute a rollback in case software updates negatively impact operations.

The CCL for network maintenance MnS producer reports the result of network maintenance, including the resultant maintenance and any other relevant information.

### 4.1.2	Potential requirements

REQ-MaintenanceCCL -1: The 3GPP management system should have the capability to allow MnS consumer to request a closed control loop for network maintenance delivering software updates

REQ-MaintenanceCCL-2: The 3GPP management system should have the capability to allow MnS consumer to get a report from the closed control loop regarding the network maintenance delivering software updates.

### 4.1.3	Potential solutions

#### 4.1.3.1	Potential solution #1

This solution includes defining a new IOC for Network Maintenance CCL, which can be represented by CCLPurpose <<ProxyClass>> as specified in 3GPP TS 28.567 [2], clause 6.3.12. This IOC includes attributes for the following information:

- The type of network maintenance that Network Maintenance CCL needs to deliver, e.g. software upgrade, software downgrade, software patches etc.

- Software version which Network Maintenance CCL needs to execute.

- The time window for the Network Maintenance CCL needs to execute their actions

- The maintenance order for NFs/NEs that Network Maintenance CCL needs to execute their actions

- Any requirement for isolating or backing up the managed object that Network Maintenance CCL needs to consider before executing their actions

- Relevant thresholds for the Network Maintenance CCL to start delivering network maintenance or to execute a rollback.

- Information regarding cell(s) for which network maintenance is requested.

This solution also includes defining a new dataType for Network Maintenance CCL result to be supported by CCLReport as specified in 3GPP TS 28.567 [2], clause 6.3.3. This new dataType includes attributes for the following information:

- The time window that the network maintenance took place.

- The delivered software version after network maintenance

- Reporting any rollback

If the Network Maintenance CCL conflicts with other CCLs, this conflict information can be reported by the ConflictManagementAndCoordinationEntity.

* * * End of Changes * * * *