3GPP TSG-SA5 Meeting #164	S5-255693

Dallas, USA, 17 - 21 November 2025	                                                    	  (revision of S5-255165)

# Source:	China Mobile

# Title:	New WID on Charging enhancements of next generation real time communication services phase 2

# Document for:	Approval

# Agenda Item:	7.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Charging enhancements of next generation real time communication services phase 2

Acronym:	NG_RTC_Ph2_CH_Ph2

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NG_RTC_Ph2 | SA2 | 1040026 | System architecture for Next Generation Real time Communication services Phase 2 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 970014 | System architecture for Next Generation Real time Communication services | Work Item of Stage 2 architecture and procedures (SA2) |
| 990023 | CT aspects of Next Generation Real Time Communication services | Work Item of Stage 3 (CT) |
| 1050022 | CT aspects of System architecture for Next Generation Real time Communication services Phase 2 | Work Item of Stage 3 (CT) |

Dependency on non-3GPP (draft) specification: None.

3	Justification

The charging principles, procedures have been enhanced in Rel-18 and Rel-19 to support next generation real time communication services with the following aspects:

-	Support duration based charging for IMS data channel, including charging procedures during IMS data channel establishment, update and release.

-	Support volume based charging for IMS data channel, considering QoS handling of IMS data channel.

-	Support charging aspects of IMS network capabilities exposure, standalone IMS data channel, avatar communication, services over IMS data channel as a part of 3GPP PS Data Off Exempt Services, and IMS data channel applications download.

However, there are still some SA2 Rel-19 enhancements are identified as having potential impact on charging:

Support of authorization and authentication of third party identities in IMS sessions.

-	3GPP SA2 has defined support for the authorization, signing, and verification of third party user identity information in IMS through TS 23.228 Annex AF. Corresponding procedures and functionalities have been introduced by CT1 in TS 24.229, under clauses 5.7.1.25, 5.7.1.28, 5.10.10.3, and Annex V. To fully support these capabilities, corresponding charging mechanisms are also required. A third party user in this context belongs to a third party network, such as an enterprise or private network. Based on operator policies, IMS entities in the originating network (S-CSCF, IMS AS, or IBCF) may invoke the signing of Rich Call Data (RCD) information or RCD URL. Similarly, in the terminating IMS network, one of these entities may invoke signature verification of the RCD information or RCD URL received in an incoming SIP INVITE request, via a verification AS. The network can charge for signing on the originating side or signature verification on the terminating side. Additionally, the third party user identity can be recorded for charging purposes.

Support of AR communication.

-	3GPP SA2 has enhanced the IMS architecture and procedures for data channel services to incorporate support for AR services, as specified in TS 23.228 Annex AC.9. Corresponding indications have been introduced by CT4 in TS 29.176 clauses 6.1.6.2.4 and 6.1.6.2.6. The AR application server provides network assisted rendering, while an IMS Media Function provides the required network assisted AR media processing. AR communication can be charged based on usage duration. This requires capturing specific AR charging information, allowing operators to set different rates for audio, video and AR by distinguishing their media types.

4	Objective

The objective of the work item is to specify the following charging enhancements for next generation real time communication services:

WT-1: Charging enhancements to support of authorization and authentication of third party identities in IMS sessions.

-	Update charging principles and architecture, along with potential enhancements to charging information and CDRs.

WT-2: Charging enhancements to support of AR communication.

-	Refine charging principles and architecture, and introduce AR media indications for converged charging.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) | Non-3GPP Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- | --- | --- |
| WT-1 | 0 | 2 | No | No | No |
| WT-2 | 0 | 2 | No | No | No |

Total TU estimates for the study phase: 0

Total TU estimates for the normative phase: 4

Total TU estimates: 4

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 32.240 | Enhancements to support charging for authorization and authentication of third party identities in IMS sessions and AR communication. | TSG SA#113 (Sep. 2026) |  |
| TS 32.260 | Enhancements to support charging for authorization and authentication of third party identities in IMS sessions and AR communication. | TSG SA#113 (Sep. 2026) |  |
| TS 32.291 | Update corresponding Nchf_ConvergedCharging service API. | TSG SA#114 (Dec. 2026) |  |
| TS 32.298 | Update corresponding CDR(s) definition and ASN.1. | TSG SA#114 (Dec. 2026) |  |

6	Work item Rapporteur(s)

7	Work item leadership

SA5

8	Aspects that involve other WGs

Architectures in SA2

Protocols and procedures in CT1 and CT4

9	Supporting Individual Members

Supporting IM name

China Mobile

CATT

China Telecom

HiSilicon

Huawei

MATRIXX Software

ZTE

Nokia

Deutsche Telekom