3GPP TSG-SA5 Meeting #162 	S5-254227

Wuhan, China, 13 - 17 October 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0250 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR 28.554 Add energy efficiency KPI of HDLLC network slice for XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | High Data Rate and Low Latency Communications (HDLLC) network slice is desiged for extended reality, cloud gaming and other applications that simultaneously demand low delay and high bandwidth. The Exisiting network slice EE KPIs in TS 28.554 are not applicable to HDLLC. |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add energy efficiency KPI of HDLLC network slice for XR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator cannot evaluate the energy efficiency of HDLLC network slice for XR service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.7.2.X	Energy efficiency of HDLLC network slice

a) EEHDLLC,DV,Latency

b) A KPI that shows the energy efficiency of network slices of type high data rate and low latency communications (HDLLC). The Pns for a network slice of type HDLLC is the sum of UL and DL traffic volumes at N3 interface on a per S-NSSAI basis multiplied by the inverse of the end-to-end User Plane (UP) latency of the network slice. In this KPI variant, data volume and latency are two factors considered for evaluating the performance of network slice.

PHDLLC, DV, lantency= DVN3Network slice mean latency

Where

DVN3=UPF(GTP.InDataOctN3UPF.SNSSAI+GTP.OctDataOctN3UPF.SNSSAI)*8

Network slice mean latency = DelayE2EULNs+DelayE2EDLNs

This KPI is obtained by Pns and energy consumption as the the sum of UL and DL data volumes at N3 interface(s) of the network slice, divided by mean latency and the energy consumption of the network slice.b-1) Real, bit per ms per J

b-2) RATIO

c)

EEHDLLC, DV, lantency= UPF(GTP.InDataOctN3UPF.SNSSAI+GTP.OctDataOctN3UPF.SNSSAI)*8(DelayE2EULNs + DelayE2EDLNs)*ECNS

d) NetworkSlice

e)	In case of redundant transmission paths over the N3 interface for high reliability communication (cf. TS 23.501 [7] clause 5.33.2), it is expected that the data volume is counted once. In particular:

- In case of Dual Connectivity based end to end Redundant User Plane Paths (cf. TS 23.501 [7] clause 5.33.2.1), in which a UE may set up two redundant PDU Sessions over the 5G network, the Data Volume related to only one PDU session is to be considered;

- In case of redundant transmission with two N3 tunnels between the PSA UPF and a single NG-RAN node (cf. TS 23.501 [7] figure 5.33.2.2-1) which are associated with a single PDU Session, the Data Volume related to only one N3 tunnel is to be considered;

- In case of two N3 and N9 tunnels between NG-RAN and PSA UPF for redundant transmission (cf. TS 23.501 [7] figure 5.33.2.2-2) associated with a single PDU Session, the Data Volume related to only one N3 tunnel is to be considered.

For the measurement of the energy efficiency of the 5G core network, the 3GPP management system in charge of collecting the data volume measurements listed here above shall consider them only once in case of redundant transmission over the N3 interface.

End of Change