3GPP TSG-SA5 Meeting #163 	S5-254232

Wuhan, China, 13th October 2025 - 17th October 2025

Title:	Reply to LS on a new measurement related to the maintainability of a QoS flow

Response to:	Respond LS for a new measurement related to the maintainability of a QoS flow (S2-2508116)

Release:	Rel-19

Work Item:	eNA_Ph3, TEI19

Source:	SA5

To:	SA2

Cc:	-

Contact person:	Zu Qiang			Zu.Qiang@ericsson.com

Egemen Kemal Cetinkaya 	egemen.cetinkaya@verizon.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  S5-254233, S5-254234, S5-254235

# 1	Overall description

SA5 thanks SA2 for the incoming LS on a new measurement related to the maintainability of a QoS flow (S2-2508116). SA5 has been requested to monitor and capture statistics of downgraded QoS flows, and to consider adding a QoS Maintainability measurement at the QoS flow level. Such a measurement would reflect the probability of experiencing any type of QoS degradation while the QoS flow is in use.

SA5 has studied the request and has following considerations.

- Defining "downgraded QoS flows": The term is not defined. However, the definitions in TS 23.501 clause 5.7.1.2a and clause 5.7.2.4, with respect to the GFBR, PDB, and PER of the QoS profile, could be used to progress the SA5 work on defining performance measurements and KPIs for QoS flows fulfilment.

- QoS flow monitoring at gNB:

  - The gNB currently measures

    - Number of QoS flow failed to setup (see section 5.1.1.13.3.3 of TS 28.552): This measurement indicates that one or more QoS flows could not be fulfilled at PDU session establishment. It applies to transmission of PDU SESSION RESOURCE SETUP RESPONSE, INITIAL CONTEXT SETUP RESPONSE, or PDU SESSION RESOURCE MODIFY RESPONSE messages.

    - Number of Initial QoS flow failed to setup (see section 5.1.1.13.3.6 of TS 28.552): This measurement indicates that one or more QoS flows could not be fulfilled at UE context establishment. It applies to transmission of INITIAL CONTEXT SETUP RESPONSE message.

    - Number of QoS flows failed to modify (see section 5.1.1.13.4.3 of TS 28.552): This measurement indicates the QoS modification failure, e.g. an alternative QoS profile (from the list in SMF config) reflects currently achieved QoS level. It applies to transmission of PDU SESSION RESOURCE MODIFY RESPONSE message.

  - New measurements could be added: Number of QoS flows fulfilment and non- fulfilment notification (see attached CR). Those measurements would capture RAN-initiated QoS modification cases where active QoS flows are not fulfilled (partially or fully lost) and are fulfilled to a level as described by a referenced less preferred alternative. It applies to the transmission of PDU SESSION RESOURCE NOTIFY messages.

  - The above measurements can indicate QoS non-fulfilment or fulfilment with an alternative profile, but they do not necessarily show whether the fulfilment constitutes a downgrade.

- QoS flow KPI: The existing QoS Flow Retainability KPI specifies the frequency of abnormal, premature flow release. It seems to be beneficial for QoS Sustainability analytics to introduce a KPI that captures how often the performance for an end-user QoS non-fulfilment from the agreed QoS requirements of a QoS flow occurred during the time of the QoS flow is used.

SA5 has discussed and agreed on the attached proposed changes related to QoS flow fulfilment performance measurements and QoS Flow Maintainability KPI. SA5 kindly asks SA2 to assess whether these proposed changes could meet SA2’s requirements.

# 2	Actions

To SA2

ACTION: 	SA5 kindly asks SA2 to assess whether the measurements specified by SA5 address SA2’s requirements, and to provide any feedback.

# 3	Dates of next TSG SA WG 5 meetings

SA5#164		17 Nov – 21 Nov 2025		Dallas, US

SA5#165		9 Feb – 13 Feb 2026		India, IN