3GPP TSG-SA5 Meeting #163	S5-254260

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei

Title:	Pseudo-CR on TR 28.882 Add Management support to UE-Side model training use case

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

The new SID SP-2501197 on study on MDA was approved at the SA #108 meeting. The objectives including:

WT-1: AI/ML Lifecycle Management Enhancements

WT-1.1: Investigate enhancements of AI/ML management capabilities throughout the AI/ML lifecycle in 5GS, including training, testing, emulation, deployment, inference, to support AI/ML-enabled features in the 5GS. This includes:

- ML model transfer/delivery as defined by RAN for Solution 4b: OAM can transfer/delivery AI/ML model(s) to UE.

- NG-RAN use cases including QoE optimization, network energy saving, and mobility use case(as defined in RP-250812).

- 5GC Analytics: Encompasses new 5GC analytics use cases currently under study under WT#2 (see SP-250413) and investigates OAM support for provisioning ML models to relevant 5GC functions to enable AI/ML-based analytics.

- LMF-based AI/ML Positioning including data collection and ML model training by the OAM for UE positioning.

- Study feasibility and potential requirements for data collection for (e.g., UE-side and Network-side) to enable model training.

The bullet 5 is to study feasibility and potential requirements for data collection to enable model training.

According to the agreed CR RP- 252772 in TS 38.300, for AI/ML-based beam management, both NW-side model and UE-side model are supported. And for AI/ML-based CSI prediction, only temporal-domain CSI prediction with UE-side model is supported.

This contribution proposes to add Management support to beam management and CSI prediction for management support to UE-Side model training use case.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x1]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[x2]	3GPP TR 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[x3]	3GPP TS 28.532: "Management and orchestration; Generic management services".

* * * Next Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

OTT	Over-The-Top

* * * Next Change * * * *

X	Use cases

X.Y	Management support to UE-Side model training

X.Y.Z	Management support to AI/ML-based beam management

X.Y.Z.1	Description

To support AI/ML-based beam management defined in TS 38.300 [x1], UE will send the data of beam management to UE-side training entity (e.g. a server inside MNO or an OTT server) via gNB and 3GPP management system for UE-side model training. 3GPP management system needs to collect data from gNB and report it to UE-side training entity.

Figure X.Y.Z-1: Management of UE-side data collection and reporting for UE-side model training

The UE-side training entity requests 3GPP management system to collect the UE-side data of beam management, then 3GPP management system configure the gNB(s) for UE-side data collection, and gNB shall send configuration information to the UE(s). After the UE-side data are produced, UE(s) shall report the UE-side data to gNB(s), and 3GPP management system shall collect data from the gNB(s).  3GPP management system report the collected data to UE-side training entity depending on access information specified by operator.

The request from UE-side training entity shall include the type of data to be produced and the gNB(s) where the data shall be collected.

X.Y.Z.2	Potential requirements

REQ-ML_UESIDE-x1: The 3GPP management system shall have a capability allowing the authorized UE-side training entity to request the UE-side data of beam management for the UE-side model training.

REQ-ML_UESIDE-x2: The 3GPP management system shall have a capability to report the UE-side data of beam management to authorized UE-side training entity.

REQ-ML_UESIDE-x3: The 3GPP management system shall have a capability to allowing the operator to specify access information.

X.Y.Z.3	Potential solutions

Figure X.Y.Z.3-1: Management of UE-side data collection and reporting for UE-side model training

Following are the potential management capabilities which can be used for UE-side data collection control and data report:

-	For UE-side data collection control mechanism, following potential management capabilities can be used:

- CRUD operations/notifications with ManagementDataCollection IOC defined in clause 4.3.47 in TS 28.622[x2].

- CRUD operations/notifications with TraceJob defined in clause 4.3.30 in TS 28.622[x2].

-	For UE-side data reporting mechanism, following existing management capabilities can be used:

- Streaming data reporting service defined in clause 11.5 in TS 28.532[x3].

- File data reporting service in clause 11.6 in TS 28.532[x3].

Following are the potential enhancements to support UE-side data collection control and data reporting

-	Enhance the existing trace metrics to include the UE-side data.

NOTE: the definition of UE-side data depending on RAN1 and RAN2.

X.Y.Z.4	Evaluation of solutions

It is feasible to reuse the trace mechanism and the only enhancement is to add new UE-side data to the existing trace metrics to support AI/ML-based beam management.

X.Y.Z1	Management support to AI/ML-based CSI prediction

X.Y.Z1.1	Potential requirements

To support AI/ML-based CSI prediction defined in TS 38.300 [x1], UE will send the data of CSI prediction to UE-side training entity (e.g. a server inside MNO or an OTT server) via gNB and 3GPP management system for UE-side model training. 3GPP management system needs to collect data from gNB and report it to UE-side training entity.

Figure X.Y.Z1-1: Management of UE-side data collection and reporting for UE-side model training

The UE-side training entity requests 3GPP management system to collect the UE-side data of CSI prediction, then 3GPP management system configure the gNB(s) for UE-side data collection, and gNB shall send configuration information to the UE(s). After the UE-side data are produced, UE(s) shall report the UE-side data to gNB(s), and 3GPP management system shall collect data from the gNB(s).  3GPP management system report the collected data to UE-side training entity depending on access control information specified by operator.

The request from UE-side training entity shall include the type of data to be produced and the gNB(s) where the data shall be collected.

X.Y.Z1.2	Potential requirements

REQ-ML_UESIDE-x1: The 3GPP management system shall have a capability allowing the authorized UE-side training entity to request the UE-side data of CSI prediction for UE-side model training.

REQ-ML_UESIDE-x2: The 3GPP management system shall have a capability to report the UE-side data of CSI prediction to authorized UE-side training entity.

REQ-ML_UESIDE-x3: The 3GPP management system shall have a capability to allowing the operator to specify access control information.

X.Y.Z1.3	Potential solutions

Figure X.Y.Z1.3-1: Management of UE-side data collection and reporting for UE-side model training

Following are the potential management capabilities which can be used for UE-side data collection control and data report:

-	For UE-side data collection control mechanism, following potential management capabilities can be used:

- CRUD operations/notifications with ManagementDataCollection IOC defined in clause 4.3.47 in TS 28.622[x2].

- CRUD operations/notifications with TraceJob defined in clause 4.3.30 in TS 28.622[x2].

-	For UE-side data reporting mechanism, following existing management capabilities can be used:

- Streaming data reporting service defined in clause 11.5 in TS 28.532[x3].

- File data reporting service in clause 11.6 in TS 28.532[x3].

Following are the potential enhancements to support UE-side data collection control and data reporting

-	Enhance the existing trace metrics to include the UE-side data.

NOTE: the definition of UE-side data depending on RAN1 and RAN2.

X.Y.Z1.4	Evaluation of solutions

It is feasible to reuse the trace mechanism and the only enhancement is to add new UE-side data to the existing trace metrics to support AI/ML-based CSI prediction.

* * * End of Changes * * * *