3GPP TSG-SA5 Meeting #163	S5-254387

Wuhan, China, 13. - 17. October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0739 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR 28.552 Add a new measurement related to the number of requests of ADRF storage services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_Ph3-OAM |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.288 (SA2), the services of Nadrf_DataManagement_StorageRequest is specified to enable the consumer NF to request the ADRF to store data or analytics. As described in TS 23.288, ADRF may be hosted by NWDAF. By measuring the number of ADRF storage requests, the operator can estimate the potential workload on NWDAF hosting ADRF for storage services and take management actions for configuration and resource allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Number of storage service requests received by ADRF hosted by NWDAF” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the performance of NWDAF hosting ADRF. This measurement will be considered for performing NWDAF management actions, such as configuration, resource allocation, load balancing and deployment purposes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.X      Performance measurements related to ADRF

5.X.1    Measurements related to storage services provided by ADRF

5.X.1.1   Number of storage service requests received by ADRF hosted by NWDAF

a)	This measurement provides the number of requests received by an ADRF hosted by NWDAF for data or analytics storage.

b)  CC

c)	On receiving the storage service request from the consumer NF to store data or analytics in the ADRF hosted by NWDAF, each request is added to the corresponding counter. This measurement can be split into subcounters per consumer NF. The consumer NF can be NWDAF, DCCF or MFAF. (See TS 23.288 [59]).

d)	It is an integer value

e)	The measurement name has the form DANS.DataStorageRes.NFID, where NFID identifies the consumer NF (See TS 23.288 [59]).

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Change