3GPP TSG-SA5 Meeting #163	S5-254427

Wuhan, China, 13 - 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on Clarification of Mechanisms to Request and Retrieve Management Data

Document for:	Approval

Agenda item:	6.20.7

Spec:	3GPP TR 28.887

Version:	0.1.0

Work Item:	FS_MADCOL_Ph3

Comments

The group is requested to discuss and approve the pCR below.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[XY]	3GPP TS 28.622: "Generic Network Resource Model (NRM)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

#### 4.1.A.4      Potential Solution

### 4.4.Y	Use Case#<Y>: Specified mechanisms to Request and Retrieve Management Data

#### 4.4.Y.1      Description

TS 28.622 [XY] specifies the following mechanisms to request and retrieve management data:

-	TraceJob IOC, as specified in clause 4.3.30, allows to request and retrieve trace metrics which include trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RLF, RCEF and RRC reports.

-	PerfMetricJob IOC, as specified in clause 4.3.31, allows to request and retrieve performance metrics which include performance measurements and KPIs (end-to-end key performance indicators).

- 	QMCJob IOC, as specified in clause 4.3.54, allows to request and retrieve QoE information.

- 	ManagementDataCollection IOC, as specified in clause 4.3.47, allows to request and retrieve trace metrics and/or performance metrics.

-	ExternalDataType IOC, as specified in clause 4.3.73, allows to request external management data.

-	ThresholdMonitor IOC, as specified in clause 4.3.16, allows to check for threshold crossings of performance metric values related to specified managed objects.

-	Scheduler IOC, as specified in clause 4.3.62, allows to monitor whether a certain time criterion is fulfilled.

-	ConditionMonitor IOC, as specified in clause 4.3.65, allows to monitor whether one or several conditions are satisfied.

-	Files IOC, as specified in clause 4.3.44, represents a collection of files. It may be used together with a data collection job representing the files created in context of the collection.

-	File IOC, as specified in clause 4.3.45, represents a file. It may be used together with a data collection job and the file collection representing one file created in context of the collection.

-	FileDownloadJob IOC, as specified in clause 4.3.46, represents a job on a MnS producer that downloads a file to the MnS producer.

#### 4.4.Y.2      Problem Statement

#### TS 28.622 [XY] specifies several mechanisms to request and retrieve management data. It is not always easy to select the appropriate IOC or combination of IOCs to request and retrieve management data or appropriate management services (MnS). There is a need to provide clarification and documentation on the usage of these specified mechanisms to request and retrieve management data.

#### 4.1.Y.3      Potential Requirements

#### 4.1.Y.4      Potential Solution

* * * End of Changes * * * *