3GPP TSG-SA5 Meeting #163	S5-254456

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei

Title:	pCR TR 28.884 Use case on software management

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	V0.0.0

Work Item:	FS_SBMA_Ph4

Comments

To address WT-5 of Study on Service Based Management Architecture enhancement phase 4, this contribution proposes to add use case on software management in SBMA for 5G.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[XX]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

* * * Next Change * * * *

## 5.X	Use case on software management for 5G

### 5.X.1	Description

Software management enables network operators with network management capabilities including software download, activation, upgrade, fallback, software information query and monitoring, pre-check, etc, allowing operators to maintain continuous service availability while introducing new patches across multi-vendor environments.

Software management for 5G enhances 5G network operational efficiency and reduces costs. The benefits of software management retain for 5G network management:

-	One benefit of software management is interoperability: standardized procedures make it possible for operators to manage software on heterogeneous network elements in a multi-vendor environment and reduces operational complexity.

-	Another benefit is network service continuity. By supporting mechanisms such as staged upgrades, version control, and fallback strategies, software management capability minimizes downtime and protects user experience.

Currently, TS 28.533 [XX] does not include support for software management functions. As 5G networks evolve in scale and complexity, it is essential to introduce software management capabilities into SBMA. These capabilities will enable operators to maintain software of NEs. To enable software management of NEs within SBMA, the potential requirements and potential solutions need to be studied.

### 5.X.2	Potential requirements

REQ-SBMA-SWM-1: The 3GPP management system should support software download.

REQ-SBMA-SWM-2: The 3GPP management system should support software activation.

REQ-SBMA-SWM-3: The 3GPP management system should support software fallback.

REQ-SBMA-SWM-4: The 3GPP management system should support the capability to monitor states of software download, installation and activation.

REQ-SBMA-SWM-5: The 3GPP management system should support the capability to query the information of a software.

REQ-SBMA-SWM-6: The 3GPP management system should support the capability to pre-check the software to check whether the software can be successfully activated or not.

REQ-SBMA-SWM-7: The 3GPP management system should support the capability to pre-activate the software to check whether the software can be successfully activated or not.

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of potential solutions

* * * End of Changes * * * *