3GPP TSG-SA5 Meeting #163	S5-254459

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei

Title:	pCR TR 28.884 Management Data Streaming based on Message Bus

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.0.0

Work Item:	FS_SBMA_Ph4

Comments

To address WT-1 of Study on Service Based Management Architecture enhancement phase 4, this contribution proposes to add concept and use case for management data streaming based on message bus.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[b]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[c]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[d]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[e]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

* * * Next Change * * * *

# 4	Concepts and background

## 4.1	Management data streaming based on Message Bus technologies

3GPP TS 28.532 [a] defines Stage 2 and Stage 3 of the streaming data reporting service. Currently, the 3GPP management system supports WebSocket-based data streaming for PM, tracing, and analytics, establishing a point-to-point connection between the streaming data reporting MnS consumer and MnS producer. The producer initiates the connection with the consumer and exchanges metadata (informing the consumer about its identity and the nature of the data to be reported, i.e., streamInfo). After this phase, the producer reports streaming data to the consumer via the established connection.

As 5G networks scale in complexity and distribution, traditional point-to-point communication models for management services face limitations in scalability, resiliency, and flexibility, highlighting the advantages of introducing message bus solution. Message bus architectures employ a publish-subscribe (pub-sub) model, offering multipoint-to-multipoint communication, asynchronous delivery, and decoupled service interactions, which are well-suited for distributed management deployments. A message bus offers the following key advantages, which address the weaknesses of conventional WebSocket-based connections:

- High Performance: A message bus offers multipoint-to-multipoint communication, which is suitable for handling large-scale, real-time data with high throughput and low latency.

- Robustness and Resilience: A message bus supports message storage and can be deployed in a distributed manner, therefore minimizes downtime and data loss, ensuring consistent and reliable service experiences.

- Data Management Efficiency: A message bus provides asynchronous delivery and decoupled service interactions. It avoids redundant data collection, simplifies data pipelines and reduces operational costs.

There are industry solutions that can realize message bus functionalities. The 3GPP management system should evolve to consider the integration and compatibility with these existing industry solutions by providing a general scope which covers management data types defined in 3GPP TS 28.537 [b] (e.g. performance data, trace data, etc.).

NOTE:	The management data includes performance measurements or KPIs defined in TS 28.552 [c] and TS 28.554 [d], trace reporting data defined in TS 32.423 [e] and analytic reporting data defined in TS 28.532 [a].

Architectural Considerations:

-	Logical View: MnFs remain implementation-agnostic and interact via MnSs; the message bus acts as a transport abstraction layer.

-	Deployment View: Message bus clusters may be deployed per management domain (e.g., RAN management domain, CN management domain) or globally, depending on latency and fault domain requirements.

-	Service Interaction View: MnS Producers publish events or data streams to message queues; MnS Consumers subscribe based on service type, management domain, or operational context. The message bus handles delivery, buffering, and replay. Message queue names are exchanged by the MnS producer and consumer.

# 5	Use cases and potential solutions

## 5.1	Use case #1: Management data streaming based on Message Bus

### 5.1.1	Description

The 5G network employs a distributed, service-oriented architecture, leading to scenarios where multiple consumers request the same management data for multiple uses. Using traditional point-to-point WebSocket communication, a separate connection would be established between each consumer and the producer, causing the producer to generate and transmit multiple copies of the same data. This results in redundant data production and inefficient use of transmission resources, hindering effective data management.

By introducing a message bus mechanism, multiple consumers can subscribe to the same data. The producer only needs to generate a single copy of the data and send it to the message bus. The message bus then automatically replicates and efficiently distributes the data to all subscribed consumers, significantly improving data production, distribution, and management efficiency.

Operations for streaming data report defined in [a] are:

- establishStreamingConnection operation (M)

- terminateStreamingConnection operation (M)

- reportStreamData operation (M)

- addStream operation (M)

- deleteStream operation (M)

- getConnectionInfo operation (M)

- getStreamInfo operation (M)

These operations are defined from the producer's perspective. In the point-to-point WebSocket connection method, the producer initiates a stream connection to the consumer and sends the stream data. However, with the introduction of the message bus architecture, the interaction is no longer limited to between the producer and consumer. Instead, it is decoupled into separate interactions between each of them and the message bus.

Therefore, to support management stream data reporting services based on the message bus, enhancements are needed in the following areas:

- Producer side: Enhance the existing APIs to support the producer to report stream data to specific message queues based on the message bus protocol.

- Consumer side: Add a set of new APIs to support the consumer to retrieve stream data from specific message queues based on the message bus protocol.

- Interaction aspect: Study the mechanism to support the exchange of the endpoint information and message queues between the producer and the consumer.

### 5.1.2	Potential requirements

REQ-SBMA-MBUS-1: The 3GPP management system should provide capabilities allowing the producer to report stream data to specific message queues based on the message bus protocol.

REQ-SBMA-MBUS-2: The 3GPP management system should provide capabilities allowing MnS consumers to retrieve stream data from specific message queues based on the message bus protocol.

REQ-SBMA-MBUS-3: The 3GPP management system should provide capabilities to support the exchange of the information of the message bus endpoint and message queues between the MnS producer and the MnS consumer.

### 5.1.3	Potential solutions

### 5.1.4	Evaluation of potential solutions

* * * End of Changes * * * *