3GPP TSG-SA5 Meeting #163	S5-254498

Wu Han, China 13-17 October 2025

# Source:	Huawei

# Title:	Discussion paper on the Charging parameters categories

# Document for:	Endorsement

Agenda Item:	7.3

1	Decision/action requested

Propose to clearly specify the charging parameters categories.

2	References

[1]	3GPP TS 32.240: "Charging management; Charging architecture and principles"

[2]	3GPP TS 32.291: "Charging management; 5G system, charging service; Stage 3"

[3]	3GPP TS 32.298: "Charging management; Charging Data Record (CDR) parameter description"

[4]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3"

[5]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3"

3	Rationale

The Charging Information Element(s) and CDR parameters are specified in the middle tier TSs per domain /service / subsystem based on the common principle of charging data configuration.

3.1	Charging IE and CDR parameter Definition

TS 32.240 [1] specifies the Mandatory (M), Conditional (C) and Operator provisionable (OC or OM) designations for category of IE or CDR parameter in clause 5.4.

M	This parameter is Mandatory and shall always be present in the event / CDR.

C	This parameter shall be present in the event / CDR only when certain Conditions are met. These Conditions are specified as part of the parameter definition.

OM	This is a parameter that, if provisioned by the operator to be present, shall always be included in the events / CDRs. In other words, an OM parameter that is provisioned to be present is a mandatory parameter.

OC	This is a parameter that, if provisioned by the operator to be present, shall be included in the events / CDRs when the specified conditions are met. If provisioned by the operator not to be present, shall not be included in the events / CDRs even the specified conditions are met. In other words, an OC parameter that is configured to be present is a conditional parameter.

TS 32.291 [2] specifies the Data Type of attributes in charging data message with M, Oc, Om and the corresponding YAML with the "required" attributes Mandatory and Optional.

Figure 3.1-1: The required attribute in YAML

TS 32.298 [3] specifies the CDR parameters of CHF CDR with M, Oc, Om in clause 5.2.5.0 and Generic ASN.1 definitions in clause 5.2.1 with OPTIONAL and Mandatory.

Figure 3.1-2: The CDR parameter description

Figure 3.1-3: The CHF CDR ASN.1

3.2	SBI Definition

TS 29.501 [4] specifies the URI query parameters supported by a method on the resource in clause 5.2.2.

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| <name> or n/a | <type> or <leave empty> | <M, C or O> | 0..1 or 1 or 0..N or 1..N or <leave empty> | <only if applicable> |  |

Figure 3.2-1: URI query parameters supported by a method on the resource

P: Presence condition of URI query parameters. It shall be one of "M" (for Mandatory), "C" (for Conditional) and "O" (for Optional). If no query parameter is defined for the URI, the column is left empty.

Cardinality: Defines the allowed number of occurrence. It shall be "0..1", "1", "0..N", "1..N" . If no query parameter is defined for the URI, the column is left empty.

TS 29.512 [5] specifies the Npcf_SMPolicyControl service operation and API, can give the example how to implement the common SBI definition.

Figure 3.2-2: The example of category C attribute with the 0..1 and 1..N Cardinality

Figure 3.2-3: The example of category O attribute with the 0..1 and 1..N Cardinality

3.3	The Mapping Relationship

The table 3.3-1 describes the mapping relationship between information element, attributes, parameters in the stage2 and stage3.

Table 3.3-1 The Mapping of Charging attributes in stage 2 and stage 3

| Category of Charging data | Charging information | Data type of Attribute | YAML | CDR parameters | ASN.1 |
| --- | --- | --- | --- | --- | --- |
| TS 32.240 | TS 32.XXX | TS 32.291 | TS 32.291 | TS 32.298 | TS 32.298 |
| M | M | M | Required | M | Default(/) (NOTE 2) |
| C | NA | NA | NA | NA | NA |
| OM (Operator) | OM | OM | Default(/) (NOTE 1) | OM | OPTIONAL |
| OC (Operator) | OC | OC | Default(/) (NOTE 1) | OC | OPTIONAL |
| NOTE 1: "The Default(/)" indicates the charging attributes without carrying the explicit instructions (i.e. Required). NOTE 2: "The Default(/)" indicates the charging parameters in ASN. 1 without carrying the explicit instructions (i.e.OPTIONAL). |  |  |  |  |  |

Table 3.3-2 The Mapping of Charging attributes in stage 2 and stage 3

| SBI Common | Service | YAML |
| --- | --- | --- |
| TS 29.501 | TS 29.XXX | TS 29.XXX |
| M | M | Required (Partial) |
| C | C | Default(/) |
| O (Optional) | O | Default(/) |
| NOTE 1: "The Default(/)" indicates the attributes without carrying the explicit instructions (i.e. Required). |  |  |

3.4	Discussion

Problem 1: For Service based interface, whether all the attributes with the Mandatory should be marked as the required in the YAML (see the details in the table 3.3-2)? What are the principles for the required attribute in the YAML?

Problem 2: For charging information element and CDR parameters, whether and what kind of the charging Information Element can be categorized as the C (see the details in the table 3.3-1)?

Problem 3: How to improve the solution for the inconsistent of charging information category between stage 2 and Stage 3? The current solution mechanism is specified in the TS 32.240 clause 5.4.

The following principles apply for Information Element (IE) and CDR parameter category across the specifications:

-	Category for IEs common between the middle tier TSs (stage 2) and TS 32.290 [57]: IE category in the middle tier TSs takes precedence;

-	IE category in the middle tier TSs takes precedence over the corresponding IE stage 3 category and syntax in TS 32.291[58] and TS 32.299[50].

-	CDR parameter category in the middle tier TSs takes precedence over the corresponding ASN.1 field syntax in TS 32.298 [51].

4	Detailed proposal

In order to keep alignment with data type definition principles specified in the TS 29.501, the proposal corresponding to DP in CR 0648 is to correct the Cardinality and Presence.