3GPP TSG-SA5 Meeting #163	S5-254521

Wuhan, China, 13th Oct 2025 - 17th Oct 2025

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case support of energy-related characteristics

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.0.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for "Enhancements to support energy efficiency as a service criteria", related to WT-1.

This pCR introduces use case and potential requirements related to WT1.1 of the SI FS_Energy_Ph4_OAM.

"

WT-1: Study 3GPP management system enhancements to support energy efficiency as a service criteria.

NOTE 1: This WT is to study the impacts on 3GPP management system to support the requirements specified by SA1 (in clause 6.15a TS 22.261) as part of 5GA Rel-20 WI EnergyServ_Ph2-REQ.

WT-1.1: Study enhancements to support the energy-related characteristics (defined in TS 22.261) for the 5G Network Elements and Network Functions. This includes:

1. Study enhancements to support association and exposure (to authorized consumers) of energy-related characteristics (defined in TS 22.261) for the 5G Network Elements.

NOTE 2: This is to study if further enhancements are required related to the energy-related information that is specified in Rel-19.

…

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 22.261: "Service requirements for the 5G system".

[B]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[C]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[H]	ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[I]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent).

NOTE A:	This definition is taken from 3GPP TS 22.261 [A].

carbon intensity: quantity of CO2 equivalent emission per unit of final energy consumption for an operational period of use. See ETSI GS OEU 020 (v1.1.1) [H].

NOTE B:	This definition is taken from 3GPP TS 22.261 [A].

energy availability: the remaining amount of energy (e.g. in kWh) locally available for consumption. For devices, network elements and functions, energy availability may be limited and/or intermittent, in particular when relying on batteries and/or renewable energy sources (e.g. off-grid base stations, satellites etc) or during power grid heavy load or disruptions.

NOTE C:	This definition is taken from 3GPP TS 22.261 [A].

energy capacity: the maximum amount of energy (e.g. in kWh) that can be locally available for consumption (either locally produced and/or stored) by a device or a network element or function.

NOTE D:	This definition is taken from 3GPP TS 22.261 [A].

Energy Consumption (EC): integral of power consumption over time.

NOTE E: see ETSI ES 202 706-1 [I].

NOTE X: This definition is taken from TS 28.310 [B]

energy-related characteristics: information which characterize the energy to power the operator’s network in terms of energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions.

NOTE F:	Which energy-related characteristics are relevant depends on the scenario.

energy supply mix: the combination of the various energy sources (i.e. renewable and not) used to meet energy needs of a device or a network element or function.

NOTE G:	This definition is taken from TS 22.261 [A].

renewable energy: energy from renewable sources as energy from renewable non-fossil sources, namely wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE H:	This definition is taken from TS 28.310 [B] and TS 22.261 [A].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

* * * Next Change * * * *

### 5.1.A	Use case #<A>: Enhancements to support the energy-related characteristics for Network Elements and Network Functions

#### 5.1.A.1	Description

Energy-related characteristics, energy availability, energy capacity, energy supply mix, carbon emission and carbon intensity are defined in TS 22.261 [A].

The requirements related to energy-related characteristics are specified in "Energy related information as a service criteria" (see clause 6.15a.2 of TS 22.261 [A]), and "Information exposure" (see clause 6.15a.5 of TS 22.261 [A]).

Energy supply information can be associated with the Managed Elements (representing the Network Elements) using EnergyInfoGroup IOC (see clause 8.3.3. of TS 28.310 [B]). Energy-related information for each energy source (see EnergySourceInfo defined in clause 8.3.2 of TS 28.310 [B]) of the energy supply (see EnergySupplyInfo defined in clause 8.3.1 of TS 28.310 [B]) includes energy supply mix, carbon emission factor, renewable and non-renewable energy information.

Energy consumption related KPIs are defined in clause 6.7.3 of TS 28.554 [C] and gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are defined in clause 6.7.7. of TS 28.554 [C].

This use case is to study the gaps and enhancements to the energy-related characteristics associated with Managed Elements to satisfy the requirements in TS 22.261 [A]. This includes the following aspects for Network Elements and/or Network Functions:

1)	Carbon footprint including carbon emission and carbon intensity: How carbon footprint information Network Element and Network Function can be made available and/or estimated in 3GPP management system.

2)	Energy capacity: How energy capacity information of each energy supply of the Network Element can be made available and/or estimated in 3GPP management system.

3)	Energy availability: How energy availability information of each energy supply of the Network Element can be made available and/or estimated in 3GPP management system.

#### 5.1.A.2	Potential requirements

PREQ-Energy_Related_Characteristics-1: The 3GPP management system should be able to estimate and report the carbon footprint information Network Element and Network Function.

PREQ-Energy_Related_Characteristics-2: The 3GPP management system should enable an authorized consumer to retrieve energy capacity information of each energy supply of the Network Element.

PREQ-Energy_Related_Characteristics-3: The 3GPP management system should enable an authorized consumer to retrieve energy availability information of each energy supply of the Network Element.

#### 5.1.A.3	Potential solutions

#### 5.1.A.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.1.A.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.1.A.3.i.2	Description

Editor's Note:	This clause further details the potential solution, and any assumptions made.

#### 5.1.A.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

* * * End of Changes * * * *