3GPP TSG-SA5 Meeting #163	S5-254533

Wuhan, China, 13. - 17. October 2025

Source:	China Mobile

Title:	Pseudo-CR on TR 28.882 Add new use cases for ML model deployment phase

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	V0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This contribution proposes to address new use cases with respect to AIML lifecycle management features for TR 28.882.

Proposed Changes

* * * First Change * * * *

5	Management capabilities for AI/ML lifecycle

5.A	Management capabilities for ML model deployment phase

5.A.1	Management of ML applications

5.A.1.1	Description

In practical deployments, ML models and its lifecycle related functions (i.e., ML training functions, ML testing functions, AIML inference emulation functions, and ML inference functions, etc.) may have lineage dependencies. A given ML model version may only be applicable to a specific vendor’s environment and needs to be used together with the corresponding versions of its lifecycle related functions. Therefore, some operators require that the ML model along with its lifecycle related function be packaged as a whole and deployed at a designated location.

Such packaging has the benefit of avoiding frequent transfer of ML models when the training location and deployment location are remote, and the ML model undergoes frequent iterations. For example, the training function may reside in a cross-domain management system at a central site, while the AIML inference function with the loaded model needs to be deployed at remote sites (e.g., an inference function at an edge node or an AnLF within a CN-domain NWDAF).

In such cases, the MnS consumer may request to deliver all components with lineage dependencies as an ML application, which encapsulates the functional components requirement for the ML model lifecycle (i.e., ML training functions, ML testing functions, AIML inference emulation functions, and ML inference functions, etc.). This enables the ML application to be deployed as a unified package to a specific management entity, ensuring compatibility, consistency and operational efficiency.

5.A.1.2	Use cases

5.A.1.2.1	ML application deployment and release

An ML application encapsulates the functional components requirement for the ML model lifecycle (i.e., ML training functions, ML testing functions, AIML inference emulation functions, and ML inference functions, etc.) along with the metadata that contains the ML application requirement information.

For example, the ML application could contain all the components including the ML training function for RL, ML testing function, AIML inference function with the loaded ML model. After the ML model trained, tested and emulated, the ML application will be built and is going to be deployed in the target ML application platform, e.g., a gNB.

The consumer of ML applications would like to request to build and deploy the ML application by configuring the ML application requirement.This ML application requirement information contains functional requirement (i.e., the ML application components.) information and preset template information (e.g., environment requirement information, ML application algorithm information, ML application process information, interface configuration template information, .

The MnS producer of ML application deploymen obtains the ML application requirement information based on the functional requirement information and preset template information. According to the ML application requirement information, the MnS producer may perform resource exploration to check whether the ML application is feasible to run based on current resources and environment (e.g., NWDAF, LMF, MDA or gNB, etc.).

When the result of the resource exploration indicates that the target application is allowed to run, the MnS producer allocates the resources of the ML application for the target ML application from the resources of the target operating platform (e.g., an MNF, NF or a specific server, etc.), and create the runtime environment within the platform for the target ML application.

The producer should deploy and release the target ML application according to the allocated application resources and the runtime environment.

5.A.1.3	Potential requirements

REQ-MLAPP-1: The MnS producer of ML application deployment and release should enable an authorized MnS consumer to configure ML application requirement information contains functional requirement information and preset template information including environment requirement information, interface configuration template information, ML application algorithm information, and ML application process information, etc.

REQ- MLAPP-2: The MnS producer of ML application deployment and release should have a capability to perform resource exploration based on ML application requirement information.

REQ- MLAPP-3: The MnS producer of ML application deployment and release should have a capability to inform an authorized MnS consumer whether the ML application is feasible to run based on current resources and environment (e.g., NWDAF, LMF, MDA or gNB, etc.).

REQ- MLAPP-4: The MnS producer of ML application deployment and release should have a capability to allocate the resources of the ML application for the target ML application from the resources of the target operating platform (e.g., an MNF, NF or a specific server, etc.).

REQ- MLAPP-5: The MnS producer of ML application deployment and release should enable an authorized MnS consumer to request deploy and release the target ML application according to the allocated application resources and the runtime environment.

5.A.1.4	Possible solutions

5.A.1.5	Potential requirements

* * * End of Changes * * * *